3GPP TSG-RAN4 Meeting #114	R4-2500755

# Athens, GR, 17th – 21st February, 2025

Source:	vivo

Title:	TP for TR 38.774 on LP-WUS UE RF

Agenda item:			7.26.2.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.774 to capture a few general agreements for LP-WUS RF from WF [2-5].

2. Reference

[1] TR 38.774 v0.1.0

[2] R4-2417112, WF on UE RF requirements for LP-WUS, vivo, RAN4#112bis

[3] R4-2414309, WF on LP-WUS UE RF requirements, vivo, RAN4#112

[4] R4-2410569, WF for 111136 NR_LPWUS_UERF, vivo, RAN4#111

[5] R4-2420528, WF on UE RF requirements for LP-WUS, vivo, RAN4#113

[6] Renzhi Liu, et al., An 802.11ba-Based Wake-Up Radio Receiver with Wi-Fi Transceiver Integration, IEEE JOURNAL OF SOLID-STATE CIRCUITS, VOL. 55, NO. 5, pp. 1151-1164, MAY 2020

# 3. Text Proposal

<Start of Text Proposal>

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up.

LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

<End of Text Proposal>

<Start of Text Proposal>

5	System parameters

5.1	General

The channel arrangements presented in this clause for LP-WUS are based on the operating bands and channel raster defined in sub-clause 5.2 and 5.3 below.

5.2	Operating bands

LP-WUS is designed to operate in the operating bands defined in Table 5.2-1 of TS 38.101-1 [3] and TS 38.101-2 [4].

5.3	Channel raster

The PRB grid between LR and MR is aligned and the LP-WUS RBs can be flexibly allocated within the wider NR carrier. How to make sure the RB alignment between LP-WUS RB and MR RB grid is UE implementation specific.

<End of Text Proposal>

<Start of Text Proposal>

6.2	Simulation assumption

6.2.1	General

To evaluate the RF performance of LP-WUS/WUR, the link level simulation is performed in this WI. This clause captures the simulation assumption for LP-WUS UE RF and BS RF analysis.

6.2.2	Simulation parameters

For RF performance analysis, the following simulation assumptions are considered.

Table 6.2.2-1 Link level simulation assumption for FR1 LP-WUS

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8/16 bits |  |  |
| Chip rate | M=1 | M=1/2/4 |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% Providing the information whether the false alarm rate is considered or not |  |  |
| Note: the detailed Channel structure for LP-WUS is under discussion in RAN1, some parameters may be updated accordingly. |  |  |  |

6.2.3	Phase noise considerations

In RAN4 simulation analysis, it is identified that the phase noise will impact the ACS performance due to the reciprocal mixing, and the following phase noise models can be used as simulation assumption for FR1.

- Option 1

| Offset | 5MHz | 20MHz |
| --- | --- | --- |
| Phase noise | -97dBc/Hz | -116dBc/Hz |

- Option 2

| Offset | 5MHz | 5MHz +1RB | 5MHz +2RB | 5MHz +3RB | 5MHz +4RB |
| --- | --- | --- | --- | --- | --- |
| Phase noise | -90.5dBc/Hz | -91.1dBc/Hz | -91.7dBc/Hz | -92.2dBc/Hz | -92.7dBc/Hz |

- Option 3

|  | Phase noise (dBc/Hz) |  |  |
| --- | --- | --- | --- |
| Offset | NF 9 dB | NF 12 dB | NF 15 dB |
| @5MHz | -99.4 | -99.4 | -99.4 |
| @10MHz | -108.1 | -105 | -102.1 |
| @15MHz | -120 | -117 | -114 |

6.2.4	Timing error considerations

In RAN4 simulation analysis, it is identified that the timing error would have impacts on RF performance. The following timing error can be considered for evaluation:

- For OOK-based receiver, the timing error is 5/3/1us for OOK M=1/2/4;

- For OFDM-based receiver, the timing error is 0.9 and 1.86us;

<End of Text Proposal>