3GPP TSG-RAN WG4 Meeting #114	R4-2500757

Athens, GR, 17th – 21st February, 2025

Agenda item:	 7.26.2.2

Source:	 vivo

Title:	Discussions on LP-WUS REFSENS, ASCS and ACS

Document for:	 Approval

- Introduction

Based on agreements for REFSENS, ASCS and ACS in previous meetings [1] [2] [3], this contribution shares our further simulation results for LP-WUS REFSENS, ASCS and ACS requirements.

- LP-WUS UE RF

  - Link-level simulation for LP-WUS target SNR

In LTE/NR, SNR = -1 dB is widely used to calculate REFSENS and this value is derived from 95% throughput at link-level simulation under AWGN, 1/3QPSK, no HARQ, 12,5 symbol per subframe in [4][5].

RAN4 has agreed that AWGN channel model with 1% MDR is selected to sepecify RF requirements, and the target SNR is the performance without repetition. Using the agreed simulation parameters, our LLS simulation results for LP-WUS SNR with AWGN model at 1% MDR is presented in Table 1.

Table 1 SNR@1% MDR with 4bit ADC under AWGN (no repetition)

| Modulation | Chip rate | Channel structure | Receiver type | SNR@1% MDR |
| --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 8bit | Envelope | -3.0 dB |
|  |  |  | OFDM | -9.3 dB |
|  |  | 16bit | Envelope | -2.9 dB |
|  |  |  | OFDM | -9.2 dB |
| OOK-4 | M = 1 | 8bit | Envelope | -3.1 dB |
|  |  | 16bit |  | -3.0 dB |
|  | M = 2 | 8bit |  | -4.2 dB |
|  |  | 16bit |  | -4.0 dB |
|  | M = 4 | 8bit |  | -2.5 dB |
|  |  | 16bit |  | -2.3 dB |
| OOK-4 | M = 1 | 8bit | OFDM | -9.3 dB |
|  |  | 16bit |  | -9.1 dB |
|  | M = 2 | 8bit |  | -9.5 dB |
|  |  | 16bit |  | -9.2 dB |
|  | M = 4 | 8bit |  | -6.5 dB |
|  |  | 16bit |  | -6.2 dB |

Observation 1: The worst case is OOK-4, M=4, payload = 16bit with envelope-based receiver, and the SNR without timing error and frequency error at 1% MDR is about -2dB. With the same modulation, the SNR performance of OFDM-based receiver is always several dBs better than Envelope-based receiver.

In RAN4#112 meeting, for the ideal SNR evaluation we use the bitmap scheme, i.e., CRC is adopted. In the last RAN4#112bis meeting, companies commented that few details of waveform generation are not specified in RAN1, then RAN4 should not move forward on SNR evaluations.

However, RAN1 may not specify all the details, and some of the schemes are left to companies’ implementation choice, RAN4 could not wait for RAN1 to fix each detailed aspect for RAN4 requirements simulation. In RAN4#113 meeting, we also performed simulations to compare the SNR performance with CRC and non-CRC (codepoint scheme) cases, the results show that there is only less than 0.5dB difference. Therefore, even though some detailed schemes for waveform generation and detection is different among companies, the OOK-4 M=4 non-CRC results as worst case could be selected as SNR target.

Figure 3 OOK-4 M =4 SNR for codepoint scheme (non-CRC)

Observation 2: The SNR performance with CRC and non-CRC case is less than 0.5dB difference. non-CRC result is a worse case which could be selected as SNR target.

In this meeting, we further perform additional simulations with consideration of timing error. Timing error aspect is considered in RAN1 simulation with the following typical assumptions:

- For OOK-based, the timing error is 5/3/1us for OOK M=1/2/4

- For OFDM-based, the timing error is 0.9 and 1.86us

We consider the above timing error assumption and get the following new simulation results for OOK-based receiver:

Figure 4 Timing error and CFO impacts on OOK-based LP-WUS SNR

Observation 3: Frequency error has minor impacts on OOK-based SNR performance. But timing error has big impacts on OOK-based LP-WUR (1us typical timing error will get ~1.5dB worse SNR).

We also check the timing error impacts on OFDM-based receiver:

Figure 5 Timing error and CFO impacts on OFDM-based LP-WUS SNR

Observation 4: Frequency error and timing error also have impacts on OFDM-based receiver. High data rate (M=4) is more sensitivity to timing error (1us timing error will get ~1dB worse SNR); Low data rate (M=1) is more sensitivity to Frequency error (10ppm will get ~1.5 dB worse SNR).

In last RAN4 meeting, companies have shared the analysis of SNR, and the potential SNR value is summarized in the WF [3]:

With above analysis, we believe that the target SNR value should not consider the impact of timing error and frequency error, instead those potential performance degradation should be included in IM or NF elements when specify the REFSENS requirements.

Proposal 1: Select -2dB for OOK-based LP-WUR and -5dB for OFDM-based LP-WUR as target SNR when discuss REFSENS requirements.

Proposal 2: The introduced performance degradation (1~1.5dB) by timing error and frequency error should be considered into IM or NF, when specify the REFSENS requirements.

  - ASCS requirements

Using the agreed simulation parameters in [1], the ASCS link level simulation results are summarized in Table 2.

Table 2 SNR performance degradation for ASCS (without guard RBs inside)

| modulation | Chip rate | payload | receiver | Freq | Freq offset | Filter order | ASCS SNR @ 1% MDR (with in-band NR) | target SNR@1%MDR (no in-band NR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 16bit | Envelope | 900 MHz | The results for different offset are similar | The results for different filter are similar | -2.9 dB | -3.0 dB |
|  |  |  |  | 2.6 GHz |  |  |  |  |
|  |  |  |  | 3.5 GHz |  |  |  |  |
|  |  |  | OFDM | 900 MHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.7 dB | -7.7 dB |
|  |  |  |  |  |  | 5th | -8.0 dB | -8.0 dB |
|  |  |  |  |  | 20 ppm | 3rd | -6.0 dB | -6.0 dB |
|  |  |  |  |  |  | 5th | -6.2 dB | -6.2 dB |
|  |  |  |  | 2.6 GHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.5 dB | -7.5 dB |
|  |  |  |  |  |  | 5th | -7.7 dB | -7.7 dB |
|  |  |  |  |  | 20 ppm | 3rd | -5.0 dB | -5.0 dB |
|  |  |  |  |  |  | 5th | -5.1 dB | -5.1 dB |
|  |  |  |  | 3.5 GHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.5 dB | -7.5 dB |
|  |  |  |  |  |  | 5th | -7.5 dB | -7.5 dB |
|  |  |  |  |  | 20 ppm | 3rd | -5.0 dB | -5.0 dB |
|  |  |  |  |  |  | 5th | -5.0 dB | -5.0 dB |
| OOK-4 | M = 4 | 16bit | Envelope | 900 MHz | The results for different offset are similar | The results for different filter are similar | -2.3 dB | -2.3 dB |
|  |  |  |  | 2.6 GHz |  |  |  |  |
|  |  |  |  | 3.5 GHz |  |  |  |  |
|  |  |  | OFDM | 900 MHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -6.1 dB | -6.1 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -5.9 dB | -5.9 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  | 2.6 GHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -5.5 dB | -5.5 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -3.8 dB | -3.8 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  | 3.5 GHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -5.1 dB | -5.1 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -2.2 dB | -2.2 dB |
|  |  |  |  |  |  | 5th |  |  |

In Table 2, we don’t differentiate the performance under different guard RB (0/1/2) and the reason is that the benefit of guard RB is not significant for ASCS.

Observation 5: The benefit of guard RB in ASCS case is minor, and the impact of adjacent NR signal on the WUS sensitivity is negligible.

It was agreed tentatively that 0 guard RB is needed for ASCS, but the detailed test case is not decided.

Proposal 3: RAN4 can define the ASCS test parameters as following in Table 3. Make decision on whether margin is needed for LP-WUS power given no guard RB is allocated.

Table 3: ASCS parameters for LP-WUR

| RX parameter | Units | LP-WUS bandwidth (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth configuration | dBm | REFSENS+TBD |
| Pinterferer,NR | dBm | Same PSD as LP-WUS |
| Guard RB on each side of LP-WUS signal |  | 0 |
| NOTE: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs. |  |  |

  - ACS requirements

2.3.1 ACS High-signal and High interference approach (similar to MR)

For ACS, the simulation is performed under a similar condition of MR, i.e., a high LP-WUS SNR condition of 14dB+LP-WUS REFSENS. Based on the discussions, most companies believe that the interference condition of ACS for LR and MR is nearly the same, so in our simulation we use the same interference level as MR for LP-WUS ACS evaluation.

Our simulation results were provided in the previous RAN4 meeting in [6], and we identified that OOK4 M=4 is also the worst case of ACS. We provide the simulation results for the required RB numbers under different increased interference power level on top of LR REFSENS which vary from 20dB to 33dB. Since the MR and LR are implemented in the same UE, the common understanding is that they would suffer from same interference condition, and from this perspective, it is unnecessary for LR to endure same increased interference power ratio.

Due to the potential worse REFSENS performance of LR, the required interference power only about 17.7~22.7 dB on top of the LR REFSENS, which also depends on the LR SNR, IM difference and Noise Figure.

Observation 6: under same interference power level of MR, the required ACS of LR will not exceed 23 dB.

Based on the phase noise model options listed in previous WF, we keep using the phase noise model from IEEE [7]. We model this profile in the simulation and mimic the reciprocal mixing by multiplying the phase noise with the received signal in the time domain, as shown in Figure 6.

Figure 6: Phase noise profile and LP-WUS signal before/after reciprocal mixing

After adding the phase noise, the simulation results of ACS are shown in Figure 7.

Figure 7: Updated ACS results with consideration of phase noise model (LP-WUS level is fixed at REFSENS+14dB)

The results show that with 23 dB higher interference power level (the interference level is same as MR condition) 1 guard RB is still enough when the phase noise impact of reciprocal mixing is considered.

Observation 7: After taking the impact of reciprocal mixing of phase noise, 1 guard RB is still enough for WUR under similar MR interference level.

With above analysis and observations, due to potential worse sensitivity of LP-WUR and similar interference level, then a different ACS requirement for this new type of receiver is needed, e.g., ACS=20~23dB.

Observation 8: The ACS requirements for LP-WUS should be defined within 20~23dB (signal level is LP-WUS REFSENS+14dB and similar interference power level as MR), with number of guard RB as 0~1.

Proposal 4: LP-WUS ACS requirements can be specified as: ACS 23dB with Guard RB as 1, or 20dB with guard RB as 0. The ACS requirements and test case is defined in Table 4 and Table 5.

Table 4: Adjacent channel selectivity

| Rx Parameter | Units | LP-WUS bandwidth (11 RB) |
| --- | --- | --- |
| ACS | dB | [23] |

Table 5: Test parameters for Adjacent channel selectivity, Case 1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 11 RB LP-WUS within 10 MHz NR CBW |
| Power in transmission bandwidth configuration | dBm | LR REFSENS + 14 dB |
| Guard RB |  | [1] |
| Pinterferer | dBm | LR REFSENS + [35.5] dB |
| BWinterferer | MHz | 5MHz |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: The guard RB is placed at single side of LP-WUS, between LP-WUS and interferer. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. |  |  |

2.3.2 ACS Low signal level and low interference level approach

Last RAN4 meeting, there are also simulation results proposed to define the ACS requirement by using sensitivity-level signal but lower down the interference power level [9]. Just for comparison, we also perform the simulation with the same condition, and the following Table 4 show our simulation result. The waveform is OOK-4 M=4.

Table 4 SNR vs gurad RB under different interference power level

| Phase Noise | Interferer (dB) | Guard RB and SNR |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 4 |
| w/ | 31.5 | N/A | N/A | N/A |
|  | 26.5 | N/A | N/A | 2.04 |
|  | 19.5 | 0.67 | -1.35 | -2.13 |
|  | 13.5 | -2.20 | -2.27 | -2.40 |
|  | 7.5 | -2.41 | -2.41 | -2.44 |
|  | -INF | -2.55 | -2.55 | -2.55 |
| w/o | 31.5 | N/A | N/A | N/A |
|  | 26.5 | N/A | N/A | -2.1 |
|  | 19.5 | -1.45 | -2.20 | -2.51 |
|  | 13.5 | -2.48 | -2.52 | -2.54 |
|  | 7.5 | -2.52 | -2.54 | -2.55 |
|  | -INF | -2.55 | -2.55 | -2.55 |

The simulation shows that when phase noise is added, the receiver can achieve -2 dB SNR with 0 guard RB if the interference power is around 15 dB, but 4 guard RB is needed if the interference is around 20 dB. By defining the ACS with this approach, more guard RB is needed which would force the WUS signal allocated in the center of channel. Besides, due to the low interference power level, the receiver performance may not be guaranteed in many cases in the real network.

  - REFSENS requirements

The following agreements for REFSENS have been agreed last meeting [3]:

Agreements:

  - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported.

  - The total value of NF+IM can be considered.

  - On how to handle the NF values:

    - Option 1: For NF whether average value can be adopted.

    - Option 2: two sets NFs for different LP-WUR types with different power consumption target

    - Option 3: also consider coverage

  - Capture the table in WF as background information

Given the number of RB for LP-WUS is a fixed value, i.e., 11 RBs with 15kHz/30kHz SCS, then single REFSENS for each band at each SCS is needed. In our understanding, rather than discussing the REFSENS value per-band for LP-WUS, it is better to use 10 MHz MR REFSENS (for many bands, 10MHz is the min CBW with 30kHz SCS) as a reference to discuss the offset value, it could be a more efficient way to finalize requirements for all the bands.

Observation 9: Given fixed number of RBs for LP-WUS signal, the REFSENS is single value for each band (with each SCS). Instead of discussing the REFSENS value per-band for WUS, it is better to use MR REFSENS at 10MHz with 30kHz SCS in Table 7.3.2-1a as a reference to discuss the offset.

Proposal 5: For OOK-based LP-WUS REFSENS, the value for each key element is: SNR=-2dB, IM=2.5dB + [1]dB, diversity =0dB, NF=15dB.

Proposal 6: For OFDM-based LP-WUS REFSENS, the value for each key element is: SNR=-5dB, IM=2.5dB + [1]dB, diversity=0dB, NF=11dB.

The proposed spec update could be:

For an OOK-based LP-WUR UE equipped with 1 Rx antenna port, 10MHz reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3M.2-1 for LP-WUR at the applicable operating bands.

Table 7.3M.2-1: OOK-based LP-WUR Single antenna port reference sensitivity allowance ΔR1R

| Operating band | LP-WUS bandwidth (RB) | ΔR1R (dB) of 10MHz REFSENS |
| --- | --- | --- |
| TDD band | 11 | [TBD] |
| FDD band | 11 | [TBD] |

- Conclusion

In this contribution, we share our views on LP-WUR REFSENS, ASCS and ACS:

Observation 1: The worst case is OOK-4, M=4, payload = 16bit with envelope-based receiver, and the SNR without timing error and frequency error at 1% MDR is about -2dB. With the same modulation, the SNR performance of OFDM-based receiver is always several dBs better than Envelope-based receiver.

Observation 2: The SNR performance with CRC and non-CRC case is less than 0.5dB difference. non-CRC result is a worse case which could be selected as SNR target.

Observation 3: Frequency error has minor impacts on OOK-based SNR performance. But timing error has big impacts on OOK-based LP-WUR (1us typical timing error will get ~1.5dB worse SNR).

Observation 4: Frequency error and timing error also have impacts on OFDM-based receiver. High data rate (M=4) is more sensitivity to timing error (1us timing error will get ~1dB worse SNR); Low data rate (M=1) is more sensitivity to Frequency error (10ppm will get ~1.5 dB worse SNR).

Observation 5: The benefit of guard RB in ASCS case is minor, and the impact of adjacent NR signal on the WUS sensitivity is negligible.

Observation 6: under same interference power level of MR, the required ACS of LR will not exceed 23 dB.

Observation 7: After taking the impact of reciprocal mixing of phase noise, 1 guard RB is still enough for WUR under similar MR interference level.

Observation 8: The ACS requirements for LP-WUS should be defined within 20~23dB (signal level is LP-WUS REFSENS+14dB and similar interference power level as MR), with number of guard RB as 0~1.

Observation 9: Given fixed number of RBs for LP-WUS signal, the REFSENS is single value for each band (with each SCS). Instead of discussing the REFSENS value per-band for WUS, it is better to use MR REFSENS at 10MHz with 30kHz SCS in Table 7.3.2-1a as a reference to discuss the offset.

Proposal 1: Select -2dB for OOK-based LP-WUR and -5dB for OFDM-based LP-WUR as target SNR when discuss REFSENS requirements.

Proposal 2: The introduced performance degradation (1~1.5dB) by timing error and frequency error should be considered into IM or NF, when specify the REFSENS requirements.

Proposal 3: RAN4 can define the ASCS test parameters as following in Table 3. Make decision on whether margin is needed for LP-WUS power given no guard RB is allocated.

Proposal 4: LP-WUS ACS requirements can be specified as: ACS 23dB with Guard RB as 1, or 20dB with guard RB as 0. The ACS requirements and test case is defined in Table 4 and Table 5.

Proposal 5: For OOK-based LP-WUS REFSENS, the value for each key element is: SNR=-2dB, IM=2.5dB + [1]dB, diversity =0dB, NF=15dB.

Proposal 6: For OFDM-based LP-WUS REFSENS, the value for each key element is: SNR=-5dB, IM=2.5dB + [1]dB, diversity=0dB, NF=11dB.

# References

- R4-2414309, WF on LP-WUS UE RF requirements, vivo, RAN4#112

- R4-2417112, WF on UE RF requirements for LP-WUS, vivo, RAN4#112bis

- R4-2420528, WF on UE RF requirements for LP-WUS, vivo, RAN4#113

- R4-080125,  TP Receiver Requirements: REFSENS, ACS and blocking, Ericsson, RAN4#46

- R4-1709564, Preliminary SNR values for NR UE REFSENS, Huawei, Hisilicon, NR ad-hoc #3, 2017

- R4-2412059, Discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112

- IEEE802.11-17/0326r0, WUR Phase Noise Model Follow-Up

- R4-2415780, Further discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112bis

- R4-2415201, Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS, Huawei, HiSilicon, RAN4#112bis