3GPP TSG-RAN WG4 Meeting #114		R4-2500759

Athens, GR, 17th – 21st February, 2025

Agenda item:	 7.26.2.4

Source:	 vivo

Title:	Discussions on LP-WUS Testability

Document for:	 Approval

- Introduction

This contribution shares our views on LP-WUS RF requirements testability issues.

- LP-WUS UE RF testability

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing

Agreements:

  - The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested.

Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance

Agreements:

  - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point:

    - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance.

    - FFS if the MR should only be woken up when data transfer the system simulator is needed.

    - Other options are not precluded.

RAN4 make good progress on LP-WUS testability in last meeting, it was agreed generally that the MDR counting could be achieved without waking up the MR each time [1]:

However, there are still few open aspects:

- How many samples are sufficient to conclude the statistical performance of 1% MDR verification?

- Based on RAN1 discussion, the LP-WUR would have different capability to support connection mode or ideal/inactive mode, then how to verify the LP-WUR under a single RRC state?

- How to design the RMC for LP-WUS requirements?

In our understanding, to meet the statistics of results, for simulation evaluation typically we use 10000 samples to count the MDR value. However, it is too large for real measurements, a reasonable value as the LP-WUS samples to verify LP-WUS MDR is important. Considering 1% MDR is a very strict requirements, at least 1000 samples is need at each power level for Rx requirements.

Proposal 1: At least 1000 samples are needed to ensure the statistics of 1% MDR as RF performance verification.

Regarding the LP-WUS UE capability, RAN1 is discussing potential UE feature on whether LP-WUR would support only connected/idle mode. For which RRC state is selected as RF requirements verification, RAN4 can wait for RAN1 progress.

Proposal 2: RAN4 can wait for RAN1 progress on LP-WUS feature design and further decide which RRC state can be selected for RF performance verification.

For the last issue about how to define the RMC parameters, given some of the details for signal design are still under discussion in RAN1, RAN4 can keep on hold of this topic.

- Conclusion

In this contribution, we share our views on LP-WUR testability issue.

Proposal 1: At least 1000 samples are needed to ensure the statistics of 1% MDR as RF performance verification.

Proposal 2: RAN4 can wait for RAN1 progress on LP-WUS feature design and further decide which RRC state can be selected for RF performance verification.

# References

- R4-2420528, WF on UE RF requirements for LP-WUS, vivo, RAN4#113