3GPP TSG-RAN WG4 Meeting #114		R4-2500760

Athens, GR, 17th – 21st February, 2025

Agenda item:	 7.26.3

Source:	 vivo

Title:	Discussions on LP-WUS BS RF

Document for:	 Approval

- Introduction

This contribution shares our views on LP-WUS BS RF requirements.

- LP-WUS UE RF testability

Issue 1-1: Concept of LP-WUS power boosting

- Adhoc WF

  - “The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition.

    - FFS whether other cases can be additionally considered. E.g., single value declaration.

#### Issue 1-2: How to comprehend the de-boost on the NR transmission power

- WF

  - Consider the de-boosting behaviour together with other relevant issues

    - Companies are encouraged to think about the draft spec language to reflect corresponding proposals

#### Issue 1-3: Core requirement for LP-WUS power boosting

- WF

  - FFS the following options

    - Option 1: Leave the power boosting level to manufacturer declaration

    - Option 2: If the manufacturer declares power boosting, the minimum LP-WUS power boosting level should be X (>0) dB for Y MHz CBW

    - Other options are not precluded

RAN4 make good progress on LP-WUS BS RF last meeting, in WF [1]:

It has been concluded that the concept of power boosting is the average power of LP-WUS REs vs all in-band REs, this is similar to NB-IoT power boosting concept. To ensure the performance of LP-WUS, a requirement for power boosting should be specified.

Based on the good analysis in [2], it is observed that the LP-WUS power boosting level depends on CBW and acceptable power degradation of NR RBs. With ~1dB power degradation, the power boosting for LP-WUS can be about 3dB with 10kHz SCS in 10MHz CBW and 30kHz SCS in 20MHz CBW.

Therefore, for issue 1-3, we proper Option 2, the min LP-WUS power boosting level can be 3dB for 20MHz CBW with 30kHz SCS and 10MHz CBW with 15kHz SCS.

Proposal 1: Consider min LP-WUS power boosting level as 3dB for 20MHz CBW with 30kHz SCS and 10 MHz CBW with 15kHz SCS.

Regarding issue 1-2, how to handle the de-boost behavior on the NR transmission power, based on the discussions last meeting there would be several solutions, e.g., weather de-boost NR RBs or blank some RB resources can be BS implementation without specified mechanisms.

Proposal 2: Consider the detailed behavior of NR de-boosting as BS implementation.

- Conclusion

In this contribution, we share our views on LP-WUR BS RF.

Proposal 1: Consider min LP-WUS power boosting level as 3dB for 20MHz CBW with 30kHz SCS and 10 MHz CBW with 15kHz SCS.

Proposal 2: Consider the detailed behavior of NR de-boosting as BS implementation.

# References

- R4-2419802, Way Forward for [113][308] NR_LPWUS, Huawei, RAN4#113

- R4-2417860, Further discussion on BS RF requirements for LP-WUS, CATT, RAN4#113