3GPP TSG-RAN WG4 Meeting # 114	R4-2500768

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion of UE RF impact for LB-LB switching

Agenda Item:	7.8.2

Document for:	Approval

Introduction

In RAN#106, a new WID [1] was approved on Low band carrier aggregation via switching. A motivation paper including some pictures was provided in [2].

This paper provides an initial impact analysis on the UE RF impact.

Discussion

UE RF General

In the objective part of [1], a number of items has been defined to introduce physical layer procedures and requirements to enable low band carrier aggregation via switching. Among them, the key part related to RF requirements are:

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1]

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

-	RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary

-	Specify the switching delay and time mask for carrier switching [RAN4]

Currently, the switching delay and time mask was identified as tentative requirements that might be needed to be considered. The switching gap is more closely tied to physical layer procedures that would be further discussed in RAN1.

Observation 1: In the WID, switching delay and time mask were identified as tentative requirements that might be needed to be considered in RAN4.

For Case 1 and case 2, the following figure from [2] was also partly copied here, with TTI N1 as Case 1 and TTI N2 as Case 2:

It should be noted the two cases for switching can also be interpreted as two states:

- State1: The UE operates in both Tx and Rx on the FDD band.

- State2: The UE operates only in Rx mode on the SDL band..

It should be noted that this “switching” is considerably different from the legacy “Tx Switching” studied in previous releases. In legacy context, “Tx Switching” is a switching between Tx chainswithout any involvement of the Rx functionality. The time mask requirements defined for “Tx Switching” are solely Tx operations. So, simply reuse existing requirements for “Tx switching” may not be a viable solution for case 1 and case 2.

Observation 2: Legacy “Tx switching” requirements does not involve the Rx part, which presents a significantly different situation compared to what is being discussed within the current WID scope.

In order to show the difference between the current switching mechanism and legacyTx switching, here is an example of Tx switching requirements as referenced from TS 38.101-1:

It can be seen again that the switching period currently defined as ON/OFF mask is for Tx only. Within the scope of this WID, the switching to case2 means that the SDL band would not be impacted by another FDD band for both Tx and Rx. As other example why this is important, take CA_n12A-n29A as an example, there would be severe Tx/Rx interference between n12 UL and n29 DL if not isolated properly.

Based on this spirit, further study is needed for the detailed reference architecture. The legacy Tx switching requirements {35us, 140us, 210us} may be used as starting point, however, considering that SDL link do not have Tx counterpart and do not need a stabilizing time, the switching time needed may be depending on Rx part rather than Tx. Thus, no further proposal should be provided before a detailed analysis on the UE architecture and implementation.

Proposal 1: Further study on the reference architecture is needed to investigate on the UE implementation of switching, which involves both Tx and Rx.

In addition, the testability need discussion, since the case 2 (SDL only) do not have any Tx and no ON power could be measured. Currently we do not have mature established methods to check whether Rx chain is on or not. The usual way to test receiver by means of demodulation may face challenges to do verification in the order of Micro seconds.

Proposal 2: The testability of timing of receiver ON period may need further discussion.

Capability

The definition of UE capabilities are also included in the WID scope as following:

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

Apart from the basic capability to support this feature in a per BC basis, there are also possible capabilities on different switching performance, and also others. However, these are suggested to be study more after more study and can be left to latter stage.

Proposal 3: The initial thinking is the capabilities can be on a per BC basis and may roughly categorized as:

- Basic support of switching capability

- Switching delay

- Others.

Proposal 4 The capability part can be discussed later when more study is done.

Others

In the WID scope, some other conditions are set as following:

-	Consider the following deployment constraints:

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers

Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach

Note 2: Strive to minimize the RAN1 impact

Those might be useful for when more details are discussed.

Conclusion

In this paper, a preliminary analysis has been done for Low band carrier aggregation via switching. The following observations and proposals have been provided:

Proposal 1: Further study on the reference architecture is needed to investigate on the UE implementation of switching, which involves both Tx and Rx.

Proposal 2: The testability of timing of receiver ON period may need further discussion.

Proposal 3: The initial thinking is the capabilities can be on a per BC basis and may roughly categorized as:

- Basic support of switching capability

- Switching delay

- Others.

Proposal 4: The capability part can be discussed later when more study is done.

- References

[1] RP-243317, New WID on low band carrier aggregation via switching, Apple, RAN#106

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, RAN#106