3GPP TSG-RAN4 Meeting #114 	R4-2500781

# Athens, Greece, 17 -21 Feburary, 2025

Source: 	Huawei, HiSilicon

Title:  	Ambient IoT general aspects

Agenda Item:	7.24.2.1

Document for:	Approval

- Introduction

RAN WGs have completed a Rel-19 SI on solutions for Ambient IoT in NR, documented in TR 38.769. Subsequently, a new WI on solutions for Ambient IoT in NR was approved at RAN#106 [1]. The general scope is outlined below:

Copied from WID RP-243326 [1]:

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

- Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command).

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

The RAN4 objectives are as follows:

Copied from WID RP-243326 [1]:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the Ambient-IoT general aspects.

- Discussion

  - Operating band

According to the WID [1], Ambient IoT uses FR1 licensed spectrum in FDD. NR band n8 can be used as an example band.

Other FDD bands are not precluded, such as sub 1G bands.

  - Scenario

Rel-19 A-IoT WI focus on the D1T1-B indoor scenarios. Based on SI coexistence findings[2]:

In D1T1-B Scenario 2-1 (without legacy NR indoor UEs), coexistence is feasible, and no additional measures are needed.

In D1T1-B Scenario 2-2 (with legacy NR indoor UEs), significant performance degradation is observed for both the A-IoT and NR systems." For this case, some existing mechanisms can be considered to avoid/mitigate the interference, e.g. handover NR UEs to a different cell.

Copied from TR 38.769 [2]:

6.6.5.10	General remarks on coexistence findings

For D1T1 deployment scenario 1-1 (R2D, CW2D/D2R: DL) and 2-1(R2D: DL, CW2D/D2R: UL), which has no legacy NR UE indoor. Negligible performance degradation for NR system is observed. Less than 1 dB SINR degradation due to inter-system interference from NR to ambient IOT system is observed. Co-existence between ambient IOT and NR is feasible based on performance metric of NR throughput degradation and ambient IOT SINR degradation, no additional coexistence measures are required. Ambient IOT intra-system performance evaluated by link level simulation is also one of the key aspects affecting ambient IOT system performance, which will have impact to the outage of ambient IOT system.

For the deployment scenarios with legacy NR UE indoor (scenario 1-2, 2-2, 5-2, 6-2), which either significant NR throughput degradation or significant SINR degradation for ambient IOT system is observed, some existing mechanisms based on gNB implementation can be considered to avoid/mitigate the interference. For example, for RRC idle mode, NR UEs can camp on a frequency different from ambient IOT system based on frequency priority configured by network; for RRC connected mode, network can handover NR UEs to a different cell (e.g. indoor gNB on a different frequency) based on existing measurement report triggering and event. Besides, other interference management solutions to reduce the inter-system interference from ambient IOT to proximity NR UE and vice versa can also be considered.

Since existing mechanisms can help avoid or mitigate the interference for indoor NR UE scenarios, imposing overly strict requirement on A-IoT BS is neither necessary nor practical. Therefore, when defining A-IoT BS requirements, it is sufficient to consider only outdoor legacy NR UEs.

Proposal 1: Consider only outdoor legacy NR UEs when defining A-IoT BS requirements in R19.

  - R2D Channel bandwidth

The R2D waveform is agreed to be OFDM-based waveform. The R2D channel bandwidth can be defined as one or multiple PRBs with guardband if necessary.

According to the SI conclusions, the R2D channel bandwidth includes at least four types, as captured in TR38.769 [2].

Copied from TR38.769[2]:

6.1.1.4	R2D bandwidths

…

Table 6.1.1.4-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value.

Note:	Depending on further study, the maximum value of M may be less than 32.

Note:	The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint.

Table 6.1.1.4-1: Starting point for M values and the associated minimum Btx,R2D value

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

RAN1 is likely to concentrate on M values (M value is how many chips in per OFDM symbol transmission). For R2D, a single M value can correspond to 1 to 12 PRBs, meaning the channel bandwidth isn't directly tied to a specific M value.

R2D channel BW is necessary for reserve NR PRB when co-employment. And Many TX and RX RF requirements are related with channel bandwidths, such as occupied bandwidth, ACLR and Reference sensitivity level. It is need to define R2D CBW. For D2R, 4 types of R2D channel bandwidths corresponding to transmission bandwidth of 1PRB to 4 PRBs can be defined.

Proposal 2: Specify 4 types of R2D channel bandwidths corresponding to transmission bandwidth of 1PRB to 4 PRBs.

  - D2R Channel bandwidth

D2R channel bandwidth has no conclusion in SI.

RAN1 is likely to concentrate on chip length of D2R.

Copied from R1-2407669

Table 3. Chip length and the corresponding transmission bandwidth of Option 2

| Index | Transmission bandwidth (double sideband) | Chip length |
| --- | --- | --- |
| 0 | 15kHz | 133.33 µs |
| 1 | 30kHz | 66.67 µs |
| 2 | 60 kHz | 33.33µs |
| 3 | 150 kHz | 13.33 µs |
| 4 | 300 kHz | 6.67 µs |
| 5 | 600 kHz | 3.33 µs |
| 6 | 1200 kHz | 1.67 µs |
| 7 | 2400 kHz | 0.83 µs |

For D2R channel BW, SFO, FDMA and filter performance may have an impact.

Proposal 3: When defining the D2R channel bandwidth, consider the impacts of SFO, FDMA and the filter performance.

  - Channel arrangement

For NR, the nominal channel spacing between two adjacent A-IoT carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

For A-IoT, however, each BS supports only a single carrier because tags cannot distinguish between multiple carriers from the same BS. Additionally, A-IoT BSs do not support carrier aggregation, so defining channel spacing for A-IoT is unnecessary.

Proposal 4: No need to define channel spacing for A-IoT

Proposal 5: Channel raster for A-IoT BS is 10kHz.

  - Foffset

As described in TS36.104, for NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset. A-IoT with one PRB transmission bandwidth is with similar narrow bandwidth, it can be applied to A-IoT. The frequency offset Foffset. values for Other CBW larger than 1PRB can be FFS.

Copied from TS 36.104

For NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset  as defined in Table 5.6-3A.

Table 5.6-3A: Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

Copied form TR36.802

Figure 7.1.6.2-1: ACLR for NB-IoT standalone

Proposal 6: Apply the Foffset for Ambient IoT as Table 1:

Table 1: Foffset for Ambient IoT

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Ambient IoT for 1PRB CBW | 200 kHz |
| Note: Other CBW larger than 1PRB is FFS |  |

- Summary

In this contribution, we give our consideration on the A-IoT general aspects, and have the following proposals:

Proposal 1: Consider only outdoor legacy NR UEs when defining A-IoT BS requirements in R19.

Proposal 2: Specify 4 types of R2D channel bandwidths corresponding to transmission bandwidth of 1PRB to 4 PRBs.

Proposal 3: When defining the D2R channel bandwidth, consider the impacts of SFO, FDMA and the filter performance.

Proposal 4: No need to define channel spacing for A-IoT

Proposal 5: Channel raster for A-IoT BS is 10kHz.

Proposal 6: Apply the Foffset for Ambient IoT as Table 1:

Table 1: Foffset for Ambient IoT

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Ambient IoT for 1PRB CBW | 200 kHz |
| Note: Other CBW larger than 1PRB is FFS |  |

- References

[1]RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei), RAN#106 Dec.,2024

[2]3GPP TR 38.769 V2.0.0(2024-12), Study on solutions for ambient IoT (Internet of Things)

[3]R1-2407669, On general aspects of physical layer design for Ambient IoT, Huawei, HiSilicon, RAN1#118bis October, 2024