3GPP TSG RAN WG4 Meeting #114	R4-2500823

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.19.3

Source:	vivo

Title:	Discussion on RRM core requirements for beam management

Document for:	Discussion

- Introduction

In RAN4#113 meeting, there were some further discussions on key issues including KPIs related, the update on simulation assumption of measurement error, and the potential RRM requirements for beam prediction. The latest outcome was recorded in the latest WF [1].

In this contribution, we will further analyze the impacts on requirements for beam prediction and update some simulation results on newly proposed KPIs/ Test Metrics with consideration of the measurement error modelling derived from the updated simulation assumption [2].

- Requirements for beam predictions

In RAN4#112bis, a discussion on the RRM impacts on beam management was triggered. The package of requirements for beam predictions was initially identified as follows. In RAN4#113, there were some further agreements reached. Based on the latest agreements, we will further provide our analysis of these potential requirements.

Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed

  - Prediction delay requirements

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

For the latency on prediction, the above agreement was reached in the last meeting. It was agreed that RAN4 would define the requirement for an overall delay. Next, we will analyze these two parts of delay in more detail.

- Measurement for prediction

This process refers to UE performing the L1-RSRP measurements for set B and obtaining the corresponding results as model inputs. Basically, the legacy measurement period on L1-RSRP can be as the reference for defining requirements on measurements of set B.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

To be specific, the related parameters to be considered at least include:

- The number of samples for set B measurements

The number of samples will affect the prediction accuracy. For spatial prediction, RAN4 needs to further discuss whether the measurement of set B is based on one-shot measurement or measurement of multiple sample numbers. For time domain prediction, more discussion is needed on how to define the corresponding delay with consideration of characteristics of time dimension prediction, e.g., the number of samples corresponding to the latest measurement instances and the interval between time instances. The corresponding values depend on the network configuration. Configurable values need to be further discussed by RAN1/2.

- Beam sweeping factor

For DL Tx beam prediction evaluations, RAN1 had some preliminary discussions and simulations of the hypothesis on the Rx beam as AI/ML model input for training and/or inference during the SI phase, including:

- Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample.

- Option 2: Measurements of specific Rx beam(s).

- Option 3: Measurements of random Rx beam(s) per model input sample.

- Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping.

Among them, option 1 is implemented in the same way as the existing requirement definition, which requires beam sweeping to find the best Rx beam. The value of the existing beam sweeping factor can be reused. However, since RAN1 did not further discuss the Rx beam assumption in the SI and WI phases. It is possible to implement it as any of the above four options. Therefore, RAN4 may need to further determine the Rx beam assumption.

- sharing factor N

The value of this parameter may be related to factors such as measurement gap and SSB/SMTC period. It can wait until other factors are more clearly defined.

- inference delay

For model inference delay, the inference process is mainly from the data input to the model to the corresponding inference result is derived. Additionally, there may be some data processing time on top of that, which part of the delay may be related to the specific inference scenario (the number of Set B and Set A, time domain prediction or spatial prediction, etc.) and the processing capability of the model. It needs to be further discussed how to determine the value by RAN4.

Based on the above considerations, we have the following proposal:

Proposal 1: On the definition of prediction delay requirements for BM, RAN4 to consider:

- For Measurement for prediction, the current measurement period for L1-RSRP measurement can be the baseline and further discuss the details by considering the parameters, which at least include:

  - The number of samples for set B measurements (for spatial domain prediction); The number of the latest measurement instances and the time interval between two adjacent time instances (for time domain prediction)

  - beam sweeping factor

  - sharing factor N

- For inference delay: the reference latency and possible processing time

2.2	TCI state known/unknown

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

On whether the TCI state associated with a predicted Tx beam is known or unknown, the main divergence lies in whether the Rx beam is implicitly considered as known information for DL Tx beam prediction. In RAN4#113 meeting, RAN4 further discussed the issue of UE Rx beam knowledge and reached the further agreement as above. On how to know Rx beams information, it is somehow implementation dependent. In some implementations, UE first predicts the Tx beam and further determines the best Rx beam paired with the Tx beam by beam sweeping. For some UEs with specific model implementation, the information of Rx beam may be implicitly indicated or included as the model input, and then UE can directly know Rx beam corresponding to the best TX beam. Basically, it is not preferred to use the latter implementation as the default assumption for the DL Tx beam prediction.

To move forward, we suggest firstly identifying the case that no beam sweeping is needed after Tx beam prediction Regarding the relationship between Set A and Set B. There are two cases in RAN1 evaluation, including:

Case i): Set A and Set B are different (Set B is NOT a subset of Set A).

Case ii): Set B is a subset of Set A.

At the stage of data collection for inference, UE performs measurements on set B with a specific Rx beam assumption. As we mentioned in clause 2.1, there are several options of hypothesis on the Rx beam as AI/ML model input for training and/or inference. Based on RAN1 simulation, Option 1 with the measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample, AI/ML provides better performance than other methods. If we take Option 1 as the base assumption, when Set B is a subset of Set A and the best predicted Tx beam is in Set B, the Rx information should be known because the best Rx beam corresponding to Set B has been found by beam sweeping before the prediction. In this case, no need to perform beam sweeping again.

For other cases, e.g., the case that Set A and Set B are different or no beam sweeping is performed during data collection for training/inference, etc., RAN4 can further discuss the feasible methods to obtain Rx beam knowledge, e.g., introducing UE capability or making the reduced beam sweeping factor assumption.

Proposal 2: For the UE Rx beam knowledge,

- at least for the case that the Top-K beams to be reported are in set B and set B is a subset of set A, if beam sweeping has been done during measurements on set B, UE Rx beam information is known by UE and no further beam sweeping is needed.

- for other cases, e.g., set B is not a subset of set A, RAN4 to further discuss how to obtain Rx beam information, e.g., introducing UE capability or making the reduced beam sweeping factor assumption

2.3	Performance monitoring

The framework for performance monitoring is beginning to take shape after RAN1/2 discussion. For information, we copy the latest RAN1 conclusions on performance monitoring for BM as follows.

Agreements in RAN1#118bis - For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives - At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. Agreements in RAN1#119 - At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, the method of UE-assisted performance monitoring is supported, which is achieved by comparing the prediction results with the actual measurements results, calculating the final performance metric(s) and then reporting it to NW for assisting further decision-making on model management.

From RAN4 perspective, the requirements on performance monitoring shall be considered. However, before discussing the definition of requirements in detail, we believe that it needs to first figure out the testing goal for performance monitoring.

As we mentioned above, for UE-assisted performance monitoring, the set of monitoring actions, including the comparison between prediction results and actual measurement results and the evaluation by calculating performance metrics, are performed on UE side. From UE perspective, it is expected to correctly perform the whole monitoring process. As alternatives, the corresponding UE behavior can be verified from different perspectives as follows:

-	Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE

-	Option 2: The testing goal is to verify whether UE can report the performance metrics within a certain latency upon performance monitoring is required.

-	Option 3: both of the testing goals in Option 1 and 2 need to be verified

For Option 1, from NW perspective, the decision-making for subsequent actions (e.g., model deactivation, fallback) highly depends on the monitoring results of prediction performance. Therefore, the correctness of the performance metric reported by UE is a key point for verifying.

In terms of Option 2, it focuses on the performance metric that can be reported to the network within the specific latency and further ensures NW can make corresponding model management according to the results in time. Further details on reporting, e.g., when and how to report are still under discussion in RAN1.

For Option 3, both Option 1 and 2 are required.

Based on the above analysis, we propose to first figure out what is to be verified for performance monitoring and further work on the requirements design based on the agreed testing goal.

Proposal 3: For performance monitoring for BM, RAN4 to first determine the testing goal. The following alternatives can be considered:

- Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE;

- Option 2: The testing goal is to verify whether UE can perform monitoring and report the performance metrics within a certain latency upon performance monitoring is required;

- Option 3: The combination of Option 1 and Option 2.

  - KPIs related

KPIs on beam management were further discussed at the last meeting, and the alternatives being discussed by companies are summarized as follows:

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Issue 3-4: Metrics/KPIs for beam prediction Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Add new metric: - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived. - Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference). - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction - RSRP absolute accuracy applies to all predicted beams - Recommended WF - To be discussed

Next, we will conduct a detailed analysis of several representative alternative KPIs, mainly including the pros and cons of each KPI, simulation result evaluation, and other aspects.

Table 1 Analysis of the pros and cons of several representative KPIs

| Alt | Pros | Cons |
| --- | --- | --- |
| Alt 1: Top 1/K AND Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB | - Beam prediction accuracy and RSRP accuracy are combined: not only ensure that the predicted Top 1 beam is one of the strongest K beams in practice but also ensure that the difference between the RSRP of the reported beam and the RSRP of the actual strongest beam is within a certain range. | - To a certain extent, only the RSRP accuracy of Top-1 predicted beam was guaranteed. - If the RSRP is not included in the reference report, it cannot be calculated. |
| Alt 2: Top 1/K OR the difference of predicted RSRP between Top K and Top 1 is within [X]dB | - The Top K predicted beams are all considered. - By limiting the RSRP of the Top K predicted beams to be not too far different, all beams reported to the network can be used to a certain extent. | - If the RSRP is not included in the reference report, it cannot be calculated. - It is necessary to determine the X value, and it may be necessary to discuss the X value separately in different cases [8 predict 64, 16 predict 64] |
| Alt 3: Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams" | - Whether the RSRP is reported or not, only the beam index prediction accuracy is verified - The predicted Top K beams are all considered - Consider the observation that the RSRP corresponding to the actual Top 1 and Top N beam (N is less than or equal to 3) is not very different. | - It is necessary to determine the N value, and it may be necessary to discuss the N value separately in different cases [8 prediction 64, 16 prediction 64] - As can be seen from the simulation#1 in Table 1, when the Top-N genie-aided beam is one of the Top-K predicted beams, the prediction becomes very easy even when measurement error is considered, so that even if this KPI is met, it does not fully represent the true quality of these beams is good |

For the prediction performance of Alt 2, we provide detailed simulations with consideration of the impact of measurement error in Clause 3.

For further comparison, here, we also conducted simulation verification of Alt 3, and some simulation results are shown below.

Table 2 Evaluation results of prediction accuracy on Top K/N for BM-Case1 DL Tx beam prediction by considering measurement error

| Simulation results | Prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inference SNR | SNR-3dB |  |  | SNR-6dB |  | SNR0dB |  |
| Simulation#1: the percentage of the Top-N genie-aided beam is ONE of the Top-K predicted beams | No error | case 1 | case 2 | No error | case 3 | No error | case 4 |
| Top 2/2 | 92.18 | 91.57 | 86.02 | 90.56 | 89.23 | 93.24 | 92.48 |
| Top 2/3 | 96.09 | 95.26 | 90.44 | 94.12 | 93.29 | 96.11 | 95.18 |
| Top 2/4 | 98.24 | 97.51 | 93.73 | 97.51 | 97.02 | 98.56 | 97.63 |
| Top 2/5 | 98.64 | 98.23 | 95.50 | 98.26 | 98.09 | 99.15 | 98.48 |
| Top 3/2 | 94.17 | 93.33 | 89.64 | 92.13 | 91.05 | 95.10 | 94.25 |
| Top 3/3 | 96.73 | 96.22 | 92.93 | 95.19 | 94.53 | 97.13 | 96.28 |
| Top 3/4 | 98.48 | 98.07 | 95.74 | 98.43 | 98.34 | 98.90 | 98.06 |
| Top 3/5 | 98.80 | 98.71 | 96.71 | 98.92 | 99.01 | 99.32 | 98.82 |
| Top 4/2 | 95.37 | 94.78 | 92.29 | 93.62 | 93.12 | 96.37 | 95.77 |
| Top 4/3 | 97.37 | 97.03 | 94.78 | 96.27 | 95.69 | 97.80 | 97.38 |
| Top 4/4 | 98.80 | 98.47 | 96.95 | 99.17 | 98.92 | 99.15 | 98.65 |
| Top 4/5 | 98.96 | 98.88 | 97.67 | 99.34 | 99.42 | 99.49 | 99.24 |
| Top 5/2 | 98.00 | 97.83 | 95.02 | 96.44 | 96.27 | 98.56 | 98.22 |
| Top 5/3 | 98.96 | 98.71 | 97.35 | 98.01 | 97.60 | 99.41 | 99.15 |
| Top 5/4 | 99.44 | 99.28 | 98.55 | 99.59 | 99.50 | 99.75 | 99.58 |
| Top 5/5 | 99.60 | 99.52 | 99.12 | 99.67 | 99.83 | 99.83 | 99.75 |

In Table 2, we show the corresponding results on the percentage of the Top-N genie-aided beam is ONE of the Top-K predicted beams (Alt 3). As can be seen from the results of simulation#1, when the Top-N genie-aided beam is P(P=1) of the Top-K predicted beams, the prediction becomes easy (most of the prediction accuracy is above 95%) even if measurement error is considered. This may make the requirement too relaxed, so that even if this KPI is met, it does not fully represent the true quality of these beams is good. To further verify this observation, we compare RSRP difference between the predicted L1-RSRP of the Top 5 predicted beams and the ideal L1-RSRP of Top N (N=1,2,5, respectively) genie-aid beam in simulation#2, simulation#3 and simulation#4, respectively. As can be seen from the results in Table 3.

Table 3 90%-tile RSRP difference between the predicted L1-RSRP of the Top 5 predicted beams and the ideal L1-RSRP of Top N (N=1,2,5) genie-aid beam

| Simulation results | RSRP difference(dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inference SNR | SNR-3dB |  |  | SNR-6dB |  | SNR0dB |  |
| Simulation#2: 90%-tile RSRP difference between the predicted L1-RSRP of the Top 5 predicted beams and the ideal L1-RSRP of Top 1 genie-aid beam | No error | case 1 | case 2 | No error | case 3 | No error | case 4 |
| 1st beam | 1.10 | 1.54 | 2.36 | 1.23 | 1.58 | 1.02 | 1.21 |
| 2nd beam | 1.42 | 1.71 | 2.46 | 1.59 | 1.83 | 1.47 | 1.62 |
| 3rd beam | 2.18 | 2.48 | 3.06 | 2.33 | 2.63 | 2.46 | 2.48 |
| 4th beam | 3.98 | 4.28 | 4.91 | 4.25 | 4.42 | 4.42 | 4.48 |
| 5th beam | 12.14 | 12.11 | 12.05 | 12.04 | 11.94 | 12.56 | 12.51 |
| Simulation#3: 90%-tile RSRP difference between the predicted L1-RSRP of the Top 5 predicted beams and the ideal L1-RSRP of Top 2 genie-aid beam | No error | case 1 | case 2 | No error | case 3 | No error | case 4 |
| 1st beam | 1.06 | 1.53 | 2.38 | 1.19 | 1.65 | 0.96 | 1.24 |
| 2nd beam | 1.15 | 1.48 | 2.31 | 1.31 | 1.66 | 1.10 | 1.32 |
| 3rd beam | 1.75 | 2.10 | 2.77 | 1.94 | 2.25 | 1.96 | 2.04 |
| 4th beam | 3.78 | 4.04 | 4.51 | 3.94 | 4.10 | 4.05 | 4.11 |
| 5th beam | 11.80 | 11.73 | 11.73 | 11.69 | 11.53 | 12.18 | 12.23 |
| Simulation#3: 90%-tile RSRP difference between the predicted L1-RSRP of the Top 5 predicted beams and the ideal L1-RSRP of Top 5 genie-aid beam | No error | case 1 | case 2 | No error | case 3 | No error | case 4 |
| 1st beam | 3.97 | 4.20 | 4.71 | 4.05 | 4.28 | 4.22 | 4.24 |
| 2nd beam | 3.64 | 3.88 | 4.24 | 3.66 | 3.93 | 3.80 | 3.81 |
| 3rd beam | 2.98 | 3.16 | 3.48 | 3.04 | 3.21 | 3.13 | 3.18 |
| 4th beam | 2.68 | 2.88 | 3.19 | 2.75 | 2.91 | 2.95 | 2.98 |
| 5th beam | 9.62 | 9.55 | 9.61 | 9.44 | 9.55 | 10.18 | 10.20 |

Further, the corresponding CDF curve for RSRP difference the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of Top N(N=1,2,4,5) genie-aid beam are shown in Fig.1-1, Fig.1-2, Fig.1-3 and Fig.1-4, respectively.

| Fig.1-1 RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of Top 1 genie-aid beam | Fig.1-2 RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of Top 2 genie-aid beam |
| --- | --- |
| Fig.1-3 RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of Top 4 genie-aid beam | Fig.1-4 RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of Top 5 genie-aid beam |

Combined with Fig.1 and the above simulation results, we have the following observations:

- The RSRP difference between the ideal L1-RSRP of Top 1 genie-aid beam and Top 2 genie-aid beam is quite close and about 0.5dB. To some extent, this shows that even if there is no error, for the Top two beams, it is difficult for UE to say which beam is the real Top 1 when performing RSRP measurement. From this point of view, it is also reasonable to use the KPI of Top K/N to measure the accuracy of beam prediction.

- Except for the difference between the predicted L1-RSRP of the Top-5 predicted beams and the ideal L1-RSRP of Top N (when N=1,2) genie-aid beam reaches more than 10dB, the RSRP between the predicted L1-RSRP of the other (Top 1,2,3) predicted beams and the ideal L1-RSRP of Top N (when N=1,2) genie-aid beam are close.

- When K is 5, the predicted RSRP of the Top 5 is quite different from the actual RSRP corresponding to the Top 1/2 genie-aid beam. If the network uses the beam as the Tx beam for future use, the performance may be poor. This observation should be taken into account when defining metrics for scenarios that specify the value of K.

To summarize the points we proposed above, our observations are shown as follows:

Observation 1: For Top-K/N (%): the percentage of "the Top-N genie-aided beam is ONE of the Top-K predicted beams, the prediction becomes easy (most of the prediction accuracy is above 95%) even measurement error is considered. This may make the target too relaxed. Even if this KPI is met, it does not fully represent that the true quality of these beams is good

Observation 2: The KPI of Top K/N mitigates the effects that the RSRPs between Top 1 and Top 2 genie-aid beams are too close (within 0.5dB), which may cause the actual Top-1 beam may be indistinct

Besides, to explore the suitable value of P, we further simulated the case of P=2, corresponding to simulation#5 on the percentage of the Top-N genie-aided beam is P (P=2) of the Top-K predicted beams.

Table 4 Evaluation results of prediction accuracy on Top K/N for BM-Case1 DL Tx beam prediction by considering measurement error

| Simulation results | Prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inference SNR | SNR-3dB |  |  | SNR-6dB |  | SNR0dB |  |
| Simulation#5: the percentage of the Top-N genie-aided beam is TWO of the Top-K predicted beams | No error | case 1 | case 2 | No error | case 3 | No error | case 4 |
| Top 2/2 | 72.15 | 69.64 | 53.41 | 63.79 | 61.31 | 69.99 | 69.74 |
| Top 2/3 | 87.55 | 86.59 | 77.75 | 83.02 | 81.61 | 88.59 | 87.66 |
| Top 2/4 | 92.42 | 91.65 | 84.50 | 89.48 | 87.90 | 93.07 | 92.14 |
| Top 2/5 | 95.45 | 94.94 | 89.08 | 94.20 | 93.21 | 96.37 | 95.94 |
| Top 3/2 | 86.59 | 85.62 | 77.19 | 82.60 | 81.69 | 86.48 | 85.71 |
| Top 3/3 | 91.86 | 91.08 | 85.86 | 88.57 | 87.32 | 92.22 | 91.72 |
| Top 3/4 | 95.29 | 94.78 | 89.96 | 92.71 | 91.38 | 95.77 | 94.93 |
| Top 3/5 | 98.16 | 97.59 | 93.90 | 96.52 | 96.02 | 98.22 | 97.38 |
| Top 4/2 | 90.10 | 89.40 | 84.58 | 86.08 | 85.83 | 90.62 | 90.45 |
| Top 4/3 | 93.85 | 93.09 | 89.24 | 90.64 | 90.22 | 94.17 | 93.83 |
| Top 4/4 | 96.33 | 95.98 | 92.85 | 94.28 | 93.79 | 97.21 | 96.62 |
| Top 4/5 | 98.64 | 98.31 | 95.90 | 97.93 | 97.60 | 98.82 | 98.22 |
| Top 5/2 | 92.74 | 91.81 | 88.19 | 89.81 | 89.23 | 93.66 | 93.74 |
| Top 5/3 | 96.57 | 96.06 | 92.93 | 94.03 | 93.79 | 96.70 | 96.79 |
| Top 5/4 | 98.80 | 98.15 | 96.22 | 97.10 | 96.77 | 99.07 | 98.73 |
| Top 5/5 | 99.36 | 99.28 | 98.07 | 99.01 | 98.76 | 99.66 | 99.49 |

It can be seen that when P is 2, the prediction accuracy also decreases compared with P=1, whereas it can ensure the reliability of the reported beams to a certain extent.

Observation 3: Compared with the KPI of “the percentage of the Top-N genie-aided beam is ONE of the Top-K predicted beams”, the KPI of “the percentage of the Top-N genie-aided beam is TWO of the Top-K predicted beams” may be more reasonable for ensuring the quality of the reported beams

For the definition of performance requirements, RAN4 needs to finalize the KPIs to be used and the corresponding values. To be efficient, we think it is necessary to define a process for RAN4 discussion and alignment of results for requirement definition. Therefore, we propose the following process for further discussion:

Proposal 4: RAN4 to follow the process as above for the alignment of KPIs and definition performance requirements on BM prediction

- <Step 1> Discuss the definition of KPIs with the consideration of measurement error

  - <Step 1-1> Select one or more candidate KPIs for simulation

  - <Step 1-2> With the aligned simulation assumption, each company brings simulation results of candidate KPIs. Specifically, the measurement error modeling of simulation assumption may include

    - Option 1: only BB measurement error is considered

    - Option 2: only RF measurement error is considered

    - Option 3: both BB and RF measurement error are considered

  - <Step 1-3> Check on KPIs performance alignment from the simulation results from contributing companies

    - repeat simulations for 2-3 meetings and check if good alignment can be achieved

      - if good alignment of KPIs is achieved, move to step 3

      - if not, move to step 2

  - <Step 2> Discuss the definition of KPIs without the consideration of measurement error

    - <Step 2-1> With the aligned simulation assumption, each company brings simulation results of candidate KPIs

    - <Step 2-2> Discuss whether some margin be added to the aligned results of KPIs

    - <Step 2-3> Move to next step after alignment

  - <Step 3> RAN4 to make the final decision for KPIs definition and discuss the details of RRM requirements

In particular, for RSRP accuracy, since there was an initial alignment on the corresponding definition in RAN4#112bis and the following agreement was reached. For information, so we will analyse this further.

<RSRP accuracy>

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on ground-truth L1-RSRP].

For the FFS part in bracket [in top-K beams based on ground-truth L1-RSRP], we would like to put several possibilities on the table for further analysis.

- Definition 1: The index i may be any beam index

- Definition 2: The index i may be any beam index in top-K beams based on ground-truth L1-RSRP

- Definition 3: The index i may be any beam index in top-K beams based on predicted L1-RSRP

For definition 1, if we assume that the value of index i can range from 1 to N, where N is the total number of beams in set A, it means that the predicted RSRPs corresponding to all Beams of the predicted set A need to meet RSRP accuracy requirements. Given that only Top-K predicted beams and corresponding predicted RSRP (if needed) are required to report, for the remaining beams, whether the corresponding RSRP is predicted accurately may not be so critical. From this point of view, the definition 1 is not preferred.

For definition 2, if the predicted beams selected to meet accuracy requirement are based on ground-truth L1-RSRP, there may be a case to be noticeable as follows.

Case: The Top-K beam based on predicted L1-RSRP (e.g., beam X) is not any of the Top-K beams based on ground-truth L1-RSRP

To explore the probability of this case occurring in actual predictions, we have some simulations and the results provided below. Table 5 shows the observation that there is a certain possibility that the Top-K beam based on predicted L1-RSRP is not any of the Top-K beams based on ground-truth L1-RSRP. For example, for the Top-index(index=2) beam based on predicted L1-RSRP, the probability that this beam is one of the Top-2 beams based on ground-truth L1-RSRP is 72.15%. Further, if the measurement error is considered in model input, the probability of this happening will be lower.

Table 5 the probability that Top-index predicted beam is one of the Top-K(K=2/5) genie-aided beams

| Simulation results |  | Probability (%) |  |  |
| --- | --- | --- | --- | --- |
| Inference SNR |  | -3dB |  |  |
| Predicted Top-index Beam | the Top-index predicted beam is one of the Top-2 genie-aided beams | No error | Case 1NOTE | Case 2 NOTE |
| Index=1 |  | 92.18 | 86.02 | 91.57 |
| Index=2 |  | 72.15 | 53.41 | 69.64 |
| Predicted Top-K Beam | the Top-index predicted beam is one of the Top-5 genie-aided beams | No error | Case 1 | Case 2 |
| Index=1 |  | 99.60 | 99.12 | 99.52 |
| Index=2 |  | 99.36 | 98.07 | 99.28 |
| Index=3 |  | 97.21 | 93.33 | 96.79 |
| Index=4 |  | 88.91 | 83.29 | 88.59 |
| Index=5 |  | 71.43 | 63.29 | 70.12 |
| NOTE: The case 1 and case 2 refer to the measurement error modelling described in Clause 3 |  |  |  |  |

Observation 4: There is a certain probability that the Top-K predicted beam is not any of the Top-K beams based on ground-truth RSRP.

In this case, according to definition 2, the predicted L1-RSRP of beam X is not required to satisfy accuracy requirement as this beam is not any of the Top-K beams based on ground-truth L1-RSRP. However, the truth is that the corresponding predicted L1-RSRP of beam X still needs to be reported (if needed). For DL Tx beam prediction, UE is responsible for predicting and reporting the strongest beams and corresponding predicted RSRP since any reported beam may be selected by the network as the Tx beam to be used, so the corresponding RSRP for each reported beam needs to have a certain accuracy. Therefore, if we go with definition 2, in the case of the Top-K beam based on predicted L1-RSRP is not any of the Top-K beams based on ground-truth L1-RSRP, the corresponding predicted RSRP cannot be guaranteed.

Observation 5: For RSRP Absolute accuracy, if the index i can be any beam index in top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for those beams which belong to Top-K predicted beam but not any of the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Based on the above analysis, we propose definition 3 that the index i refers to any beam index in top-K beams based on predicted L1-RSRP. For this definition, it aims to ensure the accuracy of all predicted RSRPs of reported beams.

Proposal 5: The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i can be any beam index in top-K beams based on predicted L1-RSRP.

In this section, we share our initial thinking on metrics/KPIs for BM. Next, we provide further analysis on RSRP accuracy and beam prediction accuracy combined with the impact on the measurement error in clause 3.

- The impact of measurement error for inference performance

Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

In the RAN4#112bis meeting, it was agreed that RAN4 would start a specific study to assess the impact of measurement errors on prediction accuracy. In this section, we make some updates on the results based on the updated simulation assumption [2] and further provide some observations and proposals from the results. The updated simulation assumptions in RAN4#113 and AI model description are shown in the Appendix.

3.1 The baseband error impact on prediction accuracy for different metrics for beam management

Here we provide several cases to show performance on beam prediction accuracy under LLS-based measurement error modeling and truncated Gaussian distribution.

- Case 1: SNR is assigned as -3dB for LLS-based modeling

- Case 2: Truncated Gaussian distribution is modeled between -2.5dB to 2.5dB.

- Case 3: SNR is assigned as -6dB for LLS-based modeling

- Case 4: SNR is assigned as 0dB for LLS-based modeling

In Table 6, the metrics including Top-1 (%), Top-K/1 (%) and Top-1/K (%) are considered.

Table 6 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering measurement error

| Simulation results |  | Prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inference SNR |  | -3dB |  |  | -6dB |  | 0dB |  |
| Case |  | No error | Case 1 | Case 2 | No error | Case 3 | No error | Case 4 |
| Top 1 | +0dB | 68.72 | 49.00 | 66.35 | 69.10 | 66.20 | 72.19 | 71.17 |
|  | +1dB | 90.98 | 72.85 | 87.79 | 89.40 | 85.17 | 92.22 | 91.12 |
| Top 2/1 | +0dB | 87.95 | 77.35 | 86.59 | 84.09 | 82.93 | 88.67 | 87.91 |
|  | +1dB | 92.74 | 85.54 | 91.65 | 89.48 | 89.64 | 92.65 | 92.39 |
| Top 3/1 | +0dB | 91.38 | 85.54 | 90.36 | 87.99 | 87.08 | 92.31 | 91.21 |
|  | +1dB | 92.74 | 88.27 | 92.53 | 89.40 | 89.31 | 93.41 | 92.98 |
| Top 4/1 | +0dB | 93.38 | 89.00 | 92.77 | 90.22 | 89.98 | 94.00 | 93.66 |
|  | +1dB | 93.62 | 89.48 | 93.01 | 90.56 | 90.56 | 94.25 | 93.91 |
| Top 5/1 | +0dB | 96.41 | 92.29 | 95.50 | 93.45 | 93.37 | 96.70 | 96.70 |
|  | +1dB | 96.41 | 92.45 | 95.58 | 93.62 | 93.45 | 96.70 | 96.70 |
| Top 1/2 | +0dB | 87.47 | 74.78 | 86.35 | 85.34 | 83.51 | 88.59 | 87.40 |
|  | +1dB | 95.45 | 88.03 | 94.78 | 94.45 | 93.21 | 96.37 | 95.86 |
| Top 1/3 | +0dB | 94.01 | 85.22 | 92.93 | 91.22 | 89.48 | 93.66 | 92.73 |
|  | +1dB | 98.08 | 94.70 | 97.67 | 97.10 | 96.52 | 98.06 | 97.72 |
| Top 1/4 | +0dB | 96.97 | 90.12 | 95.74 | 94.95 | 93.79 | 96.96 | 95.69 |
|  | +1dB | 99.20 | 96.71 | 98.80 | 98.59 | 98.43 | 99.41 | 99.07 |
| Top 1/5 | +0dB | 98.00 | 92.85 | 97.35 | 97.02 | 96.44 | 98.22 | 97.46 |
|  | +1dB | 99.52 | 97.83 | 99.20 | 98.92 | 98.92 | 99.66 | 99.75 |

On the performance impact of measurement error, based on the simulation results, we have several observations:

- No matter for which metrics, measurement errors will degrade the beam prediction performance with AI/ML. Especially for the metric Top-K/1 and Top-1/K when K≤1,2, the degradation is non-negligible.

- With the decrease of SNR, the beam prediction accuracy will also decrease. However, the difference in prediction accuracy between Case 1 (SNR= -3dB) and Case 4 (SNR= 0dB) is not obvious.

- Compared with Truncated Gaussian distribution, the error derived from LLS-based modeling has a greater impact on the decline of beam prediction accuracy performance.

Observation 6: There is a non-negligible performance degradation in beam prediction accuracy when considering measurement error modeling for inference.

Observation 7: With the decrease of SNR, the beam prediction accuracy will also decrease. However, when the SNR reaches a certain level (e.g., 0dB), this performance decline will stabilize.

Observation 8: Compared with Truncated Gaussian distribution, the error derived from LLS-based modeling has a greater impact on the decline of beam prediction accuracy performance under similar SNR levels.

Based on the above observations, we believe the measurement error and corresponding methods of error modeling are the important factors affecting the prediction performance. Thus, it needs to be considered when RAN4 define the requirements for beam prediction accuracy

Proposal 6: RAN4 to define beam prediction accuracy requirements with consideration of the impact on measurement error.

Additionally, we also study the performance when [X]dB margin is considered in the beam prediction accuracy. Specifically, in our simulations, the Top-1 with X=1dB margin means that the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within X=1dB of the ideal L1-RSRP of the Top-1 genie-aided beam".

And for Top-K/1 with X=1dB margin, the criteria for judgment are:

- If the Top-1 genie-aided beam is one of the Top-K predicted beams, recorded as correct prediction

- Else if all the differences between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range, recorded as correct prediction, recorded as correct prediction

- Otherwise, recorded as wrong prediction

Besides, for Top-1/K with X=1dB margin, the criteria for judgment are

- If the Top-1 predicted beam is one of the Top-K genie-aided beams, recorded as correct prediction

- Else if the predicted RSRP of predicted Top-1 beam is within the range of {[ideal RSRP of ideal Top-K beam – 1dB], [ideal RSRP of ideal Top-1 beam + 1dB]}, recorded as correct prediction

- Otherwise, recorded as wrong prediction

From the results, we can observe that if 1dB margin is considered in statistics, it will bring a noticeable improvement in the rate of prediction accuracy, especially for the Top 1 case.

Observation 9: In addition to the rate of correct beam prediction, when additional 1dB RSRP margin is considered (e.g., For the success rate of the difference between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range), it will bring a noticeable improvement in the rate of prediction accuracy.

Furthermore, to figure out the impact of difference values of X on prediction accuracy and to facilitate RAN4 discussion on whether and how to determine the appropriate value of X margin, we provide some simulations on the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X= [1, 2.5, 5] dB range. Specifically, for the selection of the benchmark (i.e., the RSRP of Top-1 beam), we consider including

1)	Predicted RSRP of predicted Top-1 beam;

2)	Ideal RSRP of ideal Top-1 beam;

3)	Predicted RSRP of predicted Top-1 beam with consideration of 4dB measurement error;

4)	Ideal RSRP of ideal Top-1 beam with consideration of 4dB measurement error

Table 7 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=1dB range

| Simulation results |  | Probability (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 |  |  | Ideal top 1 |  |  |
|  |  | No error | Case 1 | Case 2 | No error | Case 1 | Case 2 |
| Top 1 | X=1dB | 100.00 | 100.00 | 100.00 | 88.91 | 52.85 | 78.88 |
| Top 2/1 |  | 98.40 | 95.90 | 98.47 | 82.68 | 43.45 | 67.47 |
| Top 3/1 |  | 59.46 | 52.21 | 60.32 | 50.52 | 25.38 | 41.04 |
| Top 4/1 |  | 4.71 | 5.70 | 4.66 | 5.27 | 4.90 | 7.31 |
| Top 5/1 |  | 1.76 | 2.65 | 1.85 | 1.92 | 1.12 | 1.29 |

Table 8 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=2.5dB range

| Simulation results |  | Probability (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 |  |  | Ideal top 1 |  |  |
|  |  | No error | Case 1 | Case 2 | No error | Case 1 | Case 2 |
| Top 1 | X=2.5dB | 100.00 | 100.00 | 100.00 | 97.45 | 91.57 | 96.95 |
| Top 2/1 |  | 99.68 | 99.60 | 99.68 | 96.09 | 88.76 | 95.34 |
| Top 3/1 |  | 97.37 | 94.30 | 97.19 | 91.86 | 79.12 | 90.04 |
| Top 4/1 |  | 60.18 | 57.35 | 58.80 | 53.87 | 48.35 | 53.49 |
| Top 5/1 |  | 13.73 | 13.09 | 13.25 | 11.41 | 11.16 | 11.33 |

Table 9 The probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the X=5dB range

| Simulation results |  | Probability (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Predicted top 1 |  |  | Ideal top 1 |  |  |
|  |  | No error | Case 1 | Case 2 | No error | Case 1 | Case 2 |
| Top 1 | X=5dB | 100.00 | 100.00 | 100.00 | 99.92 | 99.68 | 99.92 |
| Top 2/1 |  | 100.00 | 100.00 | 100.00 | 99.76 | 99.44 | 99.84 |
| Top 3/1 |  | 99.36 | 99.20 | 99.36 | 98.64 | 98.39 | 98.63 |
| Top 4/1 |  | 96.65 | 93.25 | 96.31 | 94.81 | 90.44 | 93.90 |
| Top 5/1 |  | 34.24 | 33.41 | 33.01 | 31.92 | 31.81 | 31.33 |

From the Table 7-9, we can observe that:

- As the X value increases, the probability of Top-K falling into this range increases. When X is 1, the probability of Top-2 falling into this range is only 70%-80%

- When [Predicted RSRP of predicted Top-1 beam] and [Ideal RSRP of ideal Top-1 beam] are used as benchmarks, the probability of falling into the X range is similar

- The measurement error has some impact on the probability when the value of X is small

Observation 10: The probability of predicted RSRP of predicted Top-K beam falling into the range of [the RSRP of Top-1 beam± XdB] has significant improvement with the increase of X value.

Observation 11: When using predicted RSRP of predicted Top-1 beam and Ideal RSRP of ideal Top-1 beam as the benchmark, the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the [X]dB range are similar.

Besides, based on the simulation results of following metrics, i.e., Top-1, Top-K/1 and Top-1/K, we have the following observations:

- With the increasing number of K (i.e., Top K/1 and Top 1/K), the prediction accuracy increases accordingly

- When measurement error is considered, there is significant performance degradation on Top-1 beam prediction (even lower than 50% accuracy in some cases).

- The performance on prediction accuracy defined as Top-K/1and Top-1/K are closed.

From RRM requirement perspective, RAN4 needs to determine one metric on beam prediction accuracy among these candidate metrics. Firstly, as per the observations above, the metric of Top-1 lacks certain robustness. And for the metrics of Top-K/1 and Top-1/K, the performance is approximate so that RAN4 can only choose one of them as the KPI. In our thinking, considering there is a case in UE that just reports the beam ID of Top-K without any ordering information. The Top-1 predicted beam may not be known, Top-K/1 is more appropriate as the KPI to figure out even without any rankings.

Proposal 7: RAN4 to use Top-K/1 as the metric of beam prediction accuracy for further performance evaluation.

For RSRP accuracy, according to our analysis in clause 2.4, here we will assume the following definition as RSRP accuracy and the following simulations for RSRP difference using it as KPI.

The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i can be any beam index in top-K beams based on predicted L1-RSRP

Table 10-12 show the simulation results of the RSRP accuracy under different SNR levels, including SNR=-6, -3, 0, respectively. Note that the simulation is under the case that the ground truth without measurement error is regarded as the benchmark. The measurement errors are only added in the model input.

Table 10 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the same beams (SNR=-3dB for inference)

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.46 |  |  |  |  |
| Top 5/1 | 1.46 | 1.71 | 1.91 | 2.36 | 2.70 |
| RSRP difference (dB) considering errors case 1 (only add in model input) |  |  |  |  |  |
| Top 1 | 3.86 |  |  |  |  |
| Top 5/1 | 3.86 | 3.58 | 3.76 | 3.99 | 4.86 |
| RSRP difference (dB) considering errors case 2 (only add in model input) |  |  |  |  |  |
| Top 1 | 2.08 |  |  |  |  |
| Top 5/1 | 2.08 | 2.03 | 2.47 | 2.60 | 3.22 |

Table 11 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the same beams (SNR=-6dB for inference)

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 2.79 |  |  |  |  |
| Top 5/1 | 2.79 | 2.58 | 2.80 | 3.26 | 3.40 |
| RSRP difference (dB) considering errors case 3 (only add in model input) |  |  |  |  |  |
| Top 1 | 3.14 |  |  |  |  |
| Top 5/1 | 3.14 | 3.09 | 3.03 | 3.10 | 4.18 |

Table 12 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beams and the ideal L1-RSRP of the same beams (SNR=0dB for inference)

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.40 |  |  |  |  |
| Top 5/1 | 1.40 | 1.56 | 1.71 | 2.20 | 2.47 |
| RSRP difference (dB) considering errors case 4 (only add in model input) |  |  |  |  |  |
| Top 1 | 1.60 |  |  |  |  |
| Top 5/1 | 1.60 | 1.82 | 2.06 | 2.42 | 2.78 |

3.2 RF error modeling

RF error is another important factor for measurement error modeling. Here we provide the evaluation results of prediction accuracy and RSRP difference when different RF errors modeling are considered, for which RF errors are modeled as 4dB offset, ±4dB uniform distribution, ±4dB Gaussian distribution, respectively.

Table 13 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering RF errors as 4dB offset, ±4dB uniform distribution, ±4dB Gaussian distribution (SNR=-3dB for inference)

| Simulation results | Prediction accuracy (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | No error | 4dB offset | ±4dB uniform distribution | ±4dB Gaussian distribution |
| Top 1 | 68.72 | 68.08 | 30.41 | 36.63 |
| Top 2/1 | 87.95 | 87.71 | 61.21 | 66.64 |
| Top 3/1 | 91.38 | 91.06 | 74.30 | 80.53 |
| Top 4/1 | 93.38 | 93.46 | 80.29 | 83.96 |
| Top 5/1 | 96.41 | 96.25 | 85.40 | 88.91 |
| Top 1/2 | 87.47 | 87.55 | 54.27 | 61.77 |
| Top 1/3 | 94.01 | 93.93 | 68.56 | 76.14 |
| Top 1/4 | 96.97 | 96.97 | 76.22 | 81.80 |
| Top 1/5 | 98.00 | 98.00 | 81.17 | 86.43 |

Table 14 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the same beam (SNR=-3dB for inference)

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 1.46 |  |  |  |  |
| Top 5/1 | 1.46 | 1.71 | 1.91 | 2.36 | 2.70 |
| RSRP difference (dB) considering 4dB offset errors (only add in model input) |  |  |  |  |  |
| Top 1 | 4.51 |  |  |  |  |
| Top 5/1 | 4.51 | 4.68 | 4.85 | 5.00 | 5.05 |
| RSRP difference (dB) considering ±4dB uniform distribution errors (only add in model input) |  |  |  |  |  |
| Top 1 | 7.89 |  |  |  |  |
| Top 5/1 | 7.89 | 7.47 | 7.12 | 6.55 | 6.35 |
| RSRP difference (dB) considering ±4dB gaussian distribution errors (only add in model input) |  |  |  |  |  |
| Top 1 | 5.88 |  |  |  |  |
| Top 5/1 | 5.88 | 5.77 | 4.96 | 5.35 | 5.72 |

From Table 13, it can be observed that the prediction accuracy is not impacted by the 4dB RF offset. The possible reason is that the trend of all the datasets remains unchanged when a 4dB offset is added to all the RSRP elements. Furthermore, based on the simulation result based on the error modeled by uniform distribution and Gaussian distribution, the uniform distribution error brings more degradation to prediction accuracy compared with the 4dB offset value as the errors.

From Table 14, when different RF error modeling is considered, there are some noticeable performance differences in the RSRP difference. Among them, the error modeled by uniform distribution has the greatest impact on the accuracy of RSRP prediction.

Based on the initial evaluation we provided above, there is some impact on beam prediction and predicted RSRP value due to RF error. From this perspective, we believe the modeling method for RF error also needs to be involved in the study.

Observation 12: There is some impact on beam prediction and predicted RSRP value due to RF error.

Proposal 8: For the study on the impact of measurement error, the method of RF error modeling needs to be involved in RAN4 discussion, e.g.,

- An offset value in the original dataset;

- Truncated Uniform distribution;

- Truncated Gaussian distribution.

- Summary

Proposal 1: On the definition of prediction delay requirements for BM, RAN4 to consider:

- For Measurement for prediction, the current measurement period for L1-RSRP measurement can be the baseline and further discuss the details by considering the parameters, which at least include:

  - The number of samples for set B measurements (for spatial domain prediction); The number of the latest measurement instances and the time interval between two adjacent time instances (for time domain prediction)

  - beam sweeping factor

  - sharing factor N

- For inference delay: the reference latency and possible processing time

Proposal 2: For the UE Rx beam knowledge,

- at least for the case that the Top-K beams to be reported are in set B and set B is a subset of set A, if beam sweeping has been done during measurements on set B, UE Rx beam information is known by UE and no further beam sweeping is needed.

- for other cases, e.g., set B is not a subset of set A, RAN4 to further discuss how to obtain Rx beam information, e.g., introducing UE capability or making the reduced beam sweeping factor assumption

Proposal 3: For performance monitoring for BM, RAN4 to first determine the testing goal. The following alternatives can be considered:

- Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE;

- Option 2: The testing goal is to verify whether UE can perform monitoring and report the performance metrics within a certain latency upon performance monitoring is required;

- Option 3: The combination of Option 1 and Option 2.

Observation 1: For Top-K/N (%): the percentage of "the Top-N genie-aided beam is ONE of the Top-K predicted beams, the prediction becomes easy (most of the prediction accuracy is above 95%) even measurement error is considered. This may make the target too relaxed. Even if this KPI is met, it does not fully represent that the true quality of these beams is good

Observation 2: The KPI of Top K/N mitigates the effects that the RSRPs between Top 1 and Top 2 genie-aid beams are too close (within 0.5dB), which may cause the actual Top-1 beam may be indistinct

Observation 3: Compared with the KPI of “the percentage of the Top-N genie-aided beam is ONE of the Top-K predicted beams”, the KPI of “the percentage of the Top-N genie-aided beam is TWO of the Top-K predicted beams” may be more reasonable for ensuring the quality of the reported beams

Proposal 4: RAN4 to follow the process as above for the alignment of KPIs and definition performance requirements on BM prediction

- <Step 1> Discuss the definition of KPIs with the consideration of measurement error

  - <Step 1-1> Select one or more candidate KPIs for simulation

  - <Step 1-2> With the aligned simulation assumption, each company brings simulation results of candidate KPIs. Specifically, the measurement error modeling of simulation assumption may include

    - Option 1: only BB measurement error is considered

    - Option 2: only RF measurement error is considered

    - Option 3: both BB and RF measurement error are considered

  - <Step 1-3> Check on KPIs performance alignment from the simulation results from contributing companies

    - repeat simulations for 2-3 meetings and check if good alignment can be achieved

      - if good alignment of KPIs is achieved, move to step 3

      - if not, move to step 2

  - <Step 2> Discuss the definition of KPIs without the consideration of measurement error

    - <Step 2-1> With the aligned simulation assumption, each company brings simulation results of candidate KPIs

    - <Step 2-2> Discuss whether some margin be added to the aligned results of KPIs

    - <Step 2-3> Move to next step after alignment

  - <Step 3> RAN4 to make the final decision for KPIs definition and discuss the details of RRM requirements

Observation 4: There is a certain probability that the Top-K predicted beam is not any of the Top-K beams based on ground-truth RSRP.

Observation 5: For RSRP Absolute accuracy, if the index i can be any beam index in top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for those beams which belong to Top-K predicted beam but not any of the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Proposal 5: The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i can be any beam index in top-K beams based on predicted L1-RSRP.

Observation 6: There is a non-negligible performance degradation in beam prediction accuracy when considering measurement error modeling for inference.

Observation 7: With the decrease of SNR, the beam prediction accuracy will also decrease. However, when the SNR reaches a certain level (e.g., 0dB), this performance decline will stabilize.

Observation 8: Compared with Truncated Gaussian distribution, the error derived from LLS-based modeling has a greater impact on the decline of beam prediction accuracy performance under similar SNR levels.

Proposal 6: RAN4 to define beam prediction accuracy requirements with consideration of the impact on measurement error.

Observation 9: In addition to the rate of correct beam prediction, when additional 1dB RSRP margin is considered (e.g., For the success rate of the difference between the ideal RSRP of ideal Top-1 beam and the predicted RSRP of predicted Top-K beam are within the X=1dB range), it will bring a noticeable improvement in the rate of prediction accuracy.

Observation 10: The probability of predicted RSRP of predicted Top-K beam falling into the range of [the RSRP of Top-1 beam± XdB] has significant improvement with the increase of X value.

Observation 11: When using predicted RSRP of predicted Top-1 beam and Ideal RSRP of ideal Top-1 beam as the benchmark, the probability that the difference between the RSRP of Top-1 beam and the predicted RSRP of Top-K beam is within the [X]dB range are similar.

Proposal 7: RAN4 to use Top-K/1 as the metric of beam prediction accuracy for further performance evaluation.

Observation 12: There is some impact on beam prediction and predicted RSRP value due to RF error.

Proposal 8: For the study on the impact of measurement error, the method of RF error modeling needs to be involved in RAN4 discussion, e.g.,

- An offset value in the original dataset;

- Truncated Uniform distribution;

- Truncated Gaussian distribution.

- References

- R4-2414323, WF on AI/ML, Qualcomm

- R4-2420480, Updated simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

- Appendix

6.1 Simulation assumptions

Link level simulation (LLS) assumptions for evaluation of measurement error on AI/ML based BM case-1 performance are defined in Table 1-5.

Table 1 Baseline Link Level Simulation assumptions for AI/ML in beam management evaluations

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Channel model | NLOS channel: option 1: CDL-C , option 2: TDL-C, DS = 100ns Other channel models are not precluded. |
| BS antenna configurations | Option 1: One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. Number of Tx beams is 32 is baseline Other number of Tx beams is not precluded Option 2: 1 Tx antenna Number of TX beams: N/A Companies are encouraged to provide the codebook used in SLS |
| BS antenna element radiation pattern | Option1: Table 2 Option 2: N/A |
| BS antenna height and antenna array down-tilt angle, if needed (Option 1 above) | Option1: 25m, 110° Option 2: N/A |
| Beamforming characteristic of the BS pattern | Option 1a [corresponding to BS antenna configurations option 1]: 32 beams with grid of 4 elevation angles from [-25 to 25] and 8 azimuth angles from [-60 to 60] Option 1b [corresponding to BS antenna configurations option 1]: The codebook construction can be clarified by companies. Option 2: N/A |
| UE antenna configurations | Option 1: Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional\ Number of Rx beams is up to UE Option 2: 2 Rx |
| UE antenna element radiation pattern | Option 1: Table 3 Option 2: N/A |
| UE moving speed | 3km/h |
| Reference signal | SSB or CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | 1 as baseline, other number of samples are not precluded |
| Note 1: TXRU weights mapping for BS and UE is up to company. |  |

Table 2 BS antenna radiation pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 3 UE antenna radiation pattern model

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in dim (dB) |  |
| Antenna element radiation pattern in dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
| Note: are in local coordinate system. |  |

Table 4 Cell-specific parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 |
| SSB periodicity | ms | 20 |
| SSB bandwidth | RB | 20 |
| SSB SCS | kHz | 120 |
| RB Utilization | % | 100 |
| Data Modulation | - | QPSK |
| CP Length | - | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| Relative Delay of 1st Path (synchronous) | µs | 0 |
| SNR | dB | -3 Note: Simulation with other SNR levels are not precluded, e.g., for determining the SNR for ideal measurement. |
| Note: When analog beamforming is used in LLS, SNR value of above row represents the post beamforming SNR of the strongest Tx-Rx beam pair. For other Tx-Rx beam pairs, there are two options. Option 1: Apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS. When analog beamforming is not used in LLS, Apply error distribution based on the SNR of above row to the strongest Tx-Rx beam pair’s measurements in the SLS Option 1: Apply error distribution from the SNR of above row to all Tx-Rx beam pairs’ measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pairs’ SNR values and apply to the corresponding Tx-Rx beam pair measurements in the SLS. SNR of the strongest Tx-Rx beam pair can be fixed or not fixed. |  |  |

Table 5 CSI-RS configuration parameters

| Parameter | Value |
| --- | --- |
| CSI-RS bandwidth | 48 PRBs; |
| CSI-RS SCS | 120 kHz |
| Periodicity | 5 ms (optional); 40 ms |
| CSI-RS configuration: <X,D,N,CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot | <32,1,4,FD-CDM2> |

Simulation assumptions for RF error

The impact of RF errors should also be considered. ±4.5dB RF error is considered. Following options can be used to generate RF errors.

- Option 1: Independent for each TX beam

- Option 2: Same for each Tx beam

- Option 3: Independent for each module/panel

- Option 4: Same for each module/panel

- Other options are not precluded

  - Companies are requested to provide details of other options.

Following options about distributions can be used:

- Option 1: Uniform distribution

- Option 2: Gaussian distribution

- Option 3: A fixed value

- Other options are not precluded

  - Companies are requested to provide details of other options.

Companies are encouraged to provide simulation results with and without considering RF errors.

6.2 AI/ML model

For better alignment of the performance between companies, it would be better to perform simulations based on a reference model. The model description in Fig. 1 is an example of reference model for BM-case 1 performance evaluation, while it is not the intention to preclude other reference models.

Fig. 1 Reference model description