3GPP TSG-RAN WG4 Meeting #114		R4-2500849

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.25.2.2

Source: 	CMCC

Title: 			  Discussion on RRM requirement of OD-SIB1 and signal adaptation for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed on demand SIB1 and SSB signal adaptation related issues, and some conclusions or recommendations have been approved in topic summary [1, 2] and the WF [3]. In this paper, we further discuss related issues, and identify RAN4 impacts according to the progress of other working group.

- OD-SIB1 requirement

Issue 2-1-2: Te requirement

- Proposals

  - Option 1: vivo

    - For R19 OD-SIB1 related procedure, clarify DL timing reference of UL-WUS in TS 38.133. RAN4 may further discuss whether additional enhancement/relaxation on Te requirements is needed.

Agreement from RAN1#119

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Based on the current progress of RAN1, Case2 will be specified in R19.

Case 2:

- UE transmits UL-WUS to NES Cell

- UE obtains the UL-WUS configuration from Cell A

- UE receives on-demand SIB1 from NES Cell

- Since UE need to transmit the UL WUS to NES Cell, the UL timing requirement should be specified in RAN4. Different from legacy PRACH transmission, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc,

- reference cell is the NES cell

- NTA for UL-WUS is defined as 0

- is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

Proposal 1: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that:

- reference cell is the NES cell

- NTA for UL-WUS is defined as 0

- is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

- SSB Adaptation

Issue 1-2-1: L1 requirements at transition

Agreement:

  - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

    - FFS the applicable L1 requirements.

For active SCell in RRC CONNENCTED mode, the transition period will be applied to BFD, L1-RSRP and L1-SINR measurement of serving cell.

Proposal 2: The L1 transition requirements will be applied to BFD, L1-RSRP and L1-SINR measurement of serving cell.

Issue 1-4-1: SCell activation requirement for SSB adaptation

- Proposals

  - Option 1: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching (CATT, Ericsson, Vivo, Samsung, OPPO)

    - Option 1: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. (CATT)

    - Option 1a: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. (Ericsson)

    - Option 1b: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. (Vivo)

    - Option 1c: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception. (Samsung)

    - Option 1d: RAN4 not to define L3 measurement based on SMTC periodicity before and during SCell activation if OD-SSB is triggered before the complete of SCell activation. (Oppo)

  - Option 2a: (CTC)

    - After SSB adaptation, for L1 measurement during SCell activation, UE shall perform SCell activation based on new SSB configurations, e.g. new SSB periodicity (CTC)

    - After SSB adaptation, for L3 measurement during SCell activation, wait for the conclusions on SMTC impact from other WGs. (CTC)

  - Option 2b: (Huawei)

    - UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation. Deprioritize the requirements when SSB adaptation happens during SCell activation.

    - Detailed triggering on SSB adaptation depends on further RAN1 discussion.

  - Option 2c: RAN4 to study the SCell activation and deactivation delay impact. (Samsung)

    - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

    - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

- Recommended WF:

  - No requirements when SSB adaptation happens during SCell activation.

  - FFS the requirements impact when SSB adaptation happen before or at the same time as SCell activation considering.

Considering OD-SSB methodology has been introduced, we don’t see the additional gain to further perform SSB adaptation during SCell activation.

Proposal 3: No requirement for the case that SSB adaptation during SCell activation.

If the SSB adaptation happens before the SCell activation, due to the deactivated SCell measurement follow the measCycleSCell, no need to define the transition requirement.

Proposal 4: Legacy requirement can be reused for the case that SSB adaptation before SCell activation.

If the SSB adaptation happens at the same time as SCell activation, UE shall use the adapted SSB for SCell activation. Meanwhile, regarding whether the SMTC will be adapted or not, RAN4 should wait for LS reply.

Proposal 5: UE shall use the adapted SSB for SCell activation for the case of SSB adaptation happens at the same time as SCell activation.

Issue 1-4-2: TCI state switching requirements for SSB adaptation

- Proposals

  - Option 1: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. (CATT)

    - Option 1a: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. (Ericsson)

    - Option 1b: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. (Vivo)

  - Option 2: RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for active SCells for BFD and CBD, and to TCI state switching delay. (Nokia)

- Recommended WF:

  - No requirements when SSB adaptation happens during TCI state switching.

If the SSB periodicity adaptation happens during TCI state switching, the L1-RSRP measurement will be impacted. However, for L1-RSRP measurement requirement at transition, the L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

Therefore, during switch procedure, if network shorten the SSB periodicity for fast TCI state switching, the delay requirement will not be shortened. If network enlarge the SSB periodicity for power saving, the TCI state switch delay will be increased, the performance will be impacted. As a consequence, we fail to see the gain to perform SSB adaptation during TCI state switching.

Proposal 6: No requirement for the case that SSB adaptation during TCI state switching.

L3 measurement related

Due to the LS on whether there is impact on SMTC hasn’t been replied by RAN2, we postpone the discussion about L3 measurements requirements and the delay requirements rely on SMTC.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that:

- reference cell is the NES cell

- NTA for UL-WUS is defined as 0

- is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

Proposal 2: The L1 transition requirements will be applied to BFD, L1-RSRP and L1-SINR measurement of serving cell.

Proposal 3: No requirement for the case that SSB adaptation during SCell activation.

Proposal 4: Legacy requirement can be reused for the case that SSB adaptation before SCell activation.

Proposal 5: UE shall use the adapted SSB for SCell activation for the case of SSB adaptation happens at the same time as SCell activation.

Proposal 6: No requirement for the case that SSB adaptation during TCI state switching.

- Reference

- R4-2418282, Topic summary for [113][224] Netw_Energy_NR_enh_Part1, Moderator (Ericsson)

- R4-2418283, Topic summary for [113][225] Netw_Energy_NR_enh_Part2, Moderator (Huawei)

- R4-2420242, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei