3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500861

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.26.2.1

Source: 	CMCC

Title: 	Discussion on LP-WUS UE system parameters requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#110-bis, the LP-WUS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about system parameters.

- Discussion

### 2.1 Channel raster for LP-WUR

Issue 1-2-1: Channel raster for LP-WUR Agreements: - No new channel raster is introduced dedicated for LP-WUS/WUR. - FFS on whether there is need of the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost

We have reached the agreement that assume PRB grid alignment between LP-WUS and NR, and LP-WUS RBs can be flexibly allocated within the wider NR carrier. If 100kHz channel raster is used for some low bands, it means that the LP-WUS RB with 30kHz/15kHz SCS couldn’t be allocated within the NR carrier.

Observation 1: LP-WUS RB with 30kHz/15kHz SCS couldn’t be flexibly allocated within the NR carrier with 100kHz channel raster, and this will conflict with the conclusions we have already reached.

Some companies suggest to introduce new channel raster like 10kHz for LP-WUS. In our opinion, there is no need to introduce new channel, and the primary reason for the above issue is that the 100kHz channel raster will restrict the placement of RB. So we suggest the legacy channel raster could be reused for LP-WUS without 100kHz channel raster.

Proposal 1: The legacy channel raster could be reused for LP-WUS without 100kHz channel raster.

### 2.2 LP-WUR types

Based on the previous conclusions of RAN1, LP-WUS types for both OOK-based and OFDM-based are needed to be defined. The two types can be defined as OOK-based and OFDM-based, respectively. The main difference between the two types of LP-WUS is power consumption. Therefore, it is recommended to use NF as the definition to distinguish between the two types of LP-WUS.

Proposal 2: Use the following table as baseline for the definitions of LP-WUR Types. The values of X and Y need to wait for the conclusions of the REFSENSE requirements.

Table 1. The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| OOK-based | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure [X] dB higher than the MR. |
| OFDM-based | The LP-WUR is designed for moderate power consumption, assuming a noise figure [Y] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

# 3. Conclusions

Observation 1: LP-WUS RB with 30kHz/15kHz SCS couldn’t be flexibly allocated within the NR carrier with 100kHz channel raster, and this will conflict with the conclusions we have already reached.

Proposal 1: The legacy channel raster could be reused for LP-WUS without 100kHz channel raster.

Proposal 2: Use the following table as baseline for the definitions of LP-WUR Types. The values of X and Y need to wait for the conclusions of the REFSENSE requirements.

Table 1. The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| OOK-based | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure [X] dB higher than the MR. |
| OFDM-based | The LP-WUR is designed for moderate power consumption, assuming a noise figure [Y] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

# 4. Reference

- R4-2420528, WF on UE RF requirements for LP-WUS