3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500867

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.2.2

Source: 	CMCC

Title: 	Discussion on existing Tx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements.[1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACLR and OBUE

Following capture the agreements from TR 38.858 for ACLR requirements:

For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

Since whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS is still FFS. that no conclusions were reached in the previous meeting, we have listed the conclusions of the RAN4#112bis meeting for reference.:

WF: - FFS on the followings: - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly? - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence. - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirement. - How to define the additional co-existence requirements: - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level. - [1dB DESENS in the victim SBFD receiver] - [typical CL values based on typical SBFD-capable gNB implementation] - [frequency offset for the additional OBUE requirement is declared] - Option 2: by improving ACLR1 and ACLR2 requirements - Option 3: others

For the SBFD network, the simulation of adjacent-channel coexistence primarily concentrates on grid shift values of no less than 10%, deliberately avoiding the 0% grid shift scenario. However, the 0% grid shift represents the most severe case. Given the difficulty in defining the co-location ACLR requirement, it is advisable to initially focus on discussing scenarios where co-location is not a factor.

For ACLR and OBUE requirement, the legacy requirement should be first checked, such as the BS spurious emissions requirements. Beside, when defining relevant requirements for macro BS, the spectrum bandwidth should be taken into account. For example, a bandwidth of 100MHz can be selected for analysis to assess the feasibility of reusing existing requirements.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The legacy BS requirement could be considered as the starting point.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The legacy BS requirement could be considered as the starting point.

# 4. Reference

[1] R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF