3GPP TSG-RAN WG4 Meeting #114	R4-2500883

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Ku-band NTN

Agenda item:	7.11.6

Document for:	Discussion

Introduction

Based on WID [1], RRM discussion for Ku-band NTN begins, and the objectives are below.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

Most issues are RF discussion, and Ka-band NTN requirements would be reused as much as possible. One clear issue for RRM is timing requirements for mobile VSAT served by NGSO. In this contribution, we provide our views on RRM requirements for Ku-band NTN.

Discussion

Mobile VSAT served by NGSO

Based on the revised WID in the last RAN plenary, new type of NTN UE for Ku-band, mobile VSAT with NGSO, was introduced. In RF discussion, it agreed to define two sets of bands for Ku-band; one for FR1 numerology as 30kHz SCS and the other for FR2 numerology as 120kHz SCS. In Ku-band NTN, mobile VSAT type 4 and 5 are considered, and RF requirements for mobile VSAT with NGSO would be defined based on VSAT type 5.

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronic steering antenna. |

Therefore, the RRM requirements should be also defined for two sets of bands for VSAT type 4 and 5 considering NGSO. Even if both FR1 and FR2 numerologies are considered, Ka-band NTN RRM requirements could be reused as baseline. But timing error requirements for FR1 and FR2 numerology could be different.

- Proposal 1: FR2 (Ka-band) NTN RRM requirements could be reused for Ku-band NTN type 4 and 5 (not to introduce separate RRM requirements for Ku-band NTN), except for timing error requirements.

The timing error requirements for L/S-band (FR1) NTN and Ka-band (FR2) NTN are defined in the specification as follow.

FR1 NTN: Te_NTN Timing Error Limit for FR1 NTN

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

FR2 NTN: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

FR2 NTN: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

For FR1 numerology for mobile VSAT with NGSO, existing timing error requirements can be reused since existing FR1 NTN is considered for mobile UE and NGSO/GSO satellite.

Mobile VSAT with NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| TBD | 15 | 30 | 24*64*Tc |
|  | 30 |  | 22*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

In the last RAN4 meeting, the CR [2] on timing error requirements for mobile VSAT with GSO on 120kHz SCS was endorsed, and the timing error was revised from 10*64*Tc to 7.5*64*Tc with some conditions considering that the margin for one-way positioning error for the UE and satellite can be up to 31.61m.

FR2 NTN: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Note 2: The value of Te_NTN for 120 kHz UL SCS assumes LOS, and GNSS signal power level above [TBD] dBm [and max acceleration of [TBD] m/s2] FFS other side conditions if needed. |  |  |  |

Even considering NGSO for mobile VSAT, the timing error requirements should not be larger than 7.5*64*Tc to avoid system performance loss. Therefore, the timing requirements for mobile VSAT with NGSO could be the same as that for Ka-band mobile VSAT with GSO. RAN4 needs further discussion on whether to need additional conditions of timing requirements for mobile VSAT with NGSO.

Mobile VSAT with NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| TBD | 120 | 120 | 7.5*64*Tc |
|  | 240 |  | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Note 2: The value of Te_NTN for 120 kHz UL SCS assumes LOS, and GNSS signal power level above [TBD] dBm [and max acceleration of [TBD] m/s2] FFS other side conditions if needed. |  |  |  |

- Proposal 2: The existing FR1 NTN and revised FR2 NTN timing error requirements can be reused for mobile VSAT with NGSO as below, and FFS whether to need additional conditions for 120kHz SCS in NGSO.

Mobile VSAT with NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| TBD | 15 | 30 | 24*64*Tc |
|  | 30 |  | 22*64*Tc |
| TBD | 120 | 120 | 7.5*64*Tc |
|  | 240 |  | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Note 2: The value of Te_NTN for 120 kHz UL SCS assumes LOS, and GNSS signal power level above [TBD] dBm [and max acceleration of [TBD] m/s2] FFS other side conditions if needed. |  |  |  |

Conclusion

In this contribution, we provide our views on RRM requirements for Ku-band NTN, and we propose

- Proposal 1: FR2 (Ka-band) NTN RRM requirements could be reused for Ku-band NTN type 4 and 5 (not to introduce separate RRM requirements for Ku-band NTN), except for timing error requirements.

- Proposal 2: The existing FR1 NTN and revised FR2 NTN timing error requirements can be reused for mobile VSAT with NGSO as below, and FFS whether to need additional conditions for 120kHz SCS in NGSO.

Mobile VSAT with NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| TBD | 15 | 30 | 24*64*Tc |
|  | 30 |  | 22*64*Tc |
| TBD | 120 | 120 | 7.5*64*Tc |
|  | 240 |  | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Note 2: The value of Te_NTN for 120 kHz UL SCS assumes LOS, and GNSS signal power level above [TBD] dBm [and max acceleration of [TBD] m/s2] FFS other side conditions if needed. |  |  |  |

Reference

[1] RP-243314, “Revised WID: Introduction of Ku bands for NR NTN,” Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

[2] R4-2420236, “Corrections to UE transmit timing for Satellite Access,” Eutelsat Group, Thales, European Space Agency, Sharp, CHTTL, Airbus