3GPP TSG-RAN WG4 Meeting # 114	                                        R4-2500886

Athens, GR, Feb. 17 – Feb. 21, 2025

Agenda Item: 7.27.2.2

Source: MediaTek Inc.

Title: Discussion on CSI-RS L1 measurement for LTM

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of CSI-RS L1 measurements.

# Discussion

## 2.1 known cell definition in CSI-RS based L1 measurement

In RAN4#112bis, RAN4 agreed that RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells. In the past two meetings, RAN4 discussed known cell definition as shown below. In the next, we would like to discuss the remaining parts.

RAN4#112bis #### Issue 2-1: CSI-RS based L1 measurement for known/unknown cell - Agreement: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells #### Issue 2-2: known cell definition in CSI-RS based L1 measurement For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition RAN4#113 #### Issue 3-1-1: Whether to consider SSB-based L3 measurement only? < Agreement > - Consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition.

Regarding known cell definition, the first bullet is to guarantee that UE has detected the cell and acquired the rough timing & frequency of the cell.

The second bullet is to make sure the associated SSB of the CSI-RS is detectable. As proposed in Proposal 3 in 2.2, CSI-RS for L1 measurement shall be Type-D QCL to SSB for L1 measurement in FR2-1.

Proposal 1: Keep L1 measurement for FR2-1 in the second condition of known cell definition in CSI-RS based L1-RSRP measurement.

The third bullet is to guarantee the channel quality of CSI-RS is not too bad. Regarding the third bullet, we prefer to use a side condition. CSI-RS is not used for detection. It is not accurate to use the wording “Detectable”.

Proposal 2: The third condition of known cell definition in CSI-RS based L1-RSRP measurement is: SNR of the CSI-RS from the target cell configured for L1 measurement is above -3dB.

## 2.2 Measurement procedure of CSI-RS based L1 measurement

In the past meetings, RAN4 had extensive discussions on the measurement procedure of CSI-RS based L1 measurement. Last meeting, the following agreement was reached. The issue to discuss is whether step 2 can be skipped in FR2-1.

#### Issue 3-2: Measurement procedure <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

Based on last meeting’s discussion, it seems there are three options on table.

- Option 1: CSI-RS L1-RSRP measurement after SSB L1-RSRP measurement, and CSI-RS resources with Repetition off

- use the fine beam refined at SSB L1-RSRP measurement

- Total measurement delay: SSB L1-RSRP measurement delay + 1*CSI-RS period

- Option 2: CSI-RS L1-RSRP measurement after SSB L3-RSRP measurement, and CSI-RS resources with Repetition off

- Rx beam sweeping over N CSI-RS periods, N is the beam sweeping factor

- Total measurement delay: N*CSI-RS period

- Option 3: CSI-RS L1-RSRP measurement after SSB L3-RSRP measurement, and CSI-RS resources with Repetition on

- Rx beam sweeping within each resource set

- Total measurement delay: CSI-RS period

Option 1 is aligned with CSI-RS L1-RSRP measurement procedure in serving cell. As UE Rx beam is refined through SSB L1-RSRP measurement, UE can finish CSI-RS L1-RSRP measurement in one CSI-RS period.

Option 2 offloads L1-RSRP measurement on neighbor cell(s) to CSI-RS. UE is supposed to refine Rx beam using several CSI-RS periods. To achieve similar measurement period, CSI-RS periodicity should be comparable to SSB periodicity.

As far as we know, CSI-RS L1-RSRP measurement with repetition on is not in use in the real field. So we propose to exclude Option 3 at first.

We admit that Option 2 can bring some benefits compared with Option 1 due to a better refined Rx beam corresponding to the CSI-RS but not the associated SSB can be obtained. But using option 2, neighbor cell will be always biased over serving cell from the point of reported L1-RSRP measurement results. So we still prefer Option 1.

Proposal 3: For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, only consider the following procedure:

- Step 1: SSB based L3 measurements

- Step 2: SSB-based L1 measurements and reports on the same candidate cell

- Step 3: CSI-RS based L1 measurements on the same candidate cell

  - Note: The CSI-RS in Step 3 should be Type-D QCL with the SSB in Step 2 and configured with repetition OFF.

## 2.3 Whether to define requirements for RTD>CP case

Single FFT is assumed for multiple cell measurements for CSI-RS based L3 measurement. For SSB based L1 measurement, RTD>CP case is supported with UE capability. Last meeting, RAN4 agreed that “For CSI-RS based L1-RSRP measurement on neighbor cell, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs”. If to support RTD>CP case for CSI-RS based L1 measurement, another FFT is needed to cover as wide as 48PRBs, while SSB only occupies 20PRBs. The cost will be much higher to support RTD>CP for CSI-RS compared to SSB. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement. We propose not to support RTD>CP case for CSI-RS based L1-RSRP measurement.

Proposal 4: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case.

It was observed that there were proposals to report the value of RTD between neighbor cell and serving cell or whether RTD is within CP. As the intention of L1-RSRP report on neighbor cell is for next step TCI state activation, early TA acquisition or cell switch, we think the way to get RTD information is through cell search and L3 measurement.

According to the approved Tdoc R4-1708278 in RAN4#84, the timing accuracy after L3 measurement is within half CP which is not enough for the purpose of indicating RTD within CP or not.

R4-1708278

Observation 1: Timing accuracy after L3 measurement is far from enough for the purpose of indicating RTD within CP or not.

## 2.4 Definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

In the past meetings, RAN4 discussed whether and how to define intra-frequency for CSI-RS L1 measurement and reached the following agreement.

<Agreement> - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and - It is applied for SCS = 60KHz - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

As the definition is only for the convenience of capturing the requirements, we are fine with definition and remove the brackets.

Proposal 5: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).

## 2.5 Whether to define requirements for CSI-RS outside active BWP

To perform CSI-RS based L1-RSRP measurement outside active BWP, gap shall be configured. Typical MGL are 6ms, 3ms, 4ms, 5.5ms and 1.5ms. To minimize the RS overhead, one CSI-RS resource is used for both serving and neighboring cell measurements, e.g., CSI-RS resources for L1-RSRP measurement on neighbor cell for UE#1 are in fact the resources for L1 measurement of serving cell for UE#2. For serving cell, there is no specific restriction in time domain on configurations of CSI-RS resources for L1-RSRP measurement. It makes difficult to ensure all the CSI-RS of the same frequency layer are within a single gap window. To solve this issue, for CSI-RS based L3 measurement, all CSI-RS resources in the same MO are confined in the same 5ms window. If to support CSI-RS based L1-RSRP measurement outside active BWP, similar restriction is also necessary. If so, it may pose a strong limitation on CSI-RS configuration. Generally, MG is configured for SSB based L3 measurement. SSB of each cell are overlapped or partially overlapped with MG. Due to measurement restriction in FR2, UE is not supposed to perform SSB based L3 measurement and CSI-RS based L1 measurement simultaneously. So the reasonable configuration is that SSB symbols and CSI-RS symbols for L1 measurement in FR2 are not overlapping. As shown below, most of the slots are occupied by SSB and only the white ones are not occupied by SSB. And SSB takes most of the symbols in the slots that contain SSB. If not to use concurrent gap, NW shall squeeze all the CSI-RS resources to the remaining symbols. It seems difficult to confine SSB and CSI-RS of the same frequency layer in the same 5ms window. In other words, it is difficult to use one gap to cover both SSB based L3 or L1 measurement and CSI-RS based L1 measurement, so concurrent gap is needed.

Also, to support CSI-RS based L1-RSRP measurement outside active BWP, it is necessary to have a definition for frequency layer, as different frequency layers should be measured TDM-ly. If there is no such a definition, it will be difficult to discuss how to share the gap occasions by different CSI-RS resources.

Observation 2: If to support CSI-RS based L1-RSRP measurement outside active BWP, all CSI-RS in the same frequency layer shall be confined in the same 5ms window.

Observation 3: If to support CSI-RS based L1-RSRP measurement outside active BWP, concurrent gap shall be used.

Observation 4: If to support CSI-RS based L1-RSRP measurement outside active BWP, the definition of frequency layer should be discussed at first.

In summary, to support CSI-RS based L1-RSRP measurement outside active BWP, there would be much more restriction on CSI-RS configurations, and an optional feature concurrent gap should be supported at first. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement for inter-f. We propose to deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Proposal 6: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

## 2.6 Whether and how to define “frequency layer”?

If RAN4 concludes not to define requirements for CSI-RS based L1-RSRP measurement outside active BWP and also RTD>CP case, maybe it is not necessary to define “frequency layer”.

In legacy, UE only performs L1-RSRP measurement on CSI-RS resources within active BWP in serving cell and RAN4 didn’t define any frequency layer concept for CSI-RS based L1 measurement. If only to consider CSI-RS based L1 measurement within active BWP and within CP, it will be similar as CSI-RS based L1 measurement in serving cell. It is not necessary to define frequency layer.

If RAN4 will define requirements for RTD>CP case or outside active BWP case, frequency layer shall be first defined. It is straight forward for CSI-RS based L1-RSRP measurement outside active BWP. In the next, we would like to discuss why frequency layer shall be first defined if to support RTD>CP case. To support RTD>CP case, another FFT supporting 48 PRBs is needed. As shown in Fig. 1, CSI-RS #1,2,3 are three resources from the same neighbor cell. It is obvious that CSI-RS #1 and #2 should be measured TDM-ly as they can not be covered by a single FFT window. If not defining the concept frequency layer, there will be ambiguity on how to measure CSI-RS#3. Or another much wider FFT is needed.

Fig. 1

Proposal 7: If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”.

# Summary

In this paper, we provide some views on the R19 mobility. We have the following observation and proposals:

Proposal 1: Keep L1 measurement for FR2-1 in the second condition of known cell definition in CSI-RS based L1-RSRP measurement.

Proposal 2: The third condition of known cell definition in CSI-RS based L1-RSRP measurement is: SNR of the CSI-RS from the target cell configured for L1 measurement is above -3dB.

Proposal 3: For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, only consider the following procedure:

- Step 1: SSB based L3 measurements

- Step 2: SSB-based L1 measurements and reports on the same candidate cell

- Step 3: CSI-RS based L1 measurements on the same candidate cell

  - Note: The CSI-RS in Step 3 should be Type-D QCL with the SSB in Step 2 and configured with repetition OFF.

Proposal 4: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case.

Observation 1: Timing accuracy after L3 measurement is far from enough for the purpose of indicating RTD within CP or not.

Proposal 5: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).

Observation 2: If to support CSI-RS based L1-RSRP measurement outside active BWP, all CSI-RS in the same frequency layer shall be confined in the same 5ms window.

Observation 3: If to support CSI-RS based L1-RSRP measurement outside active BWP, concurrent gap shall be used.

Observation 4: If to support CSI-RS based L1-RSRP measurement outside active BWP, the definition of frequency layer should be discussed at first.

Proposal 6: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Proposal 7: If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”.

# 4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#113.