3GPP TSG-RAN4 Meeting # 114	R4-2500937

Athens, Grace, Feb.17th – 21th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on left open issues for ATG BS CA

# Agenda item:	7.12.2

# Document for:	Approval

# 1 Introduction

In the past meetings, there are several WFs[1~4] approved for the RF requirements for ATG BS supporting intra-band contiguous CA and inter-band CA in Rel-19. Although it was agreed to reuse the same requirements (including TAE requirements) as defined for the legacy NR BS RF CA requirements, companies spent several time cycles to discuss how to handle CA TAE requirements for ATG BS CA in the TS38.104 specification due to the DL MIMO is not included in the WID but requested by operator.

In RAN4 #112bis meeting, the agreements in the chairman’s note are:

Chair: The agreed way forward is to postpone the discussion on TAE rewording until the conclusion of the December plenary meeting. No further delay is expected.

In RAN#106 meeting, the revised WID [5] was approved where DL MIMO is added for BS:

- Specify the TAE requirements for support of DL MIMO for BS. [RAN4].

Therefore, the situation is clear now to solve the wording on CA TAE requirements for ATG BS CA in the specification. In this paper, we gives some discussions on left open issue which is the CA TAE requirement wording.

# 2	Discussion

In the last WF, it was agreed that:

- - There are no NB-IoT requirements applied for ATG BS supporting CA operation

- - For the CR to TS38.104 to introduce ATG BS CA, remove the sentence ‘There is no multi-band operation requirement for ATG BS.’ in clause 4.8.

It can be well reflected in the specification.

For TAE requirements, there were several options included in the summary [6] in RAN4 #112bis meeting.

### Issue 1-2: Draft CRs to TS38.104 to revise the wording on the TAE requirements - Proposal: - Option 1: Remove “There is no TAE requirement for ATG BS.” (ZTE) - Option 2: Replace “There is no TAE requirement for ATG BS” with “The requirements for MIMO transmission are not applicable to ATG BS” (CATT) - Option 3: Replace “There is no TAE requirement for ATG BS” with “The TAE requirements for NR BS are applicable for ATG BS, except for MIMO which is not supported for ATG BS.” (Ericsson) - Option 4: Update “There is no TAE requirement of MIMO transmission for ATG BS.” (Huawei)

It should noted that the existing TAE requirements are agreed to be reused in previous meeting. Since TAE requirements include CA and MIMO, so as long as MIMO is supported in the revised WID, then the existing TAE requirements can

also be applied.

Regarding the above four options, options 2,3 and 4 are the same direction which aim to exclude the MIMO in the TAE requirement for ATG BS based on the previous objections. We think it is clear now in the revised WID that the MIMO is included, so option 1 is simple and straightforward way.

Proposa1. Remove  “There is no TAE requirement for ATG BS.” in the specification.

An companion draft CR to TS38.104 was proposed in [7] in this meeting.

Proposa2. To endorsed the companion draft CR to TS38.104.

# 3 Conclusion

In this contribution, we gives some discussions on left open issues.

Proposa1. Remove  “There is no TAE requirement for ATG BS.” in the specification.

Proposa2. To endorsed the companion draft CR to TS38.104.

# 4	Reference

[1] R4-2406089, Way Forward for [110bis][312] NR_ATG_enh, ZTE Corporation

[2] R4-2409957, Way Forward for [111][312] NR_ATG_enh, ZTE Corporation

[3] R4-2413510, Way Forward for [112][306] NR_ATG_enh, ZTE Corporation

[4] R4-2419796, Way Forward for [113][305] NR_ATG_enh, ZTE Corporation

[5] RP-243285, Revised WID on Enhancements for Air-to-ground network for NR, CMCC, ZTE Corporation, CATT, Huawei, HiSilicon

[6] R4-2416526, Topic summary for [305] NR_ATG_enh, Moderator (ZTE Corporation)