3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500972

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Discussion on channel raster for band n102

Agenda item:	4.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on channel raster for band n102.

- Discussion

The channel raster for band n102 in Table 5.4.2.3-1 of TS 38.104(version: 18.8.0) is copied as below:

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1 NR operating band; ΔFRaster (kHz); Uplink range of NREF (First – <Step size> – Last); Downlink range of NREF (First – <Step size> – Last). n1023; 15; 796334 – <1> – 828333; 796334 – <1> – 828333. NOTE 3: Applicable NR-ARFCN for band n102 for 20 MHz channel bandwidth, NREF = {795668, 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668} for 40 MHz channel bandwidth, NREF = {797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000} for 60 MHz channel bandwidth, NREF = {798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332} for 80 MHz channel bandwidth, NREF = {799000, 804332, 809668, 815000, 820332, 825668} for 100 MHz channel bandwidth, NREF = {799668, 803668, 810332, 814332, 821000, 825000}.

As above, for TS 38.104 version 18.8.0, it is incorrect that the channel raster point (795668) for 20MHz channel bandwidth applicable for band n102 in NOTE3 is not in range of 796334 – <1> – 828333 of Uplink range of NREF or Downlink range of NREF for band n102.

Observation 1: For TS 38.104 version 18.8.0, it is incorrect that the channel raster point (795668) for 20MHz channel bandwidth applicable for band n102 in NOTE3 is not in range of 796334 – <1> – 828333 of Uplink range of NREF or Downlink range of NREF for band n102.

The channel raster for band n102 in Table 5.4.2.3-1 of TS 38.104(version: 17.16.0) is copied as below:

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1 NR operating band; ΔFRaster (kHz); Uplink range of NREF (First – <Step size> – Last); Downlink range of NREF (First – <Step size> – Last). n1023; 15; 796334 – <1> – 828333; 796334 – <1> – 828333. NOTE 3: Applicable NR-ARFCN for band n102 for 20 MHz channel bandwidth, NREF = {797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668} for 40 MHz channel bandwidth, NREF = {797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000} for 60 MHz channel bandwidth, NREF = {798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332} for 80 MHz channel bandwidth, NREF = {799000, 804332, 809668, 815000, 820332, 825668} for 100 MHz channel bandwidth, NREF = {799668, 803668, 810332, 814332, 821000, 825000}.

As above, for TS 38.104 version 17.16.0, for band n102, the channel raster point for 20 MHz channel bandwidth is different form that in TS 38.104 version 18.8.0. This is because that in Rel-17 no channel raster points were defined for 5925-5945MHz chunk of spectrum. From the UE perspective, no channel can be configured in 5925-5945MHz because there is no channel raster point for that range in TS 38.101-1. So the channel raster point (795668) for 20MHz channel bandwidth for band n102 in R17 TS 38.104 was removed in CR [2].

Observation 2: The channel raster for band n102 is 796334 – <1> – 828333 in both TS 38.104 version 18.8.0 and TS 38.104 version 17.16.0.

The channel raster for band n102 in Table 5.4.2.3-1 of TS 38.101-1(version: 17.16.0) is copied as below:

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band NR operating band; ΔFRaster (kHz); Uplink Range of NREF (First – <Step size> – Last); Downlink Range of NREF (First – <Step size> – Last). n1024; 15; 795000 – <1> – 828333; 795000 – <1> – 828333. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4.. Table 5.4.2.3-4: Allowed NREF (NR-ARFCN) for operation in Band n102 Channel Bandwidth; Allowed NREF. 20 MHz; 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668. 40 MHz; 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000. 60 MHz; 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332. 80 MHz; 799000, 804332, 809668, 815000, 820332, 825668. 100 MHz; 799668, 803668, 810332, 814332, 821000, 825000.

As above, the channel raster for band n102 in TS 38.101-1 is 795000 – <1> – 828333, meanwhile, the channel raster for band n102 in TS 38.104 is 796334 – <1> – 828333. So the channel raster for band n102 in TS 38.104 is not aligned with that in TS38.101-1.

We also check the CRs on band n102 for TS 38.104 [1, 2] and CR on band n102 for TS 38.101-1[3], the channel raster for band n102 in TS 38.101-1 is 795000 – <1> – 828333, and the channel raster for band n102 in TS 38.104 is 796334 – <1> – 828333, they are not alignment.

To align with TS 38.101-1, the CR[2] for TS 38.104 in RAN4#102 meeting remove 20MHz channel raster points (975668) for 5925-5945MHz in the lower 6GHz bands in TS 38.104, but this CR did not correct the channel raster range in TS 38.104 to align with that in TS 38101-1.

Observation 3: The channel raster 795000 – <1> – 828333 for band n102 in TS 38.101-1 is not aligned with channel raster 796334 – <1> – 828333 for band n102 in TS 38.104.

From contribution [4], during RAN#94 it was agreed in [5] and captured in chair minutes that:

Agreements: • A new band for unlicensed operation in the lower 6GHz band is defined as 5.925-6.425GHz. • The EU/CEPT NS flag(s) will be explicitly limited to the 5.945-6.425GHz range.

As above agreement, it indicate that band n102 was introduced as 5925-6425MHz instead of 5.945-6.425GHz. So the channel raster for band n102 should be aligned with 5925-6425MHz.

Observation 4: Band n102 was introduced as 5925-6425MHz, and the channel raster for band n102 should be aligned with 5925-6425MHz.

In Table 5.2-1 of TS 38.104, the Uplink operating band and Downlink operating band for band n102 is copied as below:

Table 5.2-1: NR operating bands in FR1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz); Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz); Duplex mode; Notes. n102; 5925 – 6425; 5925 – 6425; TDD; Note 3, Note 4.

As above the FUL,low  and FUL,high for band n102 is 5925MHz and 6425MHz respectively, the channel raster for FUL,low  and FUL,high can be calculated according to the following equation in TS 38.104.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs) Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster Range of frequencies (MHz); ΔFGlobal (kHz); FREF-Offs (MHz); NREF-Offs; Range of NREF. 0 – 3000; 5; 0; 0; 0 – 599999. 3000 – 24250; 15; 3000; 600000; 600000 – 2016666. 24250 – 100000; 60; 24250.08; 2016667; 2016667 – 3279165.

To ensure the frequency corresponding to the raster number should be an integer multiple of ΔFRaster, and frequency corresponding to the raster number should be within the operating band. So we can calculate channel raster with the following method:

- For channel raster for FUL,low, NREF =((FREF - FREF-Offs)*1000)/ΔFGlobal + NREF-Offs

where, FREF =ΔFRaster*ceiling(FUL,low*1000/ΔFRaster)/1000

- For channel raster for FUL,high, NREF = ((FREF - FREF-Offs )*1000)/ΔFGlobal + NREF-Offs,

where, FREF =ΔFRaster*floor(FUL,high*1000/ΔFRaster)/1000

Since the FUL,low =5925MHz and FUL,high =6425MHz are  in range of frequencies 3000 – 24250MHz, so ΔFGlobal = 15kHz,  FREF-Offs =3000MHz, and  NREF-Offs =600000 are applicable for FUL,low =5925MHz and FUL,high =6425MHz.

Using above method with ΔFRaster =15kHz for band n102 in Table 5.4.2.3-1 of TS 38.104:

1) For FUL,low =5925MHz,

FREF =ΔFRaster*ceiling(FUL,low*1000/ΔFRaster)/1000=15*ceiling(5925*1000/15)/1000 = 5925MHz.

NREF =((FREF - FREF-Offs)*1000)/ΔFGlobal + NREF-Offs = ((5925-3000)*1000/15)+ 600000 = 795000.                         (Eq.1)

2) For FUL,high = 6425MHz,

FREF =ΔFRaster*floor(FUL,high*1000/ΔFRaster)/1000 = 15*floor(6425*1000/15)/1000 = 6424.995MHz.

NREF =((FREF - FREF-Offs)*1000)/ΔFGlobal + NREF-Offs = ((6424.995-3000)*1000/15)+ 600000 = 828333                  (Eq.2)

From Eq.1 and Eq.2, the channel raster for band n102 should be 795000 – <1> – 828333 instead of 796334 – <1> – 828333. The channel raster for band n102 in TS 38.104 is incorrect.

Observation 5: The channel raster for band n102 in Table 5.4.2.3-1 of TS 38.104 should be 795000 – <1> – 828333 instead of 796334 – <1> – 828333.

Based on above analysis, we propose,

Proposal 1: Change the channel raster for band n102 in Table 5.4.2.3-1 of TS 38.104 from 796334 – <1> – 828333 to 795000 – <1> – 828333.

Two CRs [1, 2] on channel raster for band n102 for TS 38.104 are provided for approval.

- Conclusion

This contribution provides analysis on channel raster for band n102, and the following proposal is concluded as follows:

Observation 1: For TS 38.104 version 18.8.0, it is incorrect that the channel raster point (795668) for 20MHz channel bandwidth applicable for band n102 in NOTE3 is not in range of 796334 – <1> – 828333 of Uplink range of NREF or Downlink range of NREF for band n102.

Observation 2: The channel raster for band n102 is 796334 – <1> – 828333 in both TS 38.104 version 18.8.0 and TS 38.104 version 17.16.0.

Observation 3: The channel raster 795000 – <1> – 828333 for band n102 in TS 38.101-1 is not aligned with channel raster 796334 – <1> – 828333 for band n102 in TS 38.104.

Observation 4: Band n102 was introduced as 5925-6425MHz, and the channel raster for band n102 should be aligned with 5925-6425MHz.

Observation 5: The channel raster for band n102 in Table 5.4.2.3-1 of TS 38.104 should be 795000 – <1> – 828333 instead of 796334 – <1> – 828333.

Proposal 1: Change the channel raster for band n102 in Table 5.4.2.3-1 of TS 38.104 from 796334 – <1> – 828333 to 795000 – <1> – 828333.

Two CRs [1, 2] on channel raster for band n102 for TS 38.104 are provided for approval.

- References

- R4-2206366, CR for 38.104 to introduce n102, Nokia, Nokia Shanghai Bell, RAN4#102-e

- R4-2321019, Removing 20MHz channel raster points for 5925-5945MHz in the lower 6GHz bands, Apple Inc., Nokia, RAN4#109

- R4-2206364, Introduction of the lower unlicensed 6GHz band, Apple, Skyworks Solutions Inc., Mediatek Inc., RAN4#102-e

- R4-2205560, On band definition for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell, RAN4#102-e

- RP-213628, Moderator's summary of discussion [94e-51-R17-Lower6GHz-NR-U], RAN4 chair, RAN#94-e

- R4-2500973, CR for TS 38.104, Correction on channel raster for band n102, CATT, RAN4#114.

- R4-2500974, CR for TS 38.104, Correction on channel raster for band n102, CATT, RAN4#114.