3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500982

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Discussion on sync raster for band n254 for NTN

Agenda item:	5.2

Document for:	Approval

- Introduction

In this contribution, we provide our views on sync raster for band n254 for NTN.

- Discussion

The sync raster for band n254 in Table 5.4.3.3-1 of TS 38.108 (Rel-18) is copied as below:

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN) SAN operating band; SS Block SCS; SS Block pattern (NOTE); Range of GSCN (First – <Step size> – Last). n256; 15 kHz; Case A; 5429 – <1> – 5494. n255; 15 kHz; Case A; 3818 – <1> – 3892. 30 kHz; Case B; 3824 – <1> – 3886. n254; 15 kHz; Case A; 6215 – <1> – 6244. 30kHz; Case C; 6218 – <1> – 6241. NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]..

As above the sync raster for band n254 for 30kHz SS Block SCS is 6218 – <1> – 6241, but we calculated by the excel file for sync raster in 38817-01-f50.zip file, the calculated sync raster for band n254 for 30kHz SS Block SCS is 6220 – <1> – 6238 instead of 6218 – <1> – 6241. The detail calculation is in attached excel file.

Observation 1: The sync raster for band n254 for 30kHz SS Block SCS should be 6220 – <1> – 6238 instead of 6218 – <1> – 6241.

So, we propose,

Proposal 1: Change the sync raster for band n254 for 30kHz SS Block SCS from 6218 – <1> – 6241 to 6220 – <1> – 6238 in TS 38.108 and TS 38.101-5.

Two CRs [1, 2] on sync raster for band n254 for TS 38.108 and TS 38.101-5 are provided for approval.

- Conclusion

This contribution provides analysis on sync raster for band n254 for NTN, and the following proposal is concluded as follows:

Observation 1: The sync raster for band n254 for 30kHz SS Block SCS should be 6220 – <1> – 6238 instead of 6218 – <1> – 6241.

Proposal 1: Change the sync raster for band n254 for 30kHz SS Block SCS from 6218 – <1> – 6241 to 6220 – <1> – 6238 in TS 38.108 and TS 38.101-5.

Two CRs [1, 2] on sync raster for band n254 for TS 38.108 and TS 38.101-5 are provided for approval.

- References

- TR 38.817-01 V15.5.0 (2019-09)

- R4-2500983, CR for TS 38.108, Correction on sync raster for band n254 for NTN, CATT, RAN4#114.

- R4-2500984, CR for TS 38.101-5, Correction on sync raster for band n254 for NTN, CATT, RAN4#114.