3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500985

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Discussion on figures and definitions related to non-regenerative payload and gateway for IoT NTN

Agenda item:	5.9.2

Document for:	Approval

- Introduction

In this contribution, we provide our views on figures and definitions related to non-regenerative payload and gateway for IoT NTN.

- Discussion

In RAN4#111 meeting, the agreed CRs[1,2] updated definition of satellite and satellite access node for NR NTN, using “transparent  payload” terminology to replace “band pipe payload” terminology for definition of satellite, and using “satellite-gateway” and “transparent payload” terminologies to replace “gateway” and “transparent NTN payload” terminologies for definition of satellite access node. The updated definition in CRs[1,2] is copied as below:

satellite: A space-borne vehicle embarking a transparent payload, or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO) or Geostationary Earth Orbit (GEO). Satellite Access Node: node providing NR user plane and control plane protocol terminations towards NTN satellite capable UE, and connected via the NG interface to the 5GC. It encompasses a transparent payload on board a NTN platform, with satellite-gateway and gNB functions.

As above, the updated definition of satellite and satellite access node are more accurate.

At same time, the CRs[1,2]  also updated figures for requirement reference points  using “satellite payload” terminology to replace the  “NTN payload RF” terminology, the updated figures for requirement reference points for SAN type 1-H as example is copied as below:

Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H

As above, the dash line between satellite-gateway and satellite payload in CRs[1,2] is incorrect, since the feeder link is always needed for satellite payload (transparent  payload, or a regenerative payload), so it is reasonable to use solid line for connection between satellite-gateway and satellite payload. So the agreed CRs[3,4] in RAN4#113 meeting updated figures for requirement reference points using solid line to replace the dash line for connection between satellite-gateway and satellite payload as below:

Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H

The updated definition of satellite using “transparent  payload” terminology, definition of satellite access node using “satellite-gateway” and “transparent payload” terminologies, and updated figures for requirement reference points using “satellite payload” terminology and solid line for connection between satellite-gateway and satellite payload  for NR NTN  in agreed CRs[1,2,3,4] are more accurate. The corrections for NR NTN in agreed CRs[1,2,3,4] are missing for IoT NTN in TS 36.108 and 36.181.

Observation 1: The updated definition of satellite using “transparent  payload” terminology, definition of satellite access node using “satellite-gateway” and “transparent payload” terminologies, and updated figures for requirement reference points using “satellite payload” terminology and solid line for connection between satellite-gateway and satellite payload  for NR NTN  in agreed CRs[1,2,3,4] are missing for IoT NTN in TS 36.108 and 36.181.

Based on above analysis, we propose,

Proposal 1:  Add the same corrections for NR NTN in agreed CRs [1,2,3,4] to TS 36.108 and 36.181 for IoT NTN.

Two CRs [5, 6] on non-regenerative payload and gateway for IoT NTN for TS 36.108 and TS 36.181 are provided for approval.

- Conclusion

This contribution provides analysis on sync raster for band n254 for NTN, and the following proposal is concluded as follows:

Observation 1: The updated definition of satellite using “transparent  payload” terminology, definition of satellite access node using “satellite-gateway” and “transparent payload” terminologies, and updated figures for requirement reference points using “satellite payload” terminology and solid line for connection between satellite-gateway and satellite payload  for NR NTN  in agreed CRs[1,2,3,4] are missing for IoT NTN in TS 36.108 and 36.181.

Proposal 1:  Add the same corrections for NR NTN in agreed CRs [1,2,3,4] to TS 36.108 and 36.181 for IoT NTN.

Two CRs [5, 6] on non-regenerative payload and gateway for IoT NTN for TS 36.108 and TS 36.181 are provided for approval.

- References

- R4-2409549, CR to TS 38.108: Corrections for non-regenerative payload and gateway, Huawei, HiSilicon, RAN4#111

- R4-2410000, CR to TS 38.181: Corrections for non-regenerative payload and gateway, Huawei, HiSilicon, RAN4#111

- R4-2419834, CR for TS 38.108, Correction on NTN SAN requirement reference points, CATT, RAN4#113

- R4-2419835, CR for TS 38.181, Correction on NTN SAN requirement reference points, CATT, RAN4#113

- R4-2500986, CR for TS 36.108, Correction on non-regenerative payload and gateway for IoT NTN, CATT, RAN4#114.

- R4-2500987, CR for TS 36.181, Correction on non-regenerative payload and gateway for IoT NTN, CATT, RAN4#114.