3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500993

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Further discussion on spatial channel modelling methodology

Agenda item:	7.16.2

Document for:	Approval

- Introduction

In RAN4#113 meeting, a Way Forward for [113][320] NR_SCM was agreed, and remaining issues in WF need further discussion.

So, in this contribution, we provide our views on SCM for demodulation.

- Discussion

Angles of Departure and Angles of Arrival

As per the WF [1], the issue concerning AoD and AoA is shown as below:

#### Angles of Departure and Angles of Arrival Way Forward: Option 1: Replace the mean angle in the last position of departure and arrival angles generated equation by the desired angle as . Set desired based on the LOS direction between the TX and RX Option 2: Following 38.827 (i.e., desired = model).

The difference in Option 1 and Option 2 is just desired, which is desired mean angle (or predefined mean angle) for scaling CDL channel model. The model is the mean angle of the original channel model tables (tabulated CDL model) for CDL-A, CDL-B, CDL-C, CDL-D, and CDL-E in TR 38.901 or TR 38.827.  For Option2, it defines that desired = model, which indicate that the desired mean angle is original mean angle of the original CDL models, and TR 38.827 just allows to scale the angle spread (ASdesired) and does not allow to scale mean angle (desired) for CDL model, So option 1 is more flexible. Setting desired based on the LOS direction between the TX and RX in Option 1 can ensure that the mean angle for all paths is between the TX and RX.  According to the TR 38.901, the scaling of mean angle is not related to scenario and frequency, so it is not important parameter for CDL model, for SCM for demodulation, both Option 1 and Option 2 are acceptable to us.

Proposal 1: Adopt Option 1 for Angles of Departure and Angles of Arrival.

SNR Definition with CDL

As per the WF [1], the issue concerning SNR definition with CDL is shown as below:

#### SNR Definition with CDL Agreement: Define normalised receive signal power as the following: The sum of signal power is normalised across all Rx RF chains using a single normalisation factor. The normalisation factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain]. The normalisation factor will be aligned and can be included in TR 38.753. Interested companies are welcome to provide measured normalization factor for CDL channel with different AAV options in next meeting.

The max element gain and element pattern are defined and used in CDL model, it is agreed that the BS antenna radiation pattern as defined in TR 38.901 Table 7.3-1 copied as below is used for SCM demodulation.

Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) |  |
| Horizontal cut of the radiation power pattern (dB) |  |
| 3D radiation power pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

From above, Max element gain is 8dBi, and the normal element pattern gain of antenna element is varied with angle, so, the maximum element gain, and normal element pattern gain of antenna element in CDL model will affect the signal power and SNR. For TDL model, the Max element gain, and normal element pattern gain of antenna element are not included, only the beamforming combining gain are considered with MIMO correlation matrix. If we want to compare performance gain of CDL model to TDL model, we think it is necessary to consider the effect of maximum element gain and normal element gain of antenna element in CDL model on SNR.

Proposal 2: In order to compare the performance gain of CDL model and TDL model, it is necessary to consider the effect of maximum element gain and normal element gain of antenna element in CDL model on SNR.

- Conclusion

This contribution provides analysis on SCM for demodulation, and the following proposal is concluded as follows:

Proposal 1: Adopt Option 1 for Angles of Departure and Angles of Arrival.

Proposal 2: In order to compare the performance gain of CDL model and TDL model, it is necessary to consider the effect of maximum element gain and normal element gain of antenna element in CDL model on SNR.

- References

- R4-2419782, Way Forward for [113][320] NR_SCM, Moderator (Nokia), RAN4#113.