3GPP TSG-RAN WG4 Meeting #114                                                                                             R4-2501007

Athens, GR, Feb. 17 – Feb. 21, 2025

Source: 	CATT

Title: 	Discussion on UE RF requirements for NTN support spectrum less than 5 MHz

Agenda item:	7.10.3.2

Document for:	Approval

- Introduction

In RAN4 #113 meeting, a WF [1] on NR NTN support spectrum less than 5 MHz UE RF requirements was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE RF requirements for NTN support spectrum less than 5 MHz.

- Discussion

The open issues on UE RF requirements for NR NTN support spectrum less than 5 MHz was discussed during the previous meetings, as shown below:

# WF on UE RF requirements ### UE Rx requirements Issue 2-2-1: Reference sensitivity - Option 1: - 2.2dB improvement compared with 5 MHz CBW; - Option 2: - 1.7dB improvement compared with 5 MHz CBW; - Encourage companies to consider Rx non-linearity in the reference sensitivity evaluations Issue 2-2-2: Maximum input level - Agreement: - Reuse the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE, e.g. -40dBm; Issue 2-2-3: ACS - Agreement - 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset. Table 7.5-1: ACS for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz RX parameter; Units; Channel bandwidth (MHz). 3, 5, 10; 15; 20. ACS; dB; 33; 30; 27. Table 7.5-2: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1 RX parameter; Units; Channel bandwidth (MHz). 3; 5, 10; 15; 20. Power in transmission bandwidth configuration; dBm; REFSENS + 14 dB. Pinterferer4; dBm; REFSENS + 45.5 dB; REFSENS + 42.5 dB; REFSENS + 39.5. BWinterferer; MHz; 3; 5. Finterferer (offset)2; MHz; 3 /- 3; BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5). NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value.. Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2 RX parameter; Units; Channel bandwidth (MHz). 3; 5, 10; 15; 20. Power in transmission bandwidth configuration; dBm; -71.5; -68.5; -65.5. Pinterferer; dBm; -40. BWinterferer; MHz; 3; 5. Finterferer (offset); MHz; 3 /- 3; BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5). NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value..

For the UE Rx requirements with NTN UE support 3 MHz channel bandwidth, the REFSENS was discussed during the previous meeting. Compared to the 5 MHz channel bandwidth, the NRB of the NTN operating at 3 MHz channel bandwidth will reduce to 15 RBs, resulting in a 2.2 dB reduction in total received power. Meanwhile, we noticed that the impact of Rx non-linearity may need to be considered as it may degrade the performance of REFSENS (usually 0.5 dB), but this is usually due to the fact that the affected operating bands typically have a small duplex distance. For the NTN operating bands which will introduce 3 MHz channel bandwidth, the duplex distance are quite large. Therefore, we believe that the 2.2 dB REFSENSE improvement could still be considered.

Proposal 1: For NR NTN UE support 3 MHz channel bandwidth, 2.2 dB REFSENSE improvements could be considered.

- Conclusion

This contribution provides our views on UE RF requirements for NTN support spectrum less than 5 MHz. The following proposals are concluded as follows:

Proposal 1: For NR NTN UE support 3 MHz channel bandwidth, 2.2 dB REFSENSE improvements could be considered.

- References

- R4-2419789, WF on [113][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4 #113