3GPP TSG-RAN WG4 Meeting #114                                                                                             R4-2501009

Athens, GR, Feb. 17 – Feb. 21, 2025

Source: 	CATT

Title: 	Discussion on UE Rx requirement for NTN support HPUE

Agenda item:	7.10.2.2.2

Document for:	Approval

- Introduction

In RAN4 #113 meeting, a WF [1] on UE Rx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE Rx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Rx requirements for NTN support HPUE was achieved during the previous meetings, as shown below:

Issue 3-1-1: General considerations - Proposals - Option 1: For power class 2, there is no differentiation of handheld and non-handheld UE. (P2b/R4-2418201) - Option 2: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE. (P2/R4-2419717) Issue 3-2: RSD requirements for PC1.5 - Proposals - Option 1: For NR NTN PC 1.5 handheld HPUE, the RSD value depends on the architecture of HPUE (P1/R4-2417815) - Option 2: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only (P3/R4-2418201) - Option 3: no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016) - Option 4: [2.2] dB RSD for PC1.5 with 2Tx architecture for n256. Issue 3-3: RSD requirements for PC1 - Proposals - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (P4a/R4-2418201) - Option 1-1: FFS for PC1 with 2 or 3 Tx cases (P4b/R4-2418201) - Option 2 no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016)

The UL configuration for reference sensitivity was discussed during the previous meeting, but no agreement was made. Considering that the UL configuration of REFSENS in TS 38.101-1 captures only operating band/SCS/CBW and duplex modes (no power class), we believe that it is reasonable to follow this approach.

Proposal 1: For PC2 NR NTN UE, the REFSENS test can adopt the same UL configuration i.e. Table 7.3.2-2 in TS 38.101-5.

Since FR1 NR NTN UE and FR1 NR TN UE are implemented in a similar approach, the study of TN HPUE operating in single band FDD mode could be reused for NR NTN HPUE. Referring to the RSD study TR 38.861 and TR 38.896, the 2Tx architecture causes more severe interference compared to the 1Tx architecture, but the larger Tx-Rx separation prevents potential interference from the Tx chain.

For PC1.5 NTN HPUE, the Tx architectures has not yet been determined, and the architecture of the HPUE will have a significant impact on the conclusions of the RSD. For non-handheld PC 1.5 NTN HPUE, we believed that the RSD evaluation could be lower priority than that of handheld UE (due to the hardware capabilities). It is important to determine whether the PC1.5 NTN HPUE supports the 2Tx architecture and the corresponding RSD value could be derived. After determining the RSD for the handheld PC1.5 NTN HPUE, it can be discussed whether to apply the conclusions to non-handheld PC1.5 NTN HPUE.

Proposal 2: For NR NTN PC 1.5 handheld HPUE, the RSD value depends on the architecture of HPUE.

- Conclusion

This contribution provides our initial views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: For PC2 NR NTN UE, the REFSENS test can adopt the same UL configuration i.e. Table 7.3.2-2 in TS 38.101-5.

Proposal 2: For NR NTN PC 1.5 handheld HPUE, the RSD value depends on the architecture of HPUE.

- References

- R4-2420403, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #113