3GPP TSG-RAN WG4 Meeting #114	R4-2501014

# Athens , GR, 17th – 21st Feb, 2025

Title: 	Further discussion on less than 5MHz for NTN RRM requirement

Source: 	CATT

Agenda item:	7.10.3.4

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_IoT_NTN_req_test_enh was approved and some agreements were reached, but there still are a few issues left [1].

This document will further discuss the remaining RRM issues about less than 5MHz for NTN, and give our understanding and proposals.

- Discussion

2.2 Specific NTN RRM impacts

### CHO

In RAN4#112-bis meeting, RAN4 discussed CHO delay and agreed to extend to [3 Trs] in CHO without L3 measurement requirement. For CHO delay with L3 measurement requirement, since it is different from without L3 measurement, and CHO delay with L3 measurement delay include an additional component Tmeasure compared with CHO delay without L3 measurement, so it need to be further discussed. In the last meeting, RAN4 further discussed CHO with L3 measurement requirement, and the agreement is copied as follows [1]:

RAN4#112-bis: Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] RAN4#113: Agreements: - T∆ in CHO requirement for NTN in exiting TS38.133 is extended to [3 Trs] and - Tmeasure for less than 5MHz NTN conditional handover requirements can refer to “Tidentify_intra_with_index and Tidentify_inter_with_index” defined for less than 5MHz NTN measurement which is to be extended.

For CHO with L3 measurement, the specific value of extension for T∆ was still in square brackets and Tmeasure for less than 5MHz NTN conditional handover requirements will be extended. Regarding the specific value of extension for T∆, there were some disputes between companies. Some companies support to reuse the same value of extension for T∆ like CHO with L3 measurement and HO delay, that is, 3 Trs. However, from other companies’ view, Tmeasure has been agreed for extension, if T∆ also extends to 3 Trs like CHO without L3 measurement, the whole CHO with L3 measurement delay is too long.

We can understand that many companies believed that the delay extension should be the same because the rules for reducing SSB bandwidth in NTN and TN are exactly the same. However, RAN4 did not define any relevant CHO requirements for TN with less than 5MHz in current spec [2]. If considering the requirement of HO with less than 5MHz, the situation is different from CHO because Tmeasure is not included in the delay for HO.

Observation 1: There are no any relevant CHO requirements for TN with less than 5MHz in current spec.

Observation 2: If considering the requirement of HO with less than 5MHz, the situation is different from CHO because Tmeasure is not included in the delay for HO.

Since L3 measurement time needs to be considered in CHO, for the case where Tmeasure has already been extended as legacy, if the extension of T∆ for less than 5MHz is the same with TN HO, the total CHO delay will be too long. So what we want to discuss here is whether there is any possibility of further reduction in T∆ compared to 3Trs.

Observation 3: If the extension of T∆ for less than 5MHz is the same with TN HO, the total CHO delay for CHO with L3 measurement will be too long.

Proposal 1: RAN4 continue to discuss whether there is any possibility of further reduction in T∆ compared to 3Trs for CHO with L3 measurement.

From the network perspective, we believe that T∆ for CHO with L3 measurement delay with less than 5MHz may decrease, such as 2Trs.

Firstly, when Tmeasure is already extended, the UE can obtain information on PSS/SSS and SSB index for coarse synchronization during Tmeasure. Even if the UE uses the same algorithm as coarse synchronization during detecting PSS/SSS, it can still save some time for fine tracking because it has already read MIB information as .

Observation 4: It can save some time for fine tracking because it has already read MIB information during Tmeasure.

Secondly, in the last meeting, some companies believe that even if the UE decodes the SSB index during previous measurements, the UE will still need to check whether the SSB index is correct in T∆. In our opinion, even if the UE does have such a behavior of checking the SSB index again, the time in Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms is not completely serial, and there is a lot of time margin in the entire interruption delay. This check operation can be included in the existing total time and RAN4 doesn’t need to consider the additional time of this re-check operation on the top of existing total delay.

Therefore, for CHO without L3 measurement, when Tmeasure is already extended as legacy, T∆ in Tinterrupt can be extended to 2 Trs.

Observation 5: SSB re-check operation can be included in the existing total time and RAN4 doesn’t need to consider the additional time of this re-check operation on the top of existing total delay.

Proposal 2: For CHO without L3 measurement, when Tmeasure is already extended as legacy, T∆ in Tinterrupt can be extended to 2 Trs.

### Measurement(SSB index detection requirements)

In the last meeting, RAN4 discussed how to extend the number of SSB samples for Rel-18 NTN measurement requirements for intra-frequency and inter-frequency cases respectively, and the Wayforward is copied as follows [1]:

Wayforward: - Option 1: - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Option 2: Two sets of requirements - Set 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Set 2: smaller delay with higher SNR side condition. - FFS on the exact number of delay to be extended base on the simulation results under the same simulation assumptions for TN less than 5MHz [R4-2306400] - FFS on the separate UE capability to support set 2 requirements

For how to extend the number of SSB samples in Tidentify_intra/inter_with_index for a target cell with 12 PRB SSB for NTN, we support option 1 and believe that the same extension of SSB samples for intra-frequency and inter-frequency cases can be followed as TN with less than 5MHz.

In TN with less than 5MHz, the extension on the number of samples is based on the simulation result, and it is just due to the reduction of SSB bandwidth not related to scenario. Therefore, we believe that the extension shall not be changed by the scenario. If company think the extension of T∆ can be changed, more simulation will be needed.

For option 2, we prefer to use one set of requirements. Although set 2 is network friendly, we still believe that the SNR side condition is a common parameter, and it is unreasonable to use different SNR parameters for RAN4 in different requirements.

Proposal 3: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively.

Observation 6: If the extension of T∆ in HO requirement for NTN changes, the new simulation will be expected.

Observation 7: The SNR side condition is a common parameter, and it is unreasonable to use different SNR parameters for RAN4 in different requirements.

Proposal 4: Prefer to use one set of requirements for SSB index detection requirements.

- Conclusion

This document discussed the issues for less than 5MHz for NR NTN RRM requirements and presented following observations and proposals:

Observation 1: There are no any relevant CHO requirements for TN with less than 5MHz in current spec.

Observation 2: If considering the requirement of HO with less than 5MHz, the situation is different from CHO because Tmeasure is not included in the delay for HO.

Observation 3: If the extension of T∆ for less than 5MHz is the same with TN HO, the total CHO delay for CHO with L3 measurement will be too long.

Proposal 1: RAN4 continue to discuss whether there is any possibility of further reduction in T∆ compared to 3Trs for CHO with L3 measurement.

Observation 4: It can save some time for fine tracking because it has already read MIB information during Tmeasure.

Observation 5: SSB re-check operation can be included in the existing total time and RAN4 doesn’t need to consider the additional time of this re-check operation on the top of existing total delay.

Proposal 2: For CHO without L3 measurement, when Tmeasure is already extended as legacy, T∆ in Tinterrupt can be extended to 2 Trs.

Proposal 3: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively.

Observation 6: If the extension of T∆ in HO requirement for NTN changes, the new simulation will be expected.

Observation 7: The SNR side condition is a common parameter, and it is unreasonable to use different SNR parameters for RAN4 in different requirements.

Proposal 4: Prefer to use one set of requirements for SSB index detection requirements.

- Reference

[1]     R4-2420091, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)

[2]     3GPP TS 38.133, NR Requirements for support of radio resource management, V18.7.0