3GPP TSG-RAN WG4 Meeting #114				                                              R4-2501018

Athens , GR, 17th – 21st Feb, 2025

Title:	Discussion on RRM requirements for adaptation of common signal/channel transmission

Source:	CATT

Agenda Item:	7.25.2.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for adaptation of common signal/channel transmission were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for adaptation of common signal/channel transmission and give our views.

- Discussion

It was agreed in RAN4#112 meeting as below to discuss potential impacts on the following requirements due to SSB adaptation impact.

Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

RAN4 has agreed to discuss SSB periodicity adaptation in time domain as starting point according to RAN1 agreements. According to the existing L3 measurement requirements in TS 38.133 [3], we can see that the requirements are based on SMTC periodicity. If network only indicate the periodicity adaption in time-domain, the only potential difference is that this may lead to SMTC reconfiguration which may impact L3 measurement requirements. But since RAN2 has not replied to LS about the SMTC impact yet, we can continue to wait for the reply to further discuss the L3 measurement impact.

Proposal 1: Regarding the impact on L3 measurement requirements due to SSB adaptation, RAN4 continue to wait for RAN2's response on the SMTC configuration.

For SCell activation and TCI state switching, we understand it is not typical case to indicate SSB adaptation during the activation or switching procedure. Therefore, no impact due to SSB adaptation is expected for these requirements.

Proposal 2: No impact is expected for SCell activation and TCI state switching due to SSB adaptation.

In RAN1#118 meeting, RAN1 has the following agreements for SSB adaptation, and the conclusion in RAN1#119 meeting also is copied together below:

RAN1#118: Agreement - For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) RAN1#119: Conclusion - There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Since there is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) in RAN1#119. Based on our understanding, the SSB periodicity adaptation on PCell will not continue to be discussed in RAN1. From RAN4 perspective, RAN4 also will not to define the SSB adaptation requirement for PCell in Rel-19.

Observation 1: There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) in RAN1#119, which means that RAN1 will not continue to discuss the issue.

Proposal 3: RAN4 will not to define the SSB adaptation requirement for PCell in Rel-19.

- Conclusion

In this paper, we have some further discussions on adaptation of common signal/channel transmission, and the following proposals and observation are given:

Proposal 1: Regarding the impact on L3 measurement requirements due to SSB adaptation, RAN4 continue to wait for RAN2's response on the SMTC configuration.

Proposal 2: No impact is expected for SCell activation and TCI state switching due to SSB adaptation.

Observation 1: There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) in RAN1#119, which means that RAN1 will not continue to discuss the issue.

Proposal 3: RAN4 will not to define the SSB adaptation requirement for PCell in Rel-19.

- Reference

- R4-2420099, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN4#113

- R4-2418283, Topic summary for [113][225] Netw_Energy_NR_enh_Part2, Huawei, RAN4#113