3GPP TSG-RAN WG4 Meeting #114	R4-2501021

# Athens , GR, 17th – 21st Feb, 2025

Title: 	Discussion on CSI-RS based L1 measurement for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.27.2.2

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part1 was approved [1]. This document will discuss the RRM issues about CSI-RS based L1 measurement for LTM, and give our understanding and proposals.

- Discussion

- 2.1 Known cell definition in CSI-RS based L1 measurement

In RAN4#112-bis meeting, RAN4 has discussed known cell definition in CSI-RS based L1 measurement. In the last meeting, this issue has been divided into four parts for discussion, and RAN4 agreed to consider SSB based L3 measurement as the pre-requisite condition only in the first bullet. There is no conclusion yet on the other sub-issues in remaining bullets.

The previous agreements and candidate solutions for known cell definition in CSI-RS based L1 measurement are copied as follows [1]:

RAN4#112-bis: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition RAN4#113: Issue 3-1-1: Whether to consider SSB-based L3 measurement only? < Agreement > - Consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition. Issue 3-1-2: Whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet <Candidate solutions> - Option 1: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2. (Apple, Huawei, vivo) - Option 2: Remove Rel-18 LTM SSB-based L1-RSRP measurements. (CMCC, Nokia, Samsung, MTK, Ericsson) - Option 2a: Remove L1 measurement for FR2-1 as side condition and consider “The CSI-RS from the target cell configured for L1 measurement remains measurable” as requirement applicability. (Xiaomi) - Option 3: Keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress. (ZTE) Issue 3-1-3: Whether to use detectable or measurable? <Candidate solutions> - Option 1: The difference of the definition between detectable and measurable need to be further clarified. (CATT) - Option 2: Use detectable. (Apple, ZTE) - Option 3: Use measurable. (Xiaomi, OPPO, Ericsson, Nokia) Issue 3-1-4: Side condition for CSI-RS based L1 measurement for neighbor cell <Candidate solutions> - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (MTK) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Ericsson, Nokia, Saumsung)

For whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet, to make sure the associated SSB of the CSI-RS is detectable, we prefer to keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2.

Proposal 1: RAN4 to keep L1 measurement for FR2-1 in the second condition of known cell definition in CSI-RS based L1-RSRP measurement.

For whether to use detectable or measurable, we think that the difference of the defination between detectable and measurable need to be further clarified. As far as we know, the defination of ’detectable’ exists in current spec [2], but there is no defination of ’measurable’ defined in spec. Some companies think that CSI-RS is not used for detection, so it is not accurate to use the wording of “detectable” as SSB. Therefore, side condition may need to be further clarified for CSI-RS based L1 measurement.

Observation 1: The defination of ’detectable’ exists in current spec, but there is no defination of ’measurable’ defined in spec.

Observation 2: CSI-RS is not used for detection, so it is not accurate to use the wording of “detectable” as SSB.

Proposal 2: For third condition, the difference of the defination and side condition between detectable and measurable need to be further clarified.

- 2.2 Measurement procedure

For the whole CSI-RS based L1 measurement procedure, RAN4 has agreed the entire framework in the last meeting, but there was still FFS for the issue. The agreement is copied as below [1]:

<Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

The controversy on the above issue in the last meeting lies in whether to keep Step 2 and whether Step 2 can be skipped. RAN4 has agreed to keep Step 2, and need to further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. In our view, the main difference between skipping and not skipping Step 2 is that the UE uses different Rx beams, resulting in different gains and complexity of UE implementation, so it depends more on the UE. Therefore, we supported that whether Step 2 can be skipped can depend on UE capability.

Observation 3: the main difference between skipping and not skipping Step 2 is that the UE uses different Rx beams, resulting in different gains and complexity of UE implementation.

Proposal 3: Whether Step 2 can be skipped can depend on UE capability.

- 2.3 CSI-RS based L1-SINR measurement

In the last meeting, the candidate solutions for CSI-RS based L1-SINR measurement are copied as follows [1]:

<Candidate solutions> - Option 1: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. (CATT) - Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1. (Apple, Qualcomm, ZTE) - Option 3: RAN4 focus on CSI-RS based L1-RSRP measurement. (Samsung)

In RAN1#118-bis meeting, the measurement metrics for CSI-RS based L1 measurement has been discussed, and RAN1 had no consensus on the support L1-SINR measurement based on CSI-RS for candidate cells. The conclusions in RAN1 are copied as follows:

Conclusion (RAN1#118bis) - There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells.

As far as we know, RAN1 will not have a clear and explicit agreement about not introducing some requrements in RAN1 like RAN4, and 'no consensus ' means that RAN1 will not further discuss this issue and will not introduce CSI-RS based L1-SINR measurement. Therefore, we believe that RAN4 doesn’t need to further wait more RAN1 input and can make our decision, that is, RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. If other working groups have any new agreement on this issue, RAN4 can revisit our decision.

Observation 4: There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells, which means that RAN1 will not further discuss this issue and will not introduce CSI-RS based L1-SINR measurement.

Proposal 4: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

- 2.4 CSI-RS based L1 measurement outside active BWP or with measurement gap

In the last meeting, RAN4 discussed whether CSI-RS based L1 measurement can be outside active BWP, but there was no consensus among the companies. The candidate solutions are copied as below [1]:

<Candidate solutions> - Option 1: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM. (CATT) - Option 2: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK, Xiaomi, OPPO) - Option 3: Support CSI-RS based L1-RSRP measurement within active BWP and outside active BWP, and same principle can be used as start point, e.g., defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement. (ZTE) - Option 4: If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s). (vivo) - Option 5: From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: - Option 5a: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. - Option 5b: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. - Option 5c: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

As far as we know, the bandwidth of SSB is fixed, and the transmission bandwidth of the actual system may be much larger than that of SSB, and the measurements based on SSB may not be able to truly reflect the quality of the wireless link of DL outside the transmission bandwidth of SSB, which is one of the main reasons for the introduction of CSI-RS based L3 measurement.

For CSI-RS based L1 measurement, if RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited. Generally speaking, we think that CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario.

However, we also suggest firstly aligning the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies. In our view, it is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

Observation 5: If RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited.

Observation 6: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM.

Proposal 5: RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies.

- It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

- 2.5 L1 measurement on CSI-RS with repetition ON/OFF

In the last meeting, the candidate solutions for L1 measurement on CSI-RS with repetition ON and OFF are copied as below [1]:

- Measurement on CSI-RS with repetition ON <Candidate solutions> - Option 1: Considering the potential large overhead, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition ON in FR2-1. (MTK) - Option 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. (Huawei, CATT) - Option 2a: Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1. (vivo) - Measurement on CSI-RS with repetition OFF <Candidate solutions> - Option 1: From RRM requirements perspective, for CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1. (vivo) - Option 1a: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. Considering that there is no gain compared to SSB based L1 measurement, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition OFF in FR2-1. (MTK) - Option 1b: In FR2, RAN4 will define the requirements for the following case, (CATT) - CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB. - Option 2: Define first requirements for the case where the repetition is ‘OFF’ (Nokia)

Firstly, we want to clarify that what do CSI-RS repetition ON and OFF mean. According to TS 38.214, this parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Specifically, if the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction. If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping, and the gNB will use a wide beam.

Observation 7: CSI-RS repetition parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Observation 8: If the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction.

Observation 9: If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping.

In FR2, three typical cases can be analyzed:

For P1 case, wide beam will be used for gNB Tx and wide beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘OFF’, SSB based BM is used and rough beam is used for UE.

For P2 case, narrow beam will be used for gNB Tx and fixed wide beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘OFF’, then UE applies a fixed Rx beam obtained from SSB based L3 measurement.

For P3 case, fixed narrow beam will be used for gNB Tx and narrow beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘ON’, then UE sweeps its Rx beam with a finer beamwidth, we can at least agree to not support this case for neighbor cell.

Therefore, RAN4 will not to define the requirements for CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell. Considering the overall sweeping time and measurement latency, it is also feasible for UE to use SSB based coarse beam as P2, so RAN4 can define the requirements for the following case in FR2: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

Proposal 6: In FR2, RAN4 will not to define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell.

Proposal 7: In FR2, RAN4 will define the requirements for the following case:

- CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

- 2.6 Scenarios to be considered w.r.t. RTD

RAN4#112bis

- RTD is the received time difference among CSI-RS and SSB from different cells.

  - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP.

  - Alt 2: only define requirements for RTD<CP.

    - NW shall only enable this when RTD<CP is guaranteed.

RAN4#113

<Candidate solutions>

- Option 1: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case. (MTK)

- Option 2: Either Alt 1 or Alt 2 is ok. RTD reporting might be needed if option 2 is adopted. (Apple)

- Option 3: Support RTD>CP case for CSI-RS based L1-RSRP measurement. (CT)

  - Option 3a: define the requirements for RTD>CP. Need further clear definition of RTD.

- Option 4: Define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP. (vivo)

- Option 5: RTD < CP, and RTD > CP (Ericsson, ZTE)

  - Option 5a: RTD < CP, and RTD > CP UE capability for supporting RTD>CP can be introduced. (CMCC, vivo)

- Option 6: As the network does not know UE RTD conditions, requirements for RTD > CP shall be defined for rel-19 LTM. Whether the support of these requirements is up to UE capability or not can be FFS. (Nokia)

- Option 7: RAN4 to limit the applicable condition for CSI-RS-based L1-RSRP measurement to scenarios where RTD is not larger than a certain value, e.g., CP length. The detailed side condition can be discussed together with the solution for direct/indirect RTD value reporting to NW in RAN4#114. (Qualcomm)

- Option 8: RAN4 to consider introducing a solution to address the ambiguity issue regarding RRM requirement applicability due to RTD being larger than CP. The detailed solution can be discussed in RAN4#114. (Qualcomm)

For whether to support both RTD<CP and RTD>CP, the agreement and candidate solutions are copied as follows [1]:

For this issue, we understand the UE complexity for supporting RTD>CP for CSI-RS based L1-RSRP measurement, but it has been introduced in R18 LTM and the network does not know UE RTD conditions. Since it was an optional UE capability, so we think that RTD > CP UE capability for supporting RTD>CP can be also introduced for CSI-RS based L1-RSRP measurement.

Proposal 8: RAN4 to define the requirements for RTD < CP and RTD > CP, and RTD > CP UE capability for supporting RTD>CP can be introduced.

- 2.7 CSI-RS resource type

In the last meeting, the candidate solution for CSI-RS resource type is copied as below [1]:

<Candidate solutions> - Option 1: In addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells. (CATT) - Option 2: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS. (Apple, vivo) - Option 3: Start from periodic CSI RS measurements, and wait for wait until RAN1 agrees on aperiodic and semi-persistent CSI-RS support. (Nokia) - Option 4: RAN4 define CSI-RS based L1 measurement for periodic and semi-persistent. (Samsung) - Option 5: RAN4 should define CSI-RS based L1 measurement requirements for periodic, aperiodic and semi-persistent CSI-RS (ZTE)

This issue is related to RAN1 progress, and the working assumption agreed in RAN1#118bis meeting is copied as follows:

Working Assumption (Agreement (RAN1#118bis) - In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). - Support of semi-persistent CSI-RS is subject to UE capability. - Final LS in R1-2409283.

According to the above, in addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability. Therefore, in addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells.

Observation 10: In addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability.

Proposal 9: RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

- 2.8 Measurement gap related issues

In Rel-18, RAN4 only define the requirement for inter-frequency L1-RSRP measurement with type 1 MG. In Rel-19, if‘the CSI-RS of candidate cell outside active BWP’ is supported for LTM, RAN4 need to discuss whether to introduce measurement gap for CSI-RS based L1 measurement as L3.

For measurement gap related, RAN4 need to discuss whether to reuse or introduce the new gap for CSI-RS based measurement. In our view, at least the type 1 MG should be supported.

Proposal 10: In Rel-19, if  ’the CSI-RS of candidate cell outside active BWP’ is supported for LTM,

- RAN4 need to discuss whether to introduce measurement gap for CSI-RS based L1 measurement.

- RAN4 need to discuss whether to reuse type 1 MG in R18 LTM or introduce the new gap for CSI-RS based measurement.

Proposal 11: In Rel-19 LTM, if gap is introduced, at least the type 1 MG should be supported for CSI-RS based L1 measurement.

In the previous meeting, for the case of gap-based measurement, some companies were concerned about whether the existing gap can cover the CSI-RS configuration in the same frequency layer. In our view, the same problem exists in the legacy CSI-RS based L3 measurement, and the CSI-RS configuration is also flexible. RAN4 defined the requirements applicability for CSI-RS based L3 measurement and used time window to limit CSI-RS resourse transmission, the details in clause 9.10 of TS 38.133 are copied as follows [4]:

#### 9.10.3.2 Requirements applicability The associated SSB layer of the CSI-RS follows the same requirements as SSB based measurements defined in 9.3. The requirements in clause 9.10.3 apply, provided: - The associated SSB of the cell being identified or measured is detectable, and - All CSI-RS resources on one inter-frequency layer are configured within a window of up to 5ms, and - The periodicity of the configured CSI-RS resources is 10ms, 20ms or 40ms, and - CSI-RS resources for measurements and the associated SSB for cell identification are configured within measurement gap.

Therefore, at the very least, RAN4 can follow the same approach as CSI-RS based L3 measurement to address companies' concerns, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer.

Observation 11: The CSI-RS configuration is also flexible in the legacy CSI-RS based L3 measurement, and RAN4 defined the requirements applicability for CSI-RS based L3 measurement.

Proposal 12: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer.

Another feasible approach is to introduce related gap enhancements. In R18 LTM, RAN4 just supported R16 type 1 gap as a baseline. Considering that many gap enhancements have been specified in Gap WI, which also can be used for CSI-RS based L1 measurement in R19 LTM. If RAN4 to introduce the concurrent gap, the above issue can also be resolved.

In additon, according to RAN1 conclusions on the CSI-RS resource type, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability. If RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements.

Observation 12:  Considering that gap enhancement has been specified in Gap WI, concurrent gap can be used for CSI-RS-based L1 measurements to cope with flexible CSI-RS configurations.

Proposal 13: For CSI-RS based L1 with gap measurement, if RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements.

- Conclusion

This document discussed preliminary the issues for CSI-RS based L1 measurement for LTM in Rel-19 and the following proposals and observations are presented:

Proposal 1: RAN4 to keep L1 measurement for FR2-1 in the second condition of known cell definition in CSI-RS based L1-RSRP measurement.

Observation 1: The defination of ’detectable’ exists in current spec, but there is no defination of ’measurable’ defined in spec.

Observation 2: CSI-RS is not used for detection, so it is not accurate to use the wording of “detectable” as SSB.

Proposal 2: For third condition, the difference of the defination and side condition between detectable and measurable need to be further clarified.

Observation 3: the main difference between skipping and not skipping Step 2 is that the UE uses different Rx beams, resulting in different gains and complexity of UE implementation.

Proposal 3: Whether Step 2 can be skipped can depend on UE capability.

Observation 4: There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells, which means that RAN1 will not further discuss this issue and will not introduce CSI-RS based L1-SINR measurement.

Proposal 4: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

Observation 5: If RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited.

Observation 6: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM.

Proposal 5: RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies.

- It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

Observation 7: CSI-RS repetition parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Observation 8: If the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction.

Observation 9: If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping.

Proposal 6: In FR2, RAN4 will not to define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell.

Proposal 7: In FR2, RAN4 will define the requirements for the following case:

- CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

Proposal 8: RAN4 to define the requirements for RTD < CP and RTD > CP, and RTD > CP UE capability for supporting RTD>CP can be introduced.

Observation 10: In addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability.

Proposal 9: RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

Proposal 10: In Rel-19, if  ’the CSI-RS of candidate cell outside active BWP’ is supported for LTM,

- RAN4 need to discuss whether to introduce measurement gap for CSI-RS based L1 measurement.

- RAN4 need to discuss whether to reuse type 1 MG in R18 LTM or introduce the new gap for CSI-RS based measurement.

Proposal 11: In Rel-19 LTM, if gap is introduced, at least the type 1 MG should be supported for CSI-RS based L1 measurement.

Observation 11: The CSI-RS configuration is also flexible in the legacy CSI-RS based L3 measurement, and RAN4 defined the requirements applicability for CSI-RS based L3 measurement.

Proposal 12: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer.

Observation 12:  Considering that gap enhancement has been specified in Gap WI, concurrent gap can be used for CSI-RS-based L1 measurements to cope with flexible CSI-RS configurations.

Proposal 13: For CSI-RS based L1 with gap measurement, if RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements.

- Reference

[1]      R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

[2]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.7.0