3GPP TSG-RAN WG4 Meeting # 114                                                              R4-2501029

Athens, GR, 17th – 21st February, 2025

Title: 	                 Discussion on A-IoT BS requirements

Source: 	CATT

Agenda item:	7.24.2.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for A-IoT BS Tx and Rx requirements according to the WID [1].

- A-IoT BS RF requirements analysis

2.1 General

In WID [1], RF requirements for Type 1-C Ambient-IoT BS need to be specified. Scenario is D1T1-B, R2D is located in DL spectrum and D2R and CW are located in UL spectrum. Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

The A-IoT BS RF requirement analysis is based on WID [1]. D1T1-B has two scenarios, scenario 2-1 is the NR UEs distributed only outdoor, and scenario 2-2 is part of the NR UEs distributed indoor, this paper only analyses requirements of scenario 2-1.

2.2 Tx requirements

Tx RF requirements for A-IoT BS in TR [2] can be summarized in table 1.

Table 1: Table 1: Tx RF requirements for A-IoT BS

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | Yes |
| Output power dynamic | RE power control dynamic range | TBD in WI phase |
|  | Total power dynamic range | TBD in WI phase |
|  | Power boosting | No |
| Transmit on/off | Transmitter off power | TBD in WI phase |
|  | Transmitter transient period | TBD in WI phase |
| Transmit signal quality | Frequency error | Yes |
|  | EVM | Yes |
|  | TAE | No |
| Unwanted emissions | Occupied bandwidth (OBW) | Yes |
|  | Adjacent Channel Leakage Power Ratio (ACLR) | Yes |
|  | Operating band unwanted emissions (OBUE) | Yes |
|  | Transmitter spurious emissions | Yes |
| Transmitter intermodulation |  | TBD in WI phase |

2.2.1 Base station output power

Type 1-C A-IoT BS need to be specified and A-IoT BS located indoor, so the base station output power of A-IoT micro-BS can be specified as 33dBm based on RAN4 simulation assumption in SI phase. The number of RBs of R2D signal and the number of RBs corresponding to the BS output power are not clarified in the WID, and they need to be clarified in the WID.

Proposal 1: The base station output power of A-IoT micro-BS can be specified as 33dBm.

Proposal 2: The number of RBs of R2D signal and the number of RBs corresponding to the BS output power need to be clarified in the WID.

2.2.2 Output power dynamic

In TR [2], A-IoT BS does not support power boosting, but RAN 1 does not have a conclusion on this requirement, if there is a conclusion later, this requirement needs to be updated according to the conclusion of RAN 1.

The necessity of RE power control dynamic range can be decided after RAN1 have a conclusion on whether A-IoT BS supports RE power boosting.

Proposal 3: The conclusion of power boosting needs to be updated according to the conclusion of RAN 1.

Proposal 4: The necessity of RE power control dynamic range can be decided after RAN1 have a conclusion on whether A-IoT BS supports RE power boosting.

If the signal resource of R2D is several RBs, the total power dynamic range requirement is necessary.

Proposal 5: Total power dynamic range requirement is necessary if signal resource of R2D is several RBs.

2.2.3 Transmit on/off

In the current BS Tx requirements, transmit OFF power requirement only applies to TDD operation, according to the WID, R2D is located in DL spectrum and D2R and CW are located in UL spectrum, the operation is not TDD, so the requirement is not needed.

Proposal 6: Transmitter OFF power requirement is not needed when the frequency spectrums of R2D and D2R/CW are different.

For an A-IoT BS communicating with device 1,  the A-IoT BS doesn’t transmit R2D signal all the time and the transmission needs to be turned on and off, and R2D transmission and CW transmission /D2R signal reception operations are TDM, but the R2D spectrum and CW/D2R spectrums are different, this requirement is not needed.

Proposal 7: Transmitter transient period requirement is not needed when R2D spectrum and CW/D2R spectrums are different.

2.2.4 Transmit signal quality

2.2.4.1 Frequency error

For NR BS, frequency error for MR BS is ±0.1 ppm,  this frequency error can be reused for Ambient IoT BS.

Proposal 8: Frequency error for NR MR BS can be reused for A-IoT BS.

2.2.4.1 EVM

EVM requirement of A-IoT BS is different from EVM of legacy NR BS, as OOK signal is not mapped to the legacy constellation. RAN4 needs to analyse the method of new EVM design according to the signal design of RAN1.

Proposal 9: RAN4 needs to analyse the method of new EVM design according to the signal design of RAN1.

The transmit signal quality requirements apply during the transmitter ON period.

Proposal 10: The transmit signal quality requirements apply during the transmitter ON period.

2.2.5 Unwanted emissions

2.2.5.1 Occupied bandwidth

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 (=0.5%) of the total mean transmitted power.

The same principle with existing NR (99% power should be within BS channel bandwidth) can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth.

Proposal 11: The same principle with existing NR can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth

2.2.5.2 ACLR

Based on the simulation results of scenario 2-1, the ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB.

Proposal 12: The ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB.

2.2.5.3 OBUE

The channel bandwidth of standalone NB-IoT BS is 200 KHz, and it has guardbands at the low and up ends inside the channel, transmission bandwidth configuration of it is 180 KHz, so OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

Proposal 13: OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

2.2.5.4 Transmitter spurious emission

The existing LTE transmitter spurious emission requirements can also be applied to BS that supports E-UTRA with NB-IoT in-band/guard band operation or NB-IoT standalone operation. The existing NR transmitter spurious emission requirements can be reused by BS supports NB-IoT operation in NR in-band. The requirements in-band of LTE and NR are same. According to the description of the requirements of LTE BS and NR BS, the requirements of LTE BS can be reused for A-IoT BS.

Proposal 14: The transmitter spurious emission requirements of LTE BS can be reused for A-IoT BS.

The unwanted emissions requirements apply during the transmitter ON period.

Proposal 15: The unwanted emissions requirements apply during the transmitter ON period.

2.2.6 Transmitter intermodulation

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. Although the A-IoT BS R2D signal use NR BS DL spectrum, but the two BSs are not co-located, so the transmitter intermodulation requirement is not necessary.

Proposal 16: The A-IoT BS and NR BS are not co-located, so the transmitter intermodulation requirement is not necessary.

2.3 Rx requirements

Rx RF requirements for A-IoT BS in TR [2] can be summarized in table 2.

Table 2: Rx RF requirements for A-IoT BS

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | Yes |
| Dynamic range |  | TBD in WI phase |
| Adjacent Channel Selectivity（ACS） |  | Yes |
| In-band blocking | In-band blocking | Yes |
|  | Narrowband blocking | Yes |
| Out-of-band blocking |  | Yes |
| Receiver spurious emissions |  | Yes |
| Receiver intermodulation | Receiver intermodulation | Wait for RAN1 conclusion. |
|  | Narrowband intermodulation | Yes |
| In-channel selectivity (ICS) |  | TBD in WI phase |

2.3.2 ACS

Based on the simulation results of scenario 2-1, i.e. D1T1-B, NR UEs only distributed outdoor, R2D is located in DL spectrum and CW/D2R are located in UL spectrum, the ACS for NR BS can be reused for A-IoT BS.

Proposal 17: The ACS for NR BS can be reused for A-IoT BS.

2.3.3 In-band blocking

In scenario 2-1, i.e. the NR UEs only distributed outdoor, the distances between NR UEs and A-IoT BSs are relatively far, the narrow-band blocking requirement is not necessary.

Proposal 18: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband blocking requirement is not necessary.

2.3.4 Out-of-band blocking

The out-of-band blocking requirements for NB-IoT BS operating standalone is specified in TS 36.104, and the transmission bandwidth configuration of standalone is 180KHz,  so these requirements can be reused for A-IoT BS operating standalone or in-band.

Proposal 19: The out-of-band blocking requirements for NB-IoT BS operating standalone can be reused for A-IoT BS operating standalone or in-band.

2.3.5 Receiver spurious emissions

The receiver spurious emissions requirements of type 1-C NR BS can be reuse for NR with NB-IoT operating in-band, and these requirements of LTE BS can be reused for E-UTRA with NB-IoT in-band/guard band operation or NB-IoT standalone operation, and the transmission bandwidth configuration of standalone is 180KHz, so these requirements of LTE BS can be reused for A-IoT BS operating in-band or standalone.

Proposal 20: The receiver spurious emissions requirements of LTE BS can be reused for A-IoT BS operating in-band or standalone.

2.3.6 Receiver intermodulation

In scenario 2-1, i.e. the NR UEs only distributed outdoor, the distances between NR UEs and A-IoT BSs are relatively far, receiver intermodulation and narrowband intermodulation requirements are not necessary.

Proposal 21: For D1T1-B scenario and NR UEs only distributed outdoor, the receiver intermodulation requirement is not necessary.

Proposal 22: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband intermodulation requirement is not necessary.

- Summary

This contribution provides our further analysis for A-IoT BS requirements. We have the following proposals.

Proposal 1: The base station output power of A-IoT micro-BS can be specified as 33dBm.

Proposal 2: The number of RBs of R2D signal and the number of RBs corresponding to the BS output power need to be clarified in the WID.

Proposal 3: The conclusion of power boosting needs to be updated according to the conclusion of RAN 1.

Proposal 4: The necessity of RE power control dynamic range can be decided after RAN1 have a conclusion on whether A-IoT BS supports RE power boosting.

Proposal 5: Total power dynamic range requirement is necessary if signal resource of R2D is several RBs.

Proposal 6: Transmitter OFF power requirement is not needed when the frequency spectrums of R2D and D2R/CW are different.

Proposal 7: Transmitter transient period requirement is not needed when R2D spectrum and CW/D2R spectrums are different.

Proposal 8: Frequency error for NR MR BS can be reused for A-IoT BS.

Proposal 9: RAN4 needs to analyse the method of new EVM design according to the signal design of RAN1.

Proposal 10: The transmit signal quality requirements apply during the transmitter ON period.

Proposal 11: The same principle with existing NR can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth

Proposal 12: The ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB.

Proposal 13: OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

Proposal 14: The transmitter spurious emission requirements of LTE BS can be reused for A-IoT BS.

Proposal 15: The unwanted emissions requirements apply during the transmitter ON period.

Proposal 16: The A-IoT BS and NR BS are not co-located, so the transmitter intermodulation requirement is not necessary.

Proposal 17: The ACS for NR BS can be reused for A-IoT BS.

Proposal 18: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband blocking requirement is not necessary.

Proposal 19: The out-of-band blocking requirements for NB-IoT BS operating standalone can be reused for A-IoT BS operating standalone or in-band.

Proposal 20: The receiver spurious emissions requirements of LTE BS can be reused for A-IoT BS operating in-band or standalone.

Proposal 21: For D1T1-B scenario and NR UEs only distributed outdoor, the receiver intermodulation requirement is not necessary.

Proposal 22: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband intermodulation requirement is not necessary.

Reference

[1] RP-243326, “Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, RAN#106

[2] TR 38.769