3GPP TSG-RAN WG4 Meeting # 114                                                              R4-2501030

Athens, GR, 17th – 21st February, 2025

Title: 	                 Discussion on RF requirements for CW for D1T1

Source: 	CATT

Agenda item:	7.24.2.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for RF Requirements for CW for scenario D1T1-B according to the WID [1].

- RF Requirement for CW for D1T1

2.1 General

In WID [1], RF requirements for Type 1-C Ambient-IoT BS need to be specified. Scenario is D1T1-B, R2D is located in DL spectrum and D2R and CW are located in UL spectrum. Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. For scenario D1T1-B, the CW is transmitted from outside the topology.

2.2 CW requirements

RF Requirement for CW for D1T1 in TR [2] can be summarized in table 1.

Table 1: RF Requirement for CW for D1T1

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | Yes |
| ON/OFF power | TBD in WI phase |
| Frequency Error | Yes |
| Phase noise | No for inside topology TBD in WI phase for outside topology |
| Unwanted emission | Yes |
| Spurious emissions | Yes |
| Intermodulation | TBD in WI phase |

2.3.1 Output power

In scenario D1T1-B, CW is transmitted from outside the topology and the CW spectrum is located in FDD UL operating band, so the output power of CW can be specified as 23dBm.

2.3.2 ON/OFF power

In the D1T1-B scenario, CW is transmitted from outside the topology, CW nodes only transmit signals and don’t receive signals, and the CW/D2R spectrum and R2D spectrum are different, so the ON/OFF power requirement is not necessary.

2.3.3 Frequency Error

For NR BS, frequency error for MR BS is ±0.1 ppm,  this frequency error can be used for CW signal.

2.3.4 Phase noise

For D1T1-B scenario, the carrier-wave source is separately deployed from the D2R receiver, the total CW signal cancellation capability is based on spatial isolation for separate TX/RX antenna configuration, analog cancellation and digital cancellation.

The spatial isolation relies on the distance and antenna side lobe suppression between the CW node and A-IoT node. The analog cancellation performance depends on the phase and amplitude error between the reconstructed and actual received CW interference. In the digital baseband, suppression can be achieved by filtering, and digital interference cancellation can also be achieved by using a low complexity self-interference active cancellation technique.

According to TR [2], CW interference signal levels at the A-IoT BS receiver connectors are close to the narrowband blocking requirements of small cell NR base stations at distances of 20 meters and 10 meters, respectively. Therefore, there is no blocking issue.

Considering distance and antenna side lobe suppression between the CW node and A-IoT BS, the isolation is 67dB at 20 meters distance, so the interference signal level at the A-IoT BS receiver connectors are -44dBm, this value is equal to the narrowband blocking requirements of  existing Medium Range NB-IoT BS. Therefore, the receiver will not be blocked by the interference signal.

Spatial isolation and analog CW signal cancellation capability can reach about (61+10) =71 dB at 10 meters distance. CW signal cancellation ability of the digital unit can be analysed based on the occupied bandwidth and encoding gain of the D2R signal, it is needed to wait for the discussion results of RAN1 on these two parameters.

2.3.5 Unwanted emission

The CW signal is a single frequency point signal, so it is enough to consider the phase noise, and there is no need to consider the unwanted emission.

2.3.6 Spurious emissions

Spurious emissions requirements of NB-IoT BS can be reused for CW signal, especially interferences at CW harmonic frequencies.

2.3.1 Intermodulation

Since continuous waves with multiple tones are allowed in RAN1, reverse IM products from the same location/nearby continuous wave transmitter should be suppressed. However, in WID [1], it is specified that carrier wave is only waveform 1 and without hopping, so it is a single tone, and the intermodulation requirement is not needed.

- Summary

This contribution provides our analysis for RF Requirements for CW for scenario D1T1-B according to the WID [1], and our proposals can be summarized in the following table:

| RF Requirement for CW for D1T1-B | Requirements proposals |
| --- | --- |
| Output power | 23dBm |
| ON/OFF power | This requirement is not necessary for scenario D1T1-B. |
| Frequency Error | ±0.1 ppm |
| Phase noise | It is needed to wait for RAN1’s conclusions on occupied bandwidth and encoding gain of D2R signal to analyse CW signal cancellation ability of the digital unit. |
| Unwanted emission | This requirement is not necessary for scenario D1T1-B. |
| Spurious emissions | The requirements for NB-IoT BS can be reused for CW. signal. |
| Intermodulation | This requirement is not necessary for scenario D1T1-B with only single tone CW. |

Reference

[1] RP-243326, “Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, RAN#106

[2] TR 38.769