3GPP TSG-RAN WG4 Meeting # 114                                                              R4-2501031

Athens, GR, 17th – 21st February, 2025

Title: 	                 Discussion on time mask of low band carrier aggregation

Source: 	CATT

Agenda item:	7.8.2

Document for:	Discussion

- Introduction

This contribution provides our analysis for time mask for carrier switching according to the WID [1].

- Time mask analysis

2.1 General

According to WID [1], we need to analyse the time mask for carrier switching, and the switching is between case 1 and case 2. Case 1 is Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2, and case 2 is Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1.

In WI phase, we need to analyse CA_n5A-n29A, with additional band combinations to be handled via the basket work item approach. In CA_n5A-n29A, n5 is FDD band and n29 is SDL band.

2.2 Switching period analysis

Figure 1 shows the architecture for the purpose of understanding the switching, it only shows one Tx chain and one Rx chain for illustration.  The Tx chain supports carrier 1, and the Rx chain is shared by carrier 1 and carrier 2, but RF BPF of Rx chain can’t be shared. In WID [1], since the carrier frequency is less than 1GHz in all cases, and the LO frequency span corresponding to the downlink of the two carriers is also relatively small, so a single DL PLL device may cover this range.

Figure 1 UE architecture

Table 1 Switching between Case 1 and Case 2

| Case 1 to case 2 | Case 2 to case 1 |
| --- | --- |
| (1) Rx chain is switched from carrier 1 to carrier 2: - RF BPF switching - PLL needs to be reconfigured - ADC sampling rate adjustment - New data samples generated - Digital filters need to be settled - Others (2) Carrier 1 Tx chain: turned off | (1) Rx chain is switched from carrier 2 to carrier 1: - RF BPF switching - PLL needs to be reconfigured - ADC sampling rate adjustment - New data samples generated - Digital filters need to be settled - Others (2) Carrier 1 Tx chain: turned on |

Table 1 summarizes the switching between case 1 and case 2, it includes Rx chain switching and Tx chain turned off/ turned on. Rx and Tx operations can be done in parallel, and some actions of Rx switching can also be done in parallel. It can be seen that the Rx switching time from case 1 to case 2 and case 2 to case 1 is equal, so specify only a single value for Rx switching period is enough. In order to reduce the power consumption, when case 1 is switched to case 2, the Tx chain of carrier 1 is turned off.

We can refer to the analysis method of Tx train switching time to analyse Rx train switching time, the existing Tx train switching period is {35us, 140 us, 210us}, the analysis method of Tx train switching is described in [2],

35 usec was justified by the switching between different RF chains and 140 usec was justified by LO retuning time and switching can be parallelized with LO retuning. 250 usec was justified by initializing TX chains with needed predistortion and ET coefficient uploading together with LO tuning which in part may be parallelized but not all functions can be run in parallel hence more time than just PLL retune is needed.

In table 1, if reconfigured time of PLL is less than switching times of other actions, Rx chain switching time may also require 35us, but in some special cases, reconfigured time of PLL is more than switching times of other actions, Rx chain switching time requires 140us. The Tx chain action is just turned off / turned on, and it can also be done in parallel with Rx switching, so it has no impact on the Rx switching time. Tx chain is just turned off / turned on, and doesn’t need to be initialized, so 210 us is not required.

Observation 1: The Rx chain switching requires 35us.

Observation 2: In some special cases, reconfigured time of PLL is more than switching times of other actions, Rx chain switching time requires 140us.

Observation 3: Tx chain is just turned off / turned on, and doesn’t need to be initialized, so 210 us is not needed.

Proposal 1: RAN4 to specify a single value for the switching period from case1 to case 2 and case 2 to case 1.

Proposal 2: RAN4 to specify the switching period between case 1 and case 2 as {35us, 140 us} for  low band carrier aggregation with switching.

- Summary

This contribution provides our further analysis for time mask for carrier switching according to the WID [1], and our observations and proposals can be summarized in the following:

Observation 1: The Rx chain switching requires 35us.

Observation 2: In some special cases, reconfigured time of PLL is more than switching times of other actions, Rx chain switching time requires 140us.

Observation 3: Tx chain is just turned off / turned on, and doesn’t need to be initialized, so 210 us is not needed.

Proposal 1: RAN4 to specify a single value for the switching period from case1 to case 2 and case 2 to case 1.

Proposal 2: RAN4 to specify the switching period between case 1 and case 2 as {35us, 140 us} for  low band carrier aggregation with switching.

Reference

[1] RP-243317, “low band carrier aggregation via switching”, Apple, TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE, RAN#106

[2] R4-2000113, “Switching time details”, Qualcomm Incorporated, RAN4#94

[3] R4-1910751, “Switching times and reference architectures”, Qualcomm Incorporated, RAN4#92-Bis