3GPP TSG-RAN WG4 Meeting #114				                                               R4-2501044

Athens, GR, February 17th – February 21st, 2025

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Source:	CATT

Agenda Item:	7.17.2.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the left open issues can be found in the topic summary [2]. In this paper, we will continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing Rx beam sweeping factor (BSF) and give our views.

2 Discussion

In RAN4#112bis meeting, as agreed below, faster beam sweeping always applies during handover and RRC re-establishment while trigger conditions are needed for intra-frequency and inter-frequency L3 measurements to activate or deactivate the Multi-Rx operation. And in last meeting, RAN4 further agreed that multi-Rx for L3 measurement is activated based on serving cell’s signal quality threshold.

RAN4#112bis: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment. RAN4#113: Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

However, the conditions that trigger UE to quit L3 measurement delay reduction by optimizing Rx BSF need further discussion. Candidate options are copied below for reference:

RAN4#113: For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”: - Option 2 (LGE, OPPO, Ericsson, MTK, CTC): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (HW, Samsung, Ericsson): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. - Option 7 (Samsung, Ericsson): Introduce validity timer configuration with certain ranges via dedicated RRC signalling

Firstly, we want to clarify that the serving cell’s signal quality threshold can also be used as quit condition, e.g., when serving cell RSRP is higher than the configured signal quality threshold, UE can quit “L3 measurement delay reduction by optimizing Rx BSF” after finishing the on-going measurement.

Observation 1: Serving cell’s signal quality threshold can also be used as a quit condition.

Secondly, three candidate options could fulfill the purpose of quitting L3 measurement delay reduction by optimizing Rx BSF for UE, but the delay reduction gain that each option brings is different. For option 2, as discussed in Rel-18, using UAI ‘multiRx-PreferenceFR2’ for power saving to indicate that UE will quit “L3 measurement delay reduction by optimizing Rx BSF” may not allow UE to quit the mode on time, because NW needs to indicate UE to quit via signalling and UE is not expected to quit the mode autonomously. For option 3, the gain of delay reduction by optimizing Rx BSF will be quite limited, considering usually L3 measurements are periodic while this solution only allows UE to apply the delay reduction for L3 measurement for one qualified report. Using a validity timer with the value configured by NW may find a balance between delay reduction gain and UE power consumption.

Proposal 1: RAN4 to consider the serving cell’s signal quality threshold and a validity timer as the quit conditions.

Lastly, the above configurations and UE behaviours need to be specified in RAN2 spec, but since this is a RAN4 led work item and no dedicated TU is allocated to RAN2. Therefore it is better to decide all the conditions and thresholds in RAN4 and then send LS to inform RAN2.

Proposal 2: Send LS to RAN2 about the criteria/thresholds for triggering/deactivating Multi-Rx operation on L3 measurements and the corresponding UE behaviors after the whole framework is decided.

Regarding the SSB processing time, some agreements were achieved in last meeting with scenario 5 unresolved:

Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

As we discussed in our previous contribution, there is an already extra processing time for handover, so we do not think this additional delay is needed.

Proposal 3: For handover, no additional processing delay is needed.

RAN4#113 Agreement: Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability: - FFS: details of the capability

For the solutions of optimizing Rx BSF, as we discussed previously, the conclusions in Rel-18 Multi-Rx can be used as baseline, i.e., for UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability, which is quite straightforward.

Proposal 4: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Although we do not explicitly agree to discuss the requirements for CSI-RS measurements, after the requirements for SSB measurement is decided, the enhanced Rx BSF can also apply to the associated SSB in CSI-RS based measurement since it refers to formula of SSB based measurement, but no spec updates are needed. Similarly for handover, since the requirements for CHO and DAPS HO refer to the formula of SSB based measurement and handover respectively, the enhanced Rx BSF can also apply but no specification impact is expected.

Proposal 5: The reduced Rx BSF in proposal 8 can be applied to the following formula according to the trigger conditions to be specified:

- TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap;

- Tsearch for FR1-FR2 HO and FR2-FR2 HO;

- Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay;

- Tidentify-NR for RRC connection release with redirection to NR.

Proposal 6: The reduced Rx BSF in proposal 4 can apply to the associated SSB in CSI-RS based measurement, CHO and DAPS HO without new specification impact.

Another open issue is whether to define transition period requirements for the transition between measurements with Multi-Rx and without Multi-Rx operation, and different opinions were brought up in last meeting. In the existing specification, there are such requirements defined for the transition between measurements with gaps and without gaps. If we are going to define the requirements for transition between faster and slower measurements due to Multi-Rx operation, we think it is simple and reasonable to reuse the existing requirements, i.e., use the longer delay between the two statuses. But we see there may be some differences between the Multi-Rx operation and legacy gap impact. Based on the discussions on trigger condition, whether UE is performing the measurements with or without Multi-Rx operation depends on UE current status, and network is not aware of it and never know whether UE has such transition. Then it is possible that UE always applies the longer delay, i.e., with the legacy beam scaling factor. So we think it is meaningless to define the transition requirements.

Proposal 7: Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

3 Conclusion

In this paper, we have some further discussions on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, and the following proposals are given:

Observation 1: Serving cell’s signal quality threshold can also be used as a quit condition.

Proposal 1: RAN4 to consider the serving cell’s signal quality threshold and a validity timer as the quit conditions.

Proposal 2: Send LS to RAN2 about the criteria/thresholds for triggering/deactivating Multi-Rx operation on L3 measurements and the corresponding UE behaviors after the whole framework is decided.

Proposal 3: For handover, no additional processing delay is needed.

Proposal 4: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Proposal 5: The reduced Rx BSF in proposal 8 can be applied to the following formula according to the trigger conditions to be specified:

- TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap;

- Tsearch for FR1-FR2 HO and FR2-FR2 HO;

- Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay;

- Tidentify-NR for RRC connection release with redirection to NR.

Proposal 6: The reduced Rx BSF in proposal 4 can apply to the associated SSB in CSI-RS based measurement, CHO and DAPS HO without new specification impact.

Proposal 7: Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

4 References

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#113.

- R4-2418276, Topic summary for [113][218] NR_RRM_Ph5_Part1, Apple, RAN4#113.