3GPP TSG RAN WG4 Meeting #114		R4-2501059

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Discussion on release independence for 7MHz channel bandwidth

Agenda Item:	7.9.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#106, a last-minute work item to introduce a new 7MHz channel bandwidth for band n5 and n26 was approved due to the strong and urgent commercial demand [1]. The following table shows the core part objectives:

The core part of this WI includes: - Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective. - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective. NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

In this contribution, we discuss the release independence from Rel-15 and identify one potential backward compatibility issue in some scenarios, and propose two alternatives accordingly to handle this issue.

# 2 Discussion

It is true that there is no UE capability signalling issue since the IEs channelBWs-UL-v1590 and channelBWs-DL-v1590 have many unused bits to indicate the support of 7MHz channel bandwidth, however, for a scenario where a BS has: (1) SIB1 carrier is 7MHz; (2) Initial BWP size is 6MHz (larger than 5MHz but less than 7MHz), then a legacy UE cannot access to this BS because the UE cannot support a channel bandwidth which is no larger than 7MHz but no less than 6MHz. In this case, it will cause a compatibility issue.

Observation: A backward compatibility issue may occur when BS has 7MHz channel bandwidth and an initial BWP larger than 5MHz but less than 7MHz.

Unless the initial BWP size of a 7MHz channel bandwidth is restricted to no larger than 5MHz, then the issue can be resolved. Hence, there are two alternatives to handle release independence of 7MHz channel bandwidth:

- Alt.#1: Restrict the maximum initial BWP size to 5MHz, then 7MHz channel bandwidth can be release independent from Rel-15.

- Alt.#2: No restriction on initial BWP size, and 7MHz channel bandwidth can be release independent from Rel-19.

Proposal: RAN4 to consider the following two alternatives to handle the release independence of 7MHz channel bandwidth:

- Alt.#1: Restrict the maximum initial BWP size to 5MHz, then 7MHz channel bandwidth can be release independent from Rel-15.

- Alt.#2: No restriction on initial BWP size, and 7MHz channel bandwidth can be release independent from Rel-19.

# 3 Conclusion

In this contribution, we have the following observation and proposal for release independence of 7MHz channel bandwidth:

Observation: A backward compatibility issue may occur when BS has 7MHz channel bandwidth and an initial BWP larger than 5MHz but less than 7MHz.

Proposal: RAN4 to consider the following two alternatives to handle the release independence of 7MHz channel bandwidth:

- Alt.#1: Restrict the maximum initial BWP size to 5MHz, then 7MHz channel bandwidth can be release independent from Rel-15.

- Alt.#2: No restriction on initial BWP size, and 7MHz channel bandwidth can be release independent from Rel-19.

# Reference

[1] RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA