3GPP TSG RAN WG4 Meeting #114		R4-2501060

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Discussion on spectrum utilization and UE RF requirements for 7MHz channel bandwidth

Agenda Item:	7.9.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#106, a last-minute work item to introduce a new 7MHz channel bandwidth for band n5 and n26 was approved due to the strong and urgent commercial demand [1]. The following table shows the core part objectives:

The core part of this WI includes: - Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective. - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective. NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

In this contribution, we discuss the spectrum utilization for 7MHz channel bandwidth and propose 35 PRBs as the maximum transmission bandwidth configuration accordingly. Moreover, we also discuss the impacted UE RF requirements such as REFSENS, and BCS4/5 etc.

# 2 Discussion

## 2.1 Spectrum utilization

Currently 3GPP has specified these regular channel bandwidths (integer times of 5MHz) in addition to an exceptional 3MHz introduced in Rel-18 mainly for some special scenarios. And the corresponding spectrum utilization is specified as the following table [2] [3]:

And the corresponding spectrum utilization in percentage of channel bandwidth could be found as the following table:

As other channel bandwidths introduced later than Rel-15, e.g., 35MHz and 45MHz, a simple method for deriving spectrum utilization of a new channel bandwidth is so-called linear fitting. Obviously, there could be two different fittings:

- Linear fitting is done for spectrum utilization in terms of number of PRBs

- Linear fitting is done for spectrum utilization in terms of percentages

Since 7MHz channel bandwidth only includes SCS 15kHz, the fitting parameters could be seen in the following Table – 2.1-1:

Table – 2.1-1 Linear fitting parameters

| Parameters | PRB number | Percentage |
| --- | --- | --- |
| Slope | 5.4347 | 1.45E-03 |
| Interception | -2.2592 | 0.912 |

Accordingly, the spectrum utilization for 7MHz could be derived as 35.784 and 35.862 respectively, which means the number of PRBs for 7MHz channel bandwidth is 35 from both methods. And the minimum guard band size could be calculated in the same way as other regular channel bandwidth, i.e. GBChannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2. The Table 2.1-2 below shows the result as 342.5kHz for SCS 15kHz.

Table – 2.1-2 Minimum guard band for 7MHz channel bandwidth (kHz)

| SCS (kHz) | 7 |
| --- | --- |
|  | MHz |
| 15 | 342.5 |
| 30 | N/A |
| 60 | N/A |

Proposal 1: RAN4 to specify the maximum transmission bandwidth configuration of 7MHz channel bandwidth as 35 PRBs for SCS 15kHz.

Proposal 2: RAN4 to specify the minimum guard band of 7MHz channel bandwidth as 342.5kHz for SCS 15kHz.

## 2.2 UE RF requirements

For an FDD band, whether REFSENS can be scaled with transmission bandwidth configuration of different channel bandwidth depends on the carrier’s duplex gap and duplex distance. However, these have been reflected in the current REFSENS requirements for band n5 and n26, hence it is reasonable to derive REFSENS for the new channel bandwidth.

To derive a reasonable REFSENS value for 7MHz channel bandwidth with SCS 15kHz, the first task is to decide the corresponding spectrum utilization. And as we proposes 35 PRBs for 7MHz channel bandwidth with SCS 15kHz above, the following derivation is done according to this assumption.

Secondly, the scaling factor by referring to the spectrum utilization of 5MHz channel bandwidth can be illustrated as the following Table – 2.2-1.

Table – 2.2-1 Scaling factor with respect to the spectrum utilization @ 5MHz CBW

| SCS (kHz) | 3 | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) |
| 15 | -2.2 | 0.0 | 3.2 | 5.0 | 6.3 | 7.3 | 8.1 |
| 30 | N/A | 0.0 | 3.4 | 5.4 | 6.7 | 7.7 | 8.5 |
| 60 | N/A | N/A | 0.0 | 2.1 | 3.4 | 4.5 | 5.4 |

Then for band n5, the current REFSENS is specified as Table – 2.2-2.

Table – 2.2-2: REFSENS of band n5

| Operating Band | SCS kHz | 3 | 5 | 10 | 15 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |
| n5 | 15 |  | -98 | -94.8 | -93 | -86.8 | -84.8 |
|  | 30 |  |  | -95.1 | -93.1 | -88.6 | -84.9 |

It could be observed that up to 15MHz channel bandwidth, REFSENS is aligned with the corresponding scaling factor. Hence, the scaling factor could be applied to REFSENS for 7MHz, which is 10*log10(35/25) = 1.5 dB, therefore, REFSENS for 7MHz CBW with SCS 15kHz would be -98 + 1.5 = 96.5 dBm, i.e., the following Table – 2.2-3:

Table – 2.2-3 REFSENS for band n5 with 7MHz CBW

| Operating Band | SCS kHz | 7 |
| --- | --- | --- |
|  |  | MHz |
|  |  | (dBm) |
| n5 | 15 | -96.5 |
|  | 30 | N/A |

Observation 1: For band n5, a scaling factor of 1.5dB can be applied to REFSENS of 7MHz on top of that of 5MHz CBW.

Proposal 3: RAN4 to specify REFSENS as -96.5 dBm for 7MHz CBW with SCS 15kHz for band n5.

Similarly, the current REFSENS for band n26 is specified as the following Table – 2.2-4:

Table – 2.2-4 REFSENS for band n26

| Operating Band | SCS kHz | 3 | 5 | 10 | 15 | 20 | 25 | 30 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz |  |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| n26 | 15 | -99.7 | -97.5 | -94.5 | -92.7 | -87.6 | -84.5 | -81.7 |
|  | 30 |  |  | -94.8 | -92.7 | -87.7 | -84.6 | -81.8 |

And it could be observed that up to 15MHz REFSENS is 0.2dB worse than the corresponding scaling factor, hence the scaling factor for 7MHz CBW could be 1.5 – 0.2 = 1.3dB, which means the REFSENS for 7MHz can be specified as (-97.5 + 1.3) = -96.2 dBm, as shown in Table – 2.2-5:

Table – 2.2-5 REFSENS for band n26 with 7MHz CBW

| Operating Band | SCS kHz | 7 |
| --- | --- | --- |
|  |  | MHz |
|  |  | (dBm) |
| n26 | 15 | -96.2 |
|  | 30 | N/A |

Observation 2: For band n26, a scaling factor of 1.3dB can be applied to REFSENS of 7MHz on top of that of 5MHz CBW.

Proposal 4: RAN4 to specify REFSENS as -96.2 dBm for 7MHz CBW with SCS 15kHz for band n26.

# 3 Conclusion

In this contribution, we have the following observations and proposals on the spectrum utilization and UE RF requirements for 7MHz channel bandwidth:

Observation 1: For band n5, a scaling factor of 1.5dB can be applied to REFSENS of 7MHz on top of that of 5MHz CBW.

Observation 2: For band n26, a scaling factor of 1.3dB can be applied to REFSENS of 7MHz on top of that of 5MHz CBW.

Proposal 1: RAN4 to specify the maximum transmission bandwidth configuration of 7MHz channel bandwidth as 35 PRBs.

Proposal 2: RAN4 to specify the minimum guard band of 7MHz channel bandwidth as 342.5kHz for SCS 15kHz.

Proposal 3: RAN4 to specify REFSENS as -96.5 dBm for 7MHz CBW with SCS 15kHz for band n5.

Proposal 4: RAN4 to specify REFSENS as -96.2 dBm for 7MHz CBW with SCS 15kHz for band n26.

# Reference

[1] RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA

[2] TS 38.101-1 v18.8.0

[3] TS 38.104 v18.8.0