3GPP TSG RAN WG4 Meeting #114		R4-2501066

Athens, Greece, Feb 17th – 21st , 2025

Title: 	TP to TR 38.922 on BS parameters for 14.8 - 15.35GHz frequency range

Agenda Item:	7.2.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

For 14.8 – 15.35GHz frequency range, the contents for general parameters and BS parameters in the current TR are completed [1]. In this contribution, we provide the corresponding TP according to the third reply LS and the work-split as agreed before [2].

# 2 Text Proposal

<Start of Change>

## 6.2	General parameters

### 6.2.1	Duplex mode

There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range even though FDD is not precluded. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].

### 6.2.2	Channel Bandwidth

While 3GPP may specify the 100MHz, 200MHz as well as other channel bandwidths for the requirements in this frequency range, 200MHz is considered as a typical channel bandwidth.

### 6.2.3	Signal Bandwidth

The signal bandwidth for a 200MHz typical channel bandwidth signal is associated with both SCS and spectrum utilization, and can be calculated based on the spectrum utilization:

Signal bandwidth = NRB  x SCS x 12

with NRB: Number of Resource block for 200 MHz channel bandwidth and SCS to be specified when introducing new band(s) for this frequency range.

## 6.3	BS parameters

### 6.3.1	Transmitter characteristics

#### 6.3.1.1	Power dynamic range

#### There is no power control in downlink and fixed power per resource block is assumed during the study phase. Hence 0 dB power dynamic range was agreed for the LS reply [6].

#### 6.3.1.2	Spectral mask

Since non-AAS BS type is removed for the frequency range 14.8 – 15.35GHz, it is proposed that only Category A is applicable for AAS BS for this frequency range. The Category A limits for Wide Area BS are specified in Table 6.3.1.2-1 for AAS BS where ΔfOBUE is set to 100MHz.

Table 6.3.1.2-1 AAS BS Spectral mask (Operating band unwanted emissions limits) for 14800 - 15350 MHz operation (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 50MHz | 2dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz £ Df < min(100 MHz, Dfmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | -13 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

#### 6.3.1.3	ACLR

#### The BS ACLR is agreed to be 30 dB for this frequency range.

#### 6.3.1.4	Spurious emissions

For the general spurious emissions are applicable for the frequency range, it is proposed that only Category A is applicable for AAS BS for this frequency range. The Category A limits for Wide Area BS are specified in Table 6.3.1.4-1 for AAS BS where ΔfOBUE is set to 100MHz.

Table 6.3.1.4-1 AAS BS Spurious emissions for 14800 - 15350 MHz operation (Category A)

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

#### 6.3.1.5	Maximum output power

The maximum base station output power/ sector (e.i.r.p.) was provided in the antenna parameter table of the LS [6], as extracted in Table 6.3.1.5-1. It was agreed to be aligned with antenna characteristics.

Table 6.3.1.5-1: Maximum BS output power in 14800 to 15350 MHz

|  | Macro suburban | Macro urban | Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- |
| Base station output power/sector (e.i.r.p.) (dBm) | 84.3 | 84.3 | 74.3 | 40.0 |

#### 6.3.1.6	Average output power

#### It was agreed the average output power won’t be mentioned in the reply LS in [6].

### 6.3.2	Receiver characteristics

#### 6.3.2.1	Noise figure

The BS typical noise figure is listed in Table 6.3.2.1-1, and BS class definitions can be seen in [9], § 4.4.

Table 6.3.2.1-1: Noise figure

| BS class | Noise figure (dB) |
| --- | --- |
| Wide Area | 8 |
| Medium Range | 13 |
| Local Area | 16 |

#### 6.3.2.2	Sensitivity

BS OTA sensitivity requirement for this frequency range is calculated based on the NF and the antenna gain similar to 7 and 10GHz:

EISREFSENS=PkT+10*log10BW+NF+IM+SNR-G    (dBm)

Where:

-	BW is the configured bandwidth of the FRC,

-	NF is the noise figure,

-	IM is implementation margin not related to antenna array,

-	SNR is the required SNR to reach 95% throughput, and

#### -	G is the antenna gain including RF losses and 3dB off peak margin.

#### 6.3.2.3	Blocking response

The in-band blocking requirement should apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. It is agreed to adopt ΔfOOB = 100 MHz for AAS BS for this frequency range. The in-band blocking levels are to be specified further.

The out-of-band blocking applies from 30 MHz to 2nd harmonic of the upper frequency edge of the operating band, excluding the in-band blocking frequency range, but including the downlink frequency range in case of an FDD operating band. Requirements are defined assuming a receiver desensitization of 6 dB as illustrated in Table 6.3.2.3-1:

Table 6.3.2.3-1: OTA out-of-band blocking performance requirement for 14800 – 15350 MHz operation

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 | CW |
| 12750 to FUL,low – 100 | EISREFSENS + 6 dB | 0.1 | CW |
| FUL,high + 100 to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.1 | CW |

#### 6.3.2.4	ACS

It is agreed to specify 30 dB.

<End of Change>

# Reference

[1] RP-242945 TR 38.922 v1.0.0

[2] R4-2420385, “LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson