# 3GPP TSG-RAN WG4 Meeting #114                                R4-2501093

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.2

Source: 	CMCC

Title: 			 Discussion on CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on testability and interoperability issues for  CSI prediction.

- Discussion

Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

According to the revised WID [2], CSI prediction (one-sided model) is one of the objectives for WI, and RAN4 need to specify necessary RAN4 core requirements. In RAN4#110 meeting, it was agreed that thoughput should be the default metric for inference. In last meeting, it was agreed to use relative throughput as requirement metric for CSI prediction. As for what is the baseline that throughput would be compared to, it was agreed that reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook.

For testing conditions, it was agreed to use static conditions as baseline, FFS on whether to have non-static condition. Static condition is similar as we have for legacy test. Non-static condition is more suitable to verify the generalization and is benificial in reducing the number of tests. It is good if we could have tests for both static and non-static condition.

However, considering the limited timeline and there are so many issues to be solved, it seems impossible to have parallel discussion on both cases. Especially, non-static conditions are new for RAN4 which will take a long time to discuss the detail, for example, how the channel model and SNR settings are changed over the test. It is preferred to firstly focus on static condition, and non-static condition can be further discussed if time allowed.

Proposal 1: for CSI prediction, it is preferred to consider non-static condition, however considering the timeline and workload, non-static conditions can be discussed later if time allowed.

Inference performance is eventully performance, which is impacted not only by the AI/ML model performance but may also be impacted by other factors. While model monitoring targets to evaluate whether a model works well under a centain configuation/scenario. Inference performance is good does not mean monitoring performance is good. From this point of view, intermediate KPI, at least cosine similarity (SGCS), ranging within [0, 1], can be considered as test metrics for LCM.

Proposal 2: for CSI prediction, it is proposed to use intermediate KPI, e.g. SGCS, as requirements/tests metrics for LCM.

- Conclusion

This contribution provides discussion on CSI prediction. The proposals are:

Proposal 1: for CSI prediction, it is preferred to consider non-static condition, however considering the timeline and workload, non-static conditions can be discussed later if time allowed.

Proposal 2: for CSI prediction, it is proposed to use intermediate KPI, e.g. SGCS, as requirements/tests metrics for LCM.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface