# 3GPP TSG-RAN WG4 Meeting #114                                R4-2501094

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.1

Source: 	CMCC

Title: 			 Discussion on general aspects

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on general aspects.

- Discussion

Agreements in RAN4#112bis Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

For the target for AI/ML based beam management, our view is that AI/ML based performance requirement should not be worse than the legacy. According to the discussion in last meeting, some companies mentioned that the AI/ML based performance will be worse than the legacy, since the gain come from overhead reduction. From operator point of view, we do not think that this is a good way to go. If the predicted results are worse than the legacy measured results, it will have impact on the network deployment, which is not preferred.

Proposal 1: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement performance.

For CSI prediction, it was agreed that AI/ML based performance requirement should not be worse than the legacy, and FFS which legacy requirement/feature are relevant. According to RAN1 agreement duplicated as following, R18 eType II doppler codebook is used for evaluation of CSI prediction. And From RAN4 requirements point of view, PMI requirements are specified for codebook type of  'typeII-doppler-r18' . It is proposed that PMI requirements for codebook type of  'typeII-doppler-r18'  can be used as baseline.

RAN1 #116 meeting Agreement For Rel-19 study on CSI prediction, companies are encouraged to evaluate throughput performance by comparing performance with non-AI/ML based CSI prediction. - R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction. - Companies to report the assumption for N4, which could be 1, 2, 4, 8. Note: Non-AI/ML based CSI prediction (Benchmark 2) can include statistical model based CSI prediction (e.g., based on Kalman filter, Wiener filter, Auto-regression).

Observation 1: According to RAN1 agreement, R18 eType II doppler codebook is used for evaluation of CSI prediction. And From RAN4 requirements point of view, PMI requirements are specified for codebook type of  'typeII-doppler-r18' .

Proposal 2: for CSI prediction, it is proposed that AI/ML based performance requirement should not be worse than the legacy PMI requirements for 'typeII-doppler-r18'

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement performance.

Observation 1: According to RAN1 agreement, R18 eType II doppler codebook is used for evaluation of CSI prediction. And From RAN4 requirements point of view, PMI requirements are specified for codebook type of  'typeII-doppler-r18' .

Proposal 2: for CSI prediction, it is proposed that AI/ML based performance requirement should not be worse than the legacy PMI requirements for 'typeII-doppler-r18'

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-247647, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, Nokia, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface