# 3GPP TSG-RAN WG4 Meeting #114                                R4-2501096

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.20.1

Source: 	CMCC

Title: 			 Discussion on general aspects for study on AI/ML

Document for:	Discussion

- Introduction

In RAN #106 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are duplicated as following.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [2][3]. This contribution provides discussion on general aspects.

- Discussion

WF in RAN4 #112bis Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable

The reason to consider post deployment handling is that the change/update/modification of AI/ML model/functionality may be very frequent. It is questionable whether a device that has passed the conformance test with one version of AI/ML model/functionality can also pass the test after the change/update/modification. What’s more, it is also possible that a totally new model is loaded to the device after deployment, which means its performance are not tested and cannot be guaranteed.

GCF has procedure about how to deal with changes in certified products, it is feasible to handle the changes in already certified/deployed UEs, which can be considered as baseline to discuss the post deployment testing. We firstly provide information/background about how to deal with changes in certified products in GCF. As discussed in our RAN5 contribution[5], GCF-AP Section 7.7 covers the scenario of “how to deal with the changes in Product’s Hardware and/or Software Configuration”, which addresses the issue of “how to conduct the validation of a change in functionality in already deployed UEs” from a methodology perspective.

For the change that requires a modification to the information within the declaration, the certification bodies shall be notified in time, including two scenarios as described in GCF-AP[2] Section 7.7:

- the scenario of “do not add new functionality or features”, evidence of continued compliance with the applicable certification criteria shall be provided and recorded by the certification bodies.

- the scenario of “add new functionality or features”, evidence for compliance to the certification criteria applicable to the new functionality for feature shall be provided and recorded by the certification bodies.

There are a few cases tagged with “Updated” can be found in GCF website. For an updated device, some manufacturers may choose to make a new certification rather than updating the original. In GCF, there is no additional fee for doing a new certification or updating a certification. But Recognised Test Organisation (RTO) may still charge for doing a new certification or updating a certification.

The methodology of “how to deal with the changes in already deployed UEs in GCF” can be a starting point with necessary adaptions when discussing “how to conduct the validation of a change in functionality in already deployed UEs”under“Issue 1-2: Post deployment testing”. For the UE which already supports AI functionality/features when it is certified, and the certified AI-UE is subsequently modified with updated AI models, as long as the changes in the AI models do not add new functionality/features, this scenario is similar as: “do not add new functionality or features” described in GCF-AP[2] Section 7.7. Below two scenarios are similar as: “add new functionality or features” described in GCF-AP[2] Section 7.7.

- The UE does not support AI functionality/features when it is certified, and the certified UE is subsequently modified with addition of AI functionality/features

- The UE already supports AI functionality/features when it is certified, and the certified UE is subsequently modified with updated AI models which enable the certified UE with new functionality/features

Considering RAN5 is familiar with the existing certification/validation methodology/processes in the certification bodies (quite a few RAN5 delegates attend both RAN5 and GCF), it is proposed that RAN5 discuss methods to address how to conduct the validation of changes in functionality in already deployed UEs relevant to “Issue 1-2: Post deployment testing”.

Proposal 1: it is proposed that the issue of post deployment testing can be discussed in RAN5.

WF in RAN4 #112bis Issue 1-7: Number of tests - Proposals - Option1: Each specified AI/ML-enabled functionality shall have RAN4 requirements and pass at least one test for device conformance for this functionality. - Option 2: A minimum set of Associated IDs should be defined as mandatory (mandatorily applicable) for testing of each functionality. - Option 3: RAN4 does not need to define performance requirements that imply the change of Associated ID during the test. - Option 4: others

In previous meeting, the testing goals are updated as to verify whether the minimum performance of AI/ML functionality/feature can be achieved, but FFS on whether and how many different test configurations/parameters are used for tests.

In our view, the testing goals are high level principle, which are applicable to both inference test and generalization test. For the FFS part, it is necessary to consider different test configurations/parameters at least for generalization test. According to Rel-18 discussion, the goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations.

Proposal 2: it is proposed that different test configurations/parameters are used for tests.

As for how many different test configurations/parameters are used for tests, it can be further discussed based on RAN1/2 progress. Based on RAN1 evaluation, generalization is impacted by many factors. In high level, according to TR 38.843, deployment scenarios, UE distributions, carrier frequency,  TxRU mappings are considered for generalization of CSI compression. And CSI payload sizes,  Tx port numbers, are considered for scalability evaluations for CSI compression. For beam management, the performance is impacted by deployment scenarios, UE mobility, etc. Similar situation for positioning. It seems difficult to decide the scenarios and/or configurations used for generalization tests. On one hand, if we consider all the impacted factors into account, the number of tests may be huge. On the other hand, if the number of tests is limited to reduce the test burden, it is doutful whether generlization is verified well.

According to RAN1 discussion [5], for AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. It can be seen that UE will report the supported configurations  for a certain model/functionality. The scenarios and/or configurations for generalization can be decided based on the supported configuration reported by UE.

In last RAN2 meeting, a LS on applicable functionality reporting for beam management UE-sided model is aent to RAN1. One of the questions is what is the granularity of functionality. For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others. In our understanding, the others means more detailed scenarios and/or configurations, which can be used to decide the scenarios and/or configurations used for test.

Proposal 3: RAN4 requirements and tests are specified at least based on functionality, and the details can be further discussed pending on the RAN1/2 progress on granularity of functionality.

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: it is proposed that the issue of post deployment testing can be discussed in RAN5.

Proposal 2: it is proposed that different test configurations/parameters are used for tests.

Proposal 3: RAN4 requirements and tests are specified at least based on functionality, and the details can be further discussed pending on the RAN1/2 progress on granularity of functionality.

- Reference

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-247647, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, Nokia, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface