3GPP TSG-RAN WG4 Meeting #114	R4-2501110

# Athens, GR, 17th - 21st Feburary, 2025

Agenda Item:	5.33.1

Source:	MediaTek Inc.

Title:	Discussion on NTN NB-IoT UE in-band operation

Document for:	Approval

- Introduction

The NB-IoT NTN only supports stand-alone operation but is lack of in-band operation. There is strong NTN industry demand to enable NB-IoT NTN in-band operation with NR NTN firstly. Therefore, the addition of the optional feature in TS 36.102 is needed. In this contribution, the views on how to specify NTN NB-IoT UE in-band operation are provided.

- Discussion

2.1		NTN NB-IoT in-band operation

As for supporting NTN NB-IoT in-band operation, the anchor and non-anchor carriers are not well discussed.

Observation 1: Regarding introduction of NTN NB-IoT in-band operation, the mixed-mode operations about anchor and non-anchor carriers are not well discussed.

Proposal 1: Discuss possible options below and whether to capture the agreed option in CR or WF.

- Option 1: In Rel-18/current release, operational mode of anchor and non-anchor carriers shall be the same.

- Option 1: NOTE 6: In Rel-18/current release, operational mode of anchor and non-anchor carriers will be the same.

- Option 2: NOTE 6: Mixed mode operation specified in 36.300 [xx] is not supported.

- Option 3: Mixed mode operation specified in 36.300 [xx] is not supported.

Observation 2: As for the NB-IoT in-band operation within NTN, RRC signalling indicates guardband-r13 mode in operationModeInfo IE in MasterInformationBlock-NB is used as starting point. In addition, there is opinion that RRC signalling indicates inband-samePCI/inband-differentPCI in operationModeInfo IE in MasterInformationBlock-NB is not precluded for NB-IoT in-band operation.

Proposal 2: Discuss possible options below and whether to capture the proposed content in CR or WF.

- Option 1: For the NB-IoT in-band operation in NR NTN, RRC signalling indicates inband-samePCI/inband-differentPCI in operationModeInfo IE in MasterInformationBlock-NB is not precluded.

- Option 2: NOTE 2: For the NB-IoT in-band operation in NR NTN, RRC signalling indicates inband-samePCI/inband-differentPCI in operationModeInfo IE in MasterInformationBlock-NB is not precluded.

As for NTN NB-IoT UE and SAN guard-band operation, more discussion is needed.

Proposal 3: Regarding NTN guard-band operation, discuss possible option below.

- Option 1: The NB-IoT guard-band operation in NTN is up to UE and SAN implementation with no conformance testing requirements.

  - Option 1-1: Update the agreed content in TS 36.102 with CR.

  - Option 1-2: Capture the agreed content in WF.

Proposal 4: Regarding NB-IoT in-band operation, there is new power imbalance between different RBs or new power imbalance between RBs and Tones. Discuss possible options below and whether to capture the proposed content in CR or WF.

- Option 1: Note 5: No conformance testing requirements for NTN in-band operation with power imbalance.

- Option 2: Note 5: No UE RX RF/Demod requirements to be associated with NTN in-band operation power imbalance.

3 Conclusion

In this paper, our views on how to manage NTN NB-IoT UE in-band and guard-band operations are provided.

Observation 1: Regarding introduction of NTN NB-IoT in-band operation, the mixed-mode operations about anchor and non-anchor carriers are not well discussed.

Proposal 1: Discuss possible options below and whether to capture the agreed option in CR or WF.

- Option 1: In Rel-18/current release, operational mode of anchor and non-anchor carriers shall be the same.

- Option 1: NOTE 6: In Rel-18/current release, operational mode of anchor and non-anchor carriers will be the same.

- Option 2: NOTE 6: Mixed mode operation specified in 36.300 [xx] is not supported.

- Option 3: Mixed mode operation specified in 36.300 [xx] is not supported.

Observation 2: As for the NB-IoT in-band operation within NTN, RRC signalling indicates guardband-r13 mode in operationModeInfo IE in MasterInformationBlock-NB is used as starting point. In addition, there is opinion that RRC signalling indicates inband-samePCI/inband-differentPCI in operationModeInfo IE in MasterInformationBlock-NB is not precluded for NB-IoT in-band operation.

Proposal 2: Discuss possible options below and whether to capture the proposed content in CR or WF.

- Option 1: For the NB-IoT in-band operation in NR NTN, RRC signalling indicates inband-samePCI/inband-differentPCI in operationModeInfo IE in MasterInformationBlock-NB is not precluded.

- Option 2: NOTE 2: For the NB-IoT in-band operation in NR NTN, RRC signalling indicates inband-samePCI/inband-differentPCI in operationModeInfo IE in MasterInformationBlock-NB is not precluded.

Proposal 3: Regarding NTN guard-band operation, discuss possible option below.

- Option 1: The NB-IoT guard-band operation in NTN is up to UE and SAN implementation with no conformance testing requirements.

  - Option 1-1: Update the agreed content in TS 36.102 with CR.

  - Option 1-2: Capture the agreed content in WF.

Proposal 4: Regarding NB-IoT in-band operation, there is new power imbalance between different RBs or new power imbalance between RBs and Tones. Discuss possible options below and whether to capture the proposed content in CR or WF.

- Option 1: Note 5: No conformance testing requirements for NTN in-band operation with power imbalance.

- Option 2: Note 5: No UE RX RF/Demod requirements to be associated with NTN in-band operation power imbalance.

5 Reference

- R4-250111x, “(TEI18) CR to 36.102 NB-IoT NTN inband operation with NR NTN [NTNNBIoT_inbandNTNNR]”, MediaTek Inc.