# 3GPP TSG-RAN WG4 Meeting #114				                                               R4-2501111

# Athens, Greece, 17th – 21st Februray, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on Fast SCell activation for UE supporting Rel-18 eEMR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The fast SCell activation delay requirements for UE supporting Rel-18 eEMR are introduced in RRM phase 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1 : add a new chapter 8.3.2A to specify the case where unknown Scell supporting Rel-18 eEMR Change 2: For direct Scell actiation at Scell addition adding the unknown Scell supporting the Rel-18 eEMR Change 3: For deactivated PUCCH scell activation adding the unknown scell supporting the Rel-18 eEMR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-19 fast SCell activation delay requirements for UE supporting Rel-18 eEMR are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 8.3.2A, 8.3.4, 8.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Need to introduce Rel-19 specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change 1>

### 8.3.2A	SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

Requirements in this clause apply if the SCell is unknown, provided:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

The SCell is unknown if the corresponding known conditions specified in clause 8.3.2 are not met.

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference

resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting

resources as specified in TS 38.331 [2].

Tactivation_time is the SCell activation delay in millisecond, which is defined as:

If the SCell belongs to FR1, Tactivation_time is TFirstSSB_MAX + Trs + 5ms provided that the side condition Ês/Iot ≥ -2dB is fulfilled, if the following conditions are met:

-	During the period equal to 5s for FR1 before the reception of the SCell activation command:

-	the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3.

-	the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e., a valid CQI reporting)

-	If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission.

-	If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter.

-	the SSB measured remains detectable according to the IDLE/INACTIVE mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

If the SCell belongs to FR2, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, and if the following conditions are met:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3.

-	the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e. a valid CQI reporting)

-	If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission.

-	If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter.

-	the SSB measured remains detectable according to the IDLE/ INACTIVE mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

Then Tactivation_time is:

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX  + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	TFirstSSB_MAX + 15*TSMTC_MAX + max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

Where:

TFirstSSB_MAX, Trs, TSMTC_MAX,  TFineTiming, Tuncertainty_SP and TRRC_delay are specified in clause 8.3.2.

# <End of Change 1>

# <Start of Change 2>

### 8.3.4	Direct SCell Activation at SCell addition

The requirements in this clause apply for UE being configured in the RRC reconfiguration message, TS 38.331 [2], with one SCell for which the parameter sCellState is set to activated. If the RRC reconfiguration message for direct SCell activation also configures PSCell addition or PSCell change, the direct SCell activation delay may be longer than the requirements defined in this clause.

If the RRC reconfiguration message for direct SCell activation also configures TCI state information, the requirements in section 8.3.2 based on that TCI state activation command is received at the same time as SCell activation command shall apply.

The UE shall configure the SCell in activated state upon successful completion of the RRC reconfiguration procedure within the specified delay. The UE shall be capable to transmit valid CSI report and apply actions for the directly activated SCell no later than in slot n+NdirectNR slot length ,

where:

-	Slot n is the last slot overlapping with the PDSCH containing the RRC reconfiguration message,

-	Ndirect = TRRC_Process + T1 + Tactivation_time + TCSI_Reporting - 3ms for the cases specified in clause 8.3.2 that TCI state is not indicated within Tactivation_time; otherwise, Ndirect = TRRC_Process + T1 + THARQ + Tactivation_time + TCSI_Reporting

-	TRRC_Process: RRC procedure delay as specified in clause 11.2 of TS 36.331 [16] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the RRC procedure delay defined in clause 12 of TS 38.331 [2],

-	T1: Delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationComplete message,

Note:	T1 is UE implementation dependent.

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3],

If the SCell is known and belongs to FR1, TCSI_Reporting is specified in clause 8.3.2 and Tactivation_time is defined as:

-	TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms.

-	TFirstSSB_MAX + Trs + 5ms, if measurement period of the SCell being activated is larger than 2400ms.

where,

the measurement period in Table 9.2.5.2-1 applies if the target SCell was in an intra-frequency layer corresponding to an activated SCell;

the measurement period in Table 9.2.5.2-3 applies if the target SCell was in an intra-frequency layer corresponding to a deactivated SCell;

the measurement period in Table 9.3.5-1 applies if the target SCell was in an inter-frequency layer.

-	Otherwise,

Tactivation_time is specified in 8.3.2A provided the following conditions are met:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

- Otherwise:

Tactivation_time and TCSI_Reporting are specified in clause 8.3.2, where the following definitions of TFirstSSB and TFirstSSB_MAX shall override the existing ones:

-	TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRC_Process+T1NR slot length

-	TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + TRRCProcess+T1NR slot length

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

-	If a UE supports, reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, the reduced Tactivation_time  specified in clause 8.3.2 when UE supports these capabilities is applicable for Direct SCell activation at SCell addition also.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS38.321 [7] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

The SCell in FR1 is known provided the following conditions are met for the SCell:

-	During the last 5 seconds before the reception of the direct SCell configuration command:

-	the UE has sent a valid measurement report for the SCell being directly activated, and

-	the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3,

-	the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise, the SCell is unknown.

The SCell in FR2 is known provided it meets the corresponding conditions as defined in clause 8.3.2. Otherwise, the SCell is unknown.

The UE may be allowed to cause interruptions to serving cells on other component carriers during an interruption window, as specified in clause 8.2. The starting point of an interruption window on spCell or any activated SCell shall not occur before slot n+1, and shall not occur after slot n+1+TRRC_Process+T1+TXNR slot length, where NR slot length is with respect to the numerology of the SCell being activated, and TX is:

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes TFineTiming.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

Starting from the slot n+TRRC_Process + T1NR slot length until the UE has completed the direct SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for the SCell.

# <End of Change 2>

# <Start of Change 3>

### 8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell

The requirements in this clause shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated.

If the UE has a valid TA for transmitting on an SCell then the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3*Ttarget_PL-RS)+TCSI_Reporting_afterNR slot length,

Where:

-	A TA is considered to be valid provided that the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running.

-	THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tactivation_time is the SCell activation delay in millisecond.

If the SCell is unknown, Tactivation_time is specified in 8.3.2A provided the following conditions are met:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

Otherwise:

For the UE capable of l3-MeasUnknownSCellActivation, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the Tactivation_time is the SCell activation delay with L3 reporting in millisecond as specified in section 8.3.17 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP;

-	First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results;

Otherwise, Tactivation_time is the SCell activation delay in millisecond as specified in section 8.3.2 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal determined during PUCCH SCell activation.

-	TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource.

-	TCSI_processing is the UE processing time for CSI reporting.

-	TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), 3*Ttarget_PL-RS)

If the UE does not have a valid TA for transmitting on an SCell then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_timeNR slot lengthTHARQ+Tactivation_time+TCSI_ReportingNR slot length, and shall be capable to perform uplink actions related to the SCell activation command as specified in [7] for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length n+THARQ+Tdelay_PUCCH_SCell+TCSI_Reporting_PUCCH_SCellNR slot lengthand shall transmit valid CSI report for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tdelay_PUCCH_SCellNR slot length, where:

Tdelay_PUCCH_SCell = Tactivation_time + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), 3*Ttarget_PL-RS) + TCSI_reporting_after

Where:

-	Tactivation_time is the SCell activation delay in millisecond.

If the target Scell is unknown, Tactivation_time is specified in 8.3.2A provided the following conditions are met:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

Otherwise:

For the UE capable of l3-MeasUnknownSCellActivation, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the Tactivation_time is the SCell activation delay with L3 reporting in millisecond as specified in section 8.3.17 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP;

-	First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results;

Otherwise, Tactivation_time is the SCell activation delay in millisecond as specified in section 8.3.2 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (when applicable in FR2) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal determined during PUCCH SCell activation.

-	TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource.

-	TCSI_processing is the UE processing time for CSI reporting.

-	TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), 3*Ttarget_PL-RS)

-	T1 is the delay uncertainty in acquiring the first available PDCCH triggered PRACH occasion in the PUCCH SCell after the slot n+THARQ+Tactivation_timeNR slot length.

-	T1 is up to the summation of a delay uncertainty for reception of PDCCH order, SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion association period is defined in the Table 8.1-1 of TS 38.213

-	T2 is the delay from slot n+THARQ+Tactivation_time+T1NR slot lengthuntil UE has obtained a valid TA command for the target PUCCH Scell being activated. Slot n is the slot where the UE receives PUCCH SCell activation command.

-	T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, as specified in clause 4.2 in TS 38.213.

The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal:

-	The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for L3 measurement reporting

-	The target pathloss reference signal determination is based on the latest L3 RSRP measurement reporting

-	The target pathloss reference signal remains detectable during the PUCCH SCell activation period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

-	SNR of the associated SSB ≥-3dB

For the UE capable of l3-MeasUnknownSCellActivation, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 reporting after SCell activation command and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

-	The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for the L3 reporting after SCell activation command

-	The target pathloss reference signal determination is based on the L3 reporting after SCell activation command, if UE only sends L3 reporting before receiving TCI activation command, or

-	The target pathloss reference signal determination is based on either the latest L1-RSRP measurement reporting or the L3 reporting after SCell activation command, if UE reports both before receiving TCI activation command

-	The target pathloss reference signal remains detectable during the PUCCH SCell activation period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

-	SNR of the associated SSB ≥-3dB

Otherwise, tpathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

-	The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for L1-RSRP measurement reporting

-	The target pathloss reference signal determination is based on the latest L1-RSRP measurement reporting

-	The target pathloss reference signal remains detectable during the PUCCH SCell activation period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

-	SNR of the associated SSB ≥-3dB

Otherwise, the pathloss reference signal is unknown.

The above delay requirement shall apply provided that:

-	The target pathloss reference signal determined during PUCCH SCell activation is known otherwise longer activation time is expected if the pathloss reference signal is unknown; and

-	The RA on PUCCH SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and

-	No SRS carrier based switching or SRS antenna port switching occurs during the SCell activation procedure otherwise the PUCCH SCell activation delay can be extended.

The starting point and the end-point of an interruption window on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode is the same as the interruption in single SCell activation requirement in clause 8.3.2.

In addition to the interruption due to RF retuning during PUCCH SCell activation, if the UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH, and PRACH on PUCCH SCell and PUCCH/PUSCH/SRS on other active serving cell are fully or partially overlapping in time, the UE shall transmit PRACH on PUCCH SCell and is allowed to drop or cause interruption to SRS or PUCCH or PUSCH transmission on the SpCell or on any activated SCell. Otherwise, UE is not allowed to drop or cause any interruption to SRS or PUCCH or PUSCH transmission on SpCell or on any activated SCell.

For unknown PUCCH SCell activation in FR2, the requirement only applies when UE supports CSI reporting cross PUCCH group capability, and UE is configured with CSI reporting via SpCell. For unknown PUCCH SCell activation in FR1, the requirement only applies when UE supports CSI reporting cross PUCCH group capability, and UE is configured with CSI reporting via SpCell, if ‘ssb-PositionInBurst’ indicates multiple SSBs but TCI state indication is not provided in same MAC PDU with SCell activation.

The requirement for unknown PUCCH SCell applies provided that the PDCCH order (when applicable) and the activation commands for TCI, UL spatial relation and PL-RS (when applicable) are based on the latest valid L1-RSRP reporting or the L3 reporting after SCell activation command via Primary PUCCH group.

# <End of Change 3>