3GPP TSG-RAN WG4 Meeting #114 	   R4-2501115

# Athens, Greece, 17th – 21st , Feburary 2025

Agenda Item:	7.23.3.2

Source: 	Ericsson

Title:	Discussion on RRM impacts for SBFD operations (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#113 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

## 2.1 Other RRM core requirements for SBFD operation

### Impact on SSB based measurements

#### Issue 1-2-4: Requirements for scheduling restriction #### Agreements (from Wed AH session) #### For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction. #### Further discuss the impact to inter-frequency SSB measurement without gap case.

From legacy TDD since Rel-15, the DL/UL transition time is specified TS 38.211 clause 4.3.2.

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than Rx-Txc after the end of the last received downlink symbol in the same cell where Rx-Tx is given by Table 4.3.2-3. A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than Tx-Rxc after the end of the last transmitted uplink symbol in the same cell where Tx-Rx is given by Table 4.3.2-3.

For the Rel-19 SBFD aware UE, this legacy requirement shall still be valid since the SBFD aware UE is not capable of full-duplex. There is no need in general to specify scheduling restriction due to the TX/RX or RX/TX switching.

For the case right after SSB measurement, the existing RAN1 agreement for collision case

- Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs

Our interpretation of the priority rule agreed on RAN1 is based on the exact OFDM symbols that contains SSB. Within one DL slot the typical configuration of the SSB is starting from symbol#2 and the symbol#8 span over 4 OFDM symbols. In this case the symbols that not covered by the SSB within 1 slot are only 2 OFDM symbols. The scheduling restriction on these 2 symbols is not necessary. Also considering there will be switching gaps being introduced from gNB point of view based on the SBFD and non-SBFD slot, we don’t think the network will schedule within these symbols.

- Proposal 1: Scheduling restriction on symbol level right after the SSB measurement within the SBFD slot is not necessary.

### Impact on CSI-RS based measurements

#### Issue 1-2-3: Requirements for CSI-RS based L3 measurement Agreements (from online session) Companies to check the existing requirement and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed.

The CSI-RS L3 measurement will be based on the network configuration for the SBFD aware UE in regarding the neighbor cell SBFD related configuration. This will have RAN2 specification impact since this type of configuration is not exist currently.

Currently there is some RAN2 discussion in regarding the RLM/BFD measurement, however the L3 mobility measurement have not been started or triggered yet. Our view is to wait for whether RAN2 will introduce new configuration for neighbor cell SBFD CSI-RS configuration.

- Proposal 2: Wait for RAN2 to conclude on whether there will be L3 neighbour cell SBFD CSI-RS configuration.

RAN4#113 agreements were reached for CSI-RS:

- For CSI-RS puncturing in frequency domain in SBFD symbols, - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs. - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs. - FFS for the DUD configuration. - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions. - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold. - FFS the measurement behavior for DUD configuration. - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement.

For the DU configuration within the SBFD symbols, the continuous CSI-RS resource can be guaranteed while in the DUD configuration, this CSI-RS resource continuity in the frequency domain cannot be guaranteed due to there is an UL subband within the 2DL subband.

Also, when the CSI-RS resources being configured split across two DL subbands, it is important for the network to know where this measurement is being performed within the frequency location.

For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

When CSI-RS being configured across two DL subbands, the RAN1 agreement in the processing time is up to the UE capability. For the capable UE to processing the CSI-RS as legacy, the processing time will be the same as legacy which means even though the CSI-RS is not continuous, the UE still using same processing time to combine the results. However, for the not capable UE, the processing time will be impacted.

Assuming the measurement sample is the same as the legacy, while one type of UE will introduce measurement delay due to the processing time difference, we don’t see a unified requirement can be used for different capability UEs.

- Proposal 3: For DUD configuration, the measurement requirement needs to consider the UE capability of the CSI-RS processing time and separate requirements needs to be specified.

For the DUD configuration where the CSI-RS resource is at least 48 PRBs but it is split across the two subbands, it is hard to directly draw conclusion for the requirement impact. When the CSI-RS bandwidth being reduced for one subband, the measurement accuracy is for sure will be negatively impacted. On the other side, within one SBFD symbol, the total number of the PRB will be sufficient.

Whether the measurement requirement will be impacted will be dependent on how UE implement the RSRP based measurement RLM/BFD/CBD/L1-RSRP.

The impact from the CSI-RS not being continuous have different impact for different type of UE implementation. For fair and accurate requirement for the CSI-RS measurement, we think some simple simulation comparison to illuminate the UE implementation impact is needed.

- Proposal 4: Simulation evaluation is needed for the CSI-RS resource within SBFD symbol is at least 48 PRBs but it is split across the two sub bands.

Another factor potentially will impact the measurement requirement is how the CSI-RS resources set and report being configured.

One concern raised from RAN4 113 meeting is regarding the SBFD symbol and non-SBFD symbol configuration.

Based on current RAN1 agreement for the CSI-RS reporting, our interpretation is that at least for reporting, the SBFD symbols and non-SBFD symbols will be separately reported.

For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Regarding the CSI-RS resource set configuration, RAN4 has already sent an LS to RAN1 to clarify whether the RAN1 agreement applicable for the measurement.

- Proposal 5: Wait for RAN1 reply to conclude the impact on CSI-RS measurement on SBFD and non-SBFD symbols.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: Scheduling restriction on symbol level right after the SSB measurement within the SBFD slot is not necessary.

	Proposal 2: Wait for RAN2 to conclude on whether there will be L3 neighbour cell SBFD CSI-RS configuration.

	Proposal 3: For DUD configuration, the measurement requirement needs to consider the UE capability of the CSI-RS processing time and separate requirements needs to be specified.

	Proposal 4: Simulation evaluation is needed for the CSI-RS resource within SBFD symbol is at least 48 PRBs but it is split across the two sub bands.

	Proposal 5: Wait for RAN1 reply to conclude the impact on CSI-RS measurement on SBFD and non-SBFD symbols.

# References

- R4-2420097 “WF on SBFD RRM”, Huawei, November ,2024

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024