3GPP TSG RAN WG4 Meeting #114		 R4-2501118

# Athens, Greece, February 17 – 21, 2025

Source:	EchoStar, DISH Network

Title:	Way Forward on n252/band 252 co-existence

Agenda item:			6.18.2

Document for:	Approval

# 1. Introduction

In the last RAN4#113 meeting, the following tentative way forward was agreed for UE-to-UE coexistence between n252 and n2/n25.

Issue 1-1-3: UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL - Tentative Agreement: - Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2 - FFS: -30 dBm/MHz for relaxed value. - FFS: This emission level applies and the associated A-MPR is allowed whether or not the NS value is signalled. Also, the protection level should a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.” - FFS: [-40] dBm/MHz for the stricter value based on current b23 specification - The requirement is applicable in every country where b25/n25/b2/n2 and n252 are applicable - Specify asymmetric channel bandwidths with 5, 10 and [15] MHz UL CHBW for band n252.

In this contribution, we propose a way forward to address the open issues in the last way forward.

# 2. Proposed way forward and necessary changes

## 2.1 Open issue to be addressed

The discussion on UE-to-UE coexistence requirements for n252 has been based on the good faith from all involved operators to ensure that the standards mechanisms will allow an effective satellite operation in n252 (B252) and at the same time provide a sufficient protection for adjacent n2/n25. With the understanding of the limitation of satellite link budget and that interference probability is likely small, RAN4#112bis agreed to define a relaxed emission limit(s) from n252 to n2/n25 [1]. To ensure sufficient protection for n2/n25 and also to allow deployment flexibility between neighbouring operators, two different emission limits (i.e., -30dBm/MHz and -40dBm/MHz), each associated with NS value and AMPR, were proposed during discussion in RAN#113 [2].

From the last meeting, the main outstanding issue is how to ensure the applicability of the relaxed (-30dBm/MHz) requirement.

As written as FFS in the tentative agreement in RAN4#113 [1], T-Mobile USA proposed to specify that the emission requirement and its associated AMPR apply regardless of whether the NS is signalled. This means that -30dBm requirement is mandated, i.e., all n252 UE will suffer AMPR in all service region of n252 operation regardless of whether or not n2/n25 deployment exist. Considering that none of the US operators has 100% geographic coverage of US and that n252 can be operated in Canada, US and Mexico, mandating an emission requirement and its associated AMPR in the spec would be damaging for n252 operation inside and outside US. Satellite operator shall not be required to meet more stringent emission limit than regulation in geographic areas where there is no n2/n25 terrestrial coverage.

Observation 1:   	Satellite operators shall not be required to meet the more stringent emission limit than regulation in geographic areas where there is no n2/n25 terrestrial coverage.

In US, the FCC can enforce protection for any eligible bands/services when harmful interference is observed. Base on this, we believe that coordination between neighbouring operators can effectively address the issues to ensure that the emission limit will not be exceeded. For example, n2/n25 operator could provide n2/n25 terrestrial coverage map to n252 operator, so that n252 operator can broadcast the relevant NS in those areas. As a way forward, we propose for the specification outline the need for coordination among neighbouring operators to ensure emission requirement to be met, by including the following note: “n252 operator will ensure -30dBm level is not exceeded in the satellite cell overlapping with n2/n25 terrestrial based on their coverage area map”.

Proposal 1: 	Include the following note for -30dBm/MHz requirement: “n252 operator will ensure -30dBm level is not exceeded in the satellite cell overlapping with n2/n25 terrestrial based on their coverage area map”

Proposal 2: 	 RAN4 to agree the following proposed way forward.

- Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2 - -30 dBm/MHz for relaxed value. - Include the following note: n252 operator will ensure -30dBm level is not exceeded in the satellite cell overlapping with n2/n25 terrestrial based on their coverage area map. - Include a note equivalent to Note 15 in 38.101-1 which says: “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.” - -40 dBm/MHz for the stricter value based on current b23 specification - Include a note equivalent to Note 15 in 38.101-1 which says: “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.” - Specify asymmetric channel bandwidths with 5, 10 and 15 MHz UL CHBW for band n252.

## 2.2 Necessary changes in TS 38.101-5

CR for 38.101-5:

=====

### 5.3.6	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n254 | 5 | 10,15 | 0 |
|  | 10 | 15 | 0 |
| n252 | 5 | 10.15.20 | 0 |
|  | 10 | 15,20 |  |
|  | 15 | 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

=====

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

…

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 (NOTE 3) | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.3 | n254 | 5, 10, 15 |  | Clause 6.2.3.2 |
| NS_04N | 6.5.2.3.1 6.5.3.3.4 | n254 | 5 |  | Clause 6.2.3.3 |
| NS_05N | 6.5.2.3.2 6.5.3.3.4 | n254 | 5, 10, 15 |  | Clause 6.2.3.4 |
| NS_XX | 6.5.3.3.X | n252 | 5, 10, 15, 20 |  | TBC |
| NS_YY | 6.5.3.3.Y | n252 | 5, 10, 15, 20 |  | TBC |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. |  |  |  |  |  |

=====

#### 6.5.3.3	Additional spurious emissions

…

##### 6.5.3.3.X	Requirement for network signalling value "NS_XX"

When "NS_XX" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-X. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-X: Additional requirements for "NS_XX"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1930 ≤ f <1995 | -30 | 1MHz | XX |
| Note XX: n252 operator will ensure -30dBm level is not exceeded in the satellite cell overlapping with n2/n25 terrestrial based on their coverage area map. |  |  |  |

##### 6.5.3.3.Y	Requirement for network signalling value "NS_YY"

When "NS_YY" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-Y. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-Y: Additional requirements for "NS_YY"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1930 ≤ f < 1995 | -40 | 1MHz |  |

# 3. Summary and proposal

On the matter of mandating a certain emission requirement, the following is observed:

Observation 1:   	Satellite operators shall not be required to meet the more stringent emission limit than regulation in geographic areas where there is no n2/n25 terrestrial coverage.

To progress the discussion on UE-to-UE coexistence for n252, the following are proposed:

Proposal 1: 	Include the following note for -30dBm/MHz requirement: “n252 operator will ensure -30dBm level is not exceeded in the satellite cell overlapping with n2/n25 terrestrial based on their coverage area map”

Proposal 2: 	RAN4 to agree the following proposed way forward:

- Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2 - -30 dBm/MHz for relaxed value. - Include the following note: n252 operator will ensure -30dBm level is not exceeded in the satellite cell overlapping with n2/n25 terrestrial based on their coverage area map. - Include a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.” - -40 dBm/MHz for the stricter value based on current b23 specification - Include a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.” - Specify asymmetric channel bandwidths with 5, 10 and 15 MHz UL CHBW for band n252.

The necessary changes to 38.105 is also provided in section 2.2.

# Reference

- R4-2414287, “WF on Rel-19 FR1 NTN bands”, Moderator (Inmarsat), RAN4#112bis, Hefei, China, 14th – 18th October 2024

- R4-2420465, “WF on requirements for NR IoT NTN bands”, Moderator (Inmarsat), Orlando, USA, November 18th – 22nd, 2024