# 3GPP TSG-RAN WG4 Meeting #114	R4-2501125

Athens, GR, 17th – 21st February, 2025

Title: 	On optimizing CSSF for FR2-1 L3 measurement delay

Source: 	OPPO

Agenda item:	7.17.2.2

Document for:	Discussion

Introduction

In last meeting, a WF[1] was approved on FR2-1 SSB based L3 measurement delay reduction. No progress was achieved on CSSF optimization.

In this contribution, we will provide discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSFoutside_gap.

Discussion

Issue 2-4-1: Tentative solution 1: measure one serving CC per band [Wayforwad]: FFS the following solution: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation - FFS whether indication is per-UE or per-band - Other options are not precluded. - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs [Wayforward]: FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1: - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement. - FFS whether network indication is needed - Details is FFS - FFS on whether and how PCC and PSCC resource sharing is supported Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios. Issue 2-4-4: Other tentative solutions Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

In last meeting, there are three solutions on the table. Firstly, we think 2 searchers are basic assumption for CSSF definition in current spec. For some specific cases, UE may need one more searcher, like for supporting per FR gap configuration. Even if RAN4 only consider the enhancement based on 2 searchers for CSSF as same as previous release, there still exists room by optimizing CSSF for L3 measurement delay reduction, like solution 1 and 2. Thus, in general, RAN4 can continue the 2 searchers assumption as baseline for discussion.

Proposal 1: RAN4 to support solution 1 and/or solution 2, other than 3 searchers in R19 CSSF optimization.

For solution 1 and 2, they are not conflicted with each other. Take SA as example, the current rule of CSSF can be summarized as follows (just focus on SSB and without considering CSI-RS measurements):

- The 1st searcher is dedicated to PCC. The 2nd searcher is shared among the rest CCs.

- The 2nd searcher is dedicated to PSCC for 50% of the time, and for the other 50%, the searcher is shared with the rest of the SCCs.

- Among the SCCs, the SCC where neighbor cell measurement is also required occupies 50% of the time, which the rest 50% equally shared with the rest of the SCCs.

One debate on solution 1 is how to determine the selected SCC in the band. Some network vendor think network should indicate UE to enable this CSSF enhancement feature and also indicate the dedicated SCC(s) as the one(s) to be selected for measurement in one band. The reason behind this is that some SCCs needs to be prioritized due to operator’s demand.

But it is not very clear to us because usually the measurement configuration can already reflect the measurement purposes and cared carriers from network side. If some SCC is configured in one MO, it should be taken care of by UE during the measurement and report, and the network can reconfigure and adapt the measurement configuration according the measurement reports and real-time demand. Otherwise, the SCC not in MO configuration can be taken as low priority or even gotten rid of in the measurements. Thus, we think such network indication of prioritization is not needed. UE can select one suitable SCC based on both network configuration and real-time channel conditions, which leave some flexibility to UE implementation. We think the SCC decided by UE can be more accurate than that indicated by the network, and such carrier can represent the quality of carriers in the same band.

Proposal 2:  For solution 1, the selected SCC is up to both network’s legacy measurement configuration and UE’s implementation. NOT to introduce additional indication to determine the CC(s) to be measured.

For solution 2, one question is raised whether solution 2 is needed if solution 1 is agreed. We think more searchers and higher searcher occupancy ratio of SCC measurement can help to speed up the SCell measurement. Somehow the solution is one specific case to improve the occupancy ratio of the selected SCC in one band. It is more reasonable to consider this issue per band or per FR.

However, it may be hard to indicate the prioritized SCC across all SCCs without considering the bands. If RAN4 agreed to support solution 1, we think solution 2 can be somehow covered.

Proposal 3: If RAN4 agreed to support solution 1, solution 2 can be down-prioritized.

Conclusions

In this paper we provided our views on CSSF optimization.

Proposal 1: RAN4 to support solution 1 and/or solution 2, other than 3 searchers in R19 CSSF optimization.

Proposal 2:  For solution 1, the selected SCC is up to both network’s legacy measurement configuration and UE’s implementation. NOT to introduce additional indication to determine the CC(s) to be measured.

Proposal 3: If RAN4 agreed to support solution 1, solution 2 can be down-prioritized.

Reference

- R4-2415656, WF for NR_RRM_Ph5_Part1, Apple