# 3GPP TSG-RAN WG4 Meeting #114	R4-2501128

Athens, GR, 17th – 21st February, 2025

Title: 	On event triggered L1 measurement reporting

Source: 	OPPO

Agenda item:	7.27.2.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility[1].

- Issue 2-1: RRM scope of Event triggered L1 measurement reporting <Agreement> - Confirm that CSI-RS based event triggered L1 reporting is included in RAN4 scope of this work item. - Issue 2-2: starting point in event triggered L1 report requirements <Agreement in RAN4#112bis> - Use the following as starting point. Wording can be discussed in CR. - Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting. - Issue 2-3: ending point in event triggered L1 report requirements <Agreement> - Ending point in event triggered L1 report requirements is defined as the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface. - Issue 2-4: periodic report and single report <Agreement> - RAN4 requirements shall cover both Event triggered periodic MR and Event triggered MR. - Issue 2-14: where to capture event triggered L1 report requirement <Agreement> - The event triggered reporting for SSB based L1 measurement requirements can be defined in clause 9.14.3 and clause 9.15.3, respectively for intra-frequency and inter-frequency. - FFS on event triggered reporting for CSI-RS based L1 measurement.

In this contribution, we provide our views on measurements related enhancements for LTM, including event trigger L1 measurement reporting.

Discussion

Event triggered L1 measurement reporting

RAN4 agreed in last meeting that event triggered L1 reporting starts from the time when a condition exists at the UE which will trigger the reporting, and ends at the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface. RAN4 requirements shall cover both Event triggered periodic MR and Event triggered MR.

- Issue 2-5: Event triggered reporting delay for L1 reporting <Agreement> - Agree on the following high level proposal: - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells]. - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0. Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. - Issue 2-15: Impact of UL resource waiting time. <Candidate options> - Option 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. (Xiaomi) - It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore. - Option 2: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. (CMCC, Apple, vivo) - Option 3: RAN4 to discuss components and criteria for defining TTime to first UL channel (E///) - Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel. <Recommended WF> - Continue discussion.

We would like to confirm that the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period on the cells corresponding to the event. Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. About the open issue whether to reuse L1-RSRP measurement period from the existing spec with Treport = 0, we do not prefer to remove the lower bound of measurement period, and Treport should be kept.

For event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

Proposal 1: The event triggered reporting delay for L1 reporting shall be no larger than the L1-RSRP measurement period on the cells corresponding to the event.

Proposal 2:  When reusing L1-RSRP measurement period, keep Treport  as low bound.

Proposal 3: The L1 reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

- Issue 2-7: whether to introduce/update requirements for reporting criteria per measurement category <Candidate options> - Option 1: Yes (CATT, MTK, ZTE, Apple, OPPO) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC, Ericsson) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) <Recommended WF> - Continue discussion. - Issue 2-8: whether to define a separate capability for supported number of events triggered L1 measurement reporting <Candidate options> - Option 1: Yes (MTK, Samsung, CMCC) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC) - Option 2a: RAN4 to wait for RAN2 progress on the TTT handling before discussing the UE capability of number of events to support. (E///) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) <Recommended WF> - Continue discussion.

We support to introduce/update requirements for reporting criteria per measurement category. Meanwhile, the requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting. We support to keep the same value of Ecat for intra-frequency and inter-frequency measurement category, and update the note for each of events description, as the table below for example:

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |

Proposal 4: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: The event triggered reporting delay for L1 reporting shall be no larger than the L1-RSRP measurement period on the cells corresponding to the event.

Proposal 2:  When reusing L1-RSRP measurement period, keep Treport  as low bound.

Proposal 3: The L1 reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

Proposal 4: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat.

Reference

- R4-2420101, WF on NR mobility enhancements Phase 4_part1, Apple