# 3GPP TSG-RAN WG4 Meeting #114	R4-2501130

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.29.4.1

Source: 	OPPO

Title: 	On RRM requirements of (e)RedCap UEs with NR FR1-NTN

Document for:	Approval

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1].

In this paper, we will provide our views on support of (e)RedCap UEs with NR FR1-NTN.

Discussion on support of (e)RedCap UE with NTN

- For SDT requirements in RRC_INACTIVE state, - Proposal 1 (CATT, QC, OPPO, HW): RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements. - Proposal 1a (CATT): TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - Proposal 1b (LG): TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission. - Proposal 1c (CMCC): For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Proposal 1d (vivo): For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable) - Proposal 2 (ZTE): CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be: Measurement; FR1. RSRP1; (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)). RSRP2; (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2. Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment..

For SDT requirements, regarding TA validation based on the RSRP change criterion, we think it can be removed for Redcap NTN due to the un-reliability of RSRP evaluation, and the requirements may refer to the PUR requirements for IoT for NTN.

Proposal 1: TA validation based on the RSRP change criterion is not supported for Redcap NTN UE.

- For MDT requirements in RRC_IDLE and RRC_INACTIVE state - Proposal 1 (CATT, Samsung, QC, OPPO, Ericsson, vivo, HW, MTK): - Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN as the same situation for (e)RedCap UE in TN system. - Proposal 1a (CATT): Deprioritize to define. - Proposal 2 (CMCC): - Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap over NTN, if the related features are supported in RAN1/2 spec. - Proposal 3 (ZTE): - RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

For MDT requirements, since no RAN4 MDT requirement for RedCap TN UE, we prefer not to define MDT requirements for Redcap NTN UE considering the similar UE battery, storage and computing capability.

Proposal 2: Not to define MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap NTN UE.

RAN4 agreed on a list of requirements defined for both NTN and (e)RedCap UE requirements, only defined for NTN, and defined for (e)Redcap and not been defined for FR1-NTN, which are the most fundamental ones for the basic RRM related procedures.

- Sub-topic 2-4 Measurement Procedure for 1Rx/2Rx (e)RedCap UEs with FR1-NTN - Issue 2-4-1: L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, HW): RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level. - Depend on the supported UE capabilities (e.g., multiple SMTC),, the existing NR NTN requirements may need be revised. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, ZTE, HW): The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter. - Not to extend the lower boundary. - Proposal 1a (QC): Extended delays for PSS/SSS detection. - Proposal 1b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples. - Proposal 2 (CATT, [Samsung?], CMCC, HW): The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter. - Extend the lower boundary from ‘120 ms’ to ‘160ms’. - Proposal 2a (QC): Extended delays for SSB index identification (PBCH decoding). - Proposal 2b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples. - Proposal 3 (CATT, CMCC, HW): The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples for T SSB_measurement_period_inter. - Proposal 4 (CATT, CMCC, ZTE, HW): The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same. - Proposal 5 (CATT, CMCC, HW): The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

RAN4 has agreed to define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE with FR1-NTN bands. For 2Rx (e)RedCap UEs with FR1-NTN, RAN4 reuses the existing requirements for NTN as a baseline. For 1RX (e)RedCap UE, we can also relax measurement requirements including extending the measurement delays with more measurement samples and relaxing the measurement accuracy.

In our view, we support option 1b and 2b that for 1Rx (e)RedCap UEs with FR1-NTN bands, the time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Proposal 3: For 1RX (e)RedCap UE with FR1-NTN bands, support to relax measurement requirements by extending the measurement delays with more measurement samples, including time periods for PSS/SSS detection and for SSB index identification.

- Issue 2-5-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, ZTE): RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Xiaomi, Samsung, [QC?], CMCC, ZTE, [OPPO?], HW): RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB.

Similarly, we support to relax the measurement accuracy for 1Rx Redcap NTN UE than 2 RX Redcap UE. The same relaxation defined in TN can be reused, e.g., SS-RSRP, SS-RSRQ, SS-SINR relaxed by 1dB, and L1-RSRP relaxed by 3dB.

Proposal 4: RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs, and the same relaxation defined in TN can be reused.

- Issue 1-2-4: Multiple SMTCs <Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

The most of  NTN features should be supported in RedCap + NTN requirements. But as companies pointed out, one exception is the support of multiple SMTCs. So far, NTN is mandatorily required to support 2 SMTCs per MO, and supporting more is an optional capability. When the two SMTCs do not overlap, there is no scaling in measurement period, which means UE has to support parallel measurement for the two SMTCs. This may be a bit challenging for (e)RedCap UE with reduced complexity. We also share the similar concern on support of multiple SMTCs parallelly, and prefer not to support.

Proposal 5: RAN4 to not consider the support of measurements with multiple SMTCs in parallel for (e)RedCap UE with FR1-NTN bands.

Conclusions

In this paper we provided our views on RRM requirements for support of (e)RedCap UEs with NR FR1-NTN.

Proposal 1: TA validation based on the RSRP change criterion is not supported for Redcap NTN UE.

Proposal 2: Not to define MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap NTN UE.

Proposal 3: For 1RX (e)RedCap UE with FR1-NTN bands, support to relax measurement requirements by extending the measurement delays with more measurement samples, including time periods for PSS/SSS detection and for SSB index identification.

Proposal 4: RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs, and the same relaxation defined in TN can be reused.

Proposal 5: RAN4 to not consider the support of measurements with multiple SMTCs in parallel for (e)RedCap UE with FR1-NTN bands.

Reference

- R4-2420104, WF on RRM requirements for Rel-19 NR NTN phase3, CATT