# 3GPP TSG-RAN WG4 Meeting #114	R4-2501139

# Athens, Greece, 17th – 21st February, 2025

Agenda item:	5.12.1

Source:	OPPO

Title:	Discussion on the remaining issues for further MG enhancements

Document for:	Approval

- Introduction

In last meeting, RAN4 approved one WF [1] to capture the remaining open issues of Rel-18 gap enhancement. In this contribution, we will discuss the remaining issues and give our proposal.

- Discussion

In previous meeting, RAN4 has the following agreements on the deactivated SCell measurements:

< Agreement >: Agreement from RAN4 meeting#111: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case. Agreement from RAN4 meeting#112: - When UE switches measurement between NCSG and Type 2 MG, no additional processing delay is expected.

With the above agreements, there are still the following two scenarios to be clarified.

Issue 1-1-1: When the UE is configured with Concurrent gaps with NCSG, whether all deactivated SCells are measured within NCSG, regardless the gap association? - Further discuss in the next meeting. Issue 1-1-2: When the measurement of the measurement object is related to a deactivated SCell, and after this SCell is activated, and when UE is configured with the concurrent measurement gap and NCSG, what is the expected UE behaviour? - Further discuss in the next meeting

Based on the existing requirements, deactivated SCell can be measured within NCSG or outside gap (including MG and NCSG). When the UE is configured with concurrent gaps with NCSG, we understand all the deactivated SCells can be measured within NCSG if the SMTCs to be measured are fully or partially overlapped with NCSG regardless the gap association. Otherwise, the deactivated SCells are measured outside gaps. But after this SCell is activated, the measurements on this SCC could be gap based, NCSG based or outside gap, we understand UE should measure the MO following gap association.

Proposal 1: When the UE is configured with concurrent gaps with NCSG, all the deactivated SCells can be measured within NCSG if the SMTCs to be measured are fully or partially overlapped with NCSG. Otherwise, the deactivated SCells are measured outside gaps.

Proposal 2: When UE is configured with the concurrent measurement gap and NCSG and the measurement of the measurement object is related to a deactivated SCell, after this SCell is activated, UE should measure the MO following gap association.

The other remaining issue is the misalignment between DRX-on duration and SMTC for NFG measurements.

Issue 2-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements? - Background: - Agreement from RAN4#112 meeting: - Interruptions are not allowed during DRX ON duration - Exclude the time extended due to drx-inactivityTimer - if there is no SMTC occasion within a time period starting [4ms] before the starting point of the DRX ON duration and ending [4ms] after the ending point of the DRX ON duration.

We have seen several interval discussions in the gap related features, e.g., the collision between gaps and the collision between SMTC and gap. After evaluation and discussion, 4ms is used to define the collision and we do not see the problem. For this interruption requirements for NFG, we think we should stick to the previous agreement with 4ms interval between SMTC and DRX-on duration which considers the need for UE processing and implementation. And if the SMTC configuration would cause interruption, gap can be considered for measurements.

Proposal 3: Keep the previous agreement on misalignment between DRX-on duration and SMTC for NFG measurements, i.e., remove the brackets for 4ms.

- Conclusion

This contribution discussed the remaining issues of Rel-18 gap enhancements and gave the following proposal:

Proposal 1: When the UE is configured with concurrent gaps with NCSG, all the deactivated SCells can be measured within NCSG if the SMTCs to be measured are fully or partially overlapped with NCSG. Otherwise, the deactivated SCells are measured outside gaps.

Proposal 2: When UE is configured with the concurrent measurement gap and NCSG and the measurement of the measurement object is related to a deactivated SCell, after this SCell is activated, UE should measure the MO following gap association.

Proposal 3: Keep the previous agreement on misalignment between DRX-on duration and SMTC for NFG measurements, i.e., remove the brackets for 4ms.

- References

- R4-2420213 WF on RRM requirements for NR_MG_enh2