3GPP TSG-RAN WG4 Meeting #114	R4-2501151

# Athens, GR, 17th - 21st Feburary, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0058 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_FDD_LS_band) CR to TS 36.102 addressing incomplete Rel-18 requirements in clause 6.2B.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_LS_band |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Regarding IoT-NTN B254, NS_04N/NS_05N A-MPR values are missing in TS 36.102 clause 6.2B.3. For NS_04N/NS_05N, it is observed the 3/6/12 Tones allocation A-MPR values for B254 were indicated in TR 36.764. These A-MPR values need to be updated into TS 36.102. - The 1-Tone allocation A-MPR values for B254 is not analyzed for NS_04N/NS_05N. To provide the evaluated result is necessary for Cat-M1 and Cat-NB1/NB2 UEs. Based on evalaution, no NS_04N/NS_05N A-MPR for Cat-M1. But NS_04N/NS_05N A-MPR values for Cat-NB1/NB2 are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 6.2B.3, the NS_04N/NS_05N A-MPR values for 3/6/12 Tones operation for IoT-NTN B254 are updated based on TR 36.764. - In clause 6.2B.3, regarding Cat-NB1/NB2 UE, the missing 1-Tone allocation A-MPR values for B254 for NS_04N/NS_05N is further updated based on simulation results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The required NS_04N/NS_05N A-MPR values for IoT-NTN B254 are not provided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TR 36.764... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================Start of the change 1==========================

6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 | Table 6.2B.3-2, Table 6.2B.3-4 |  |
| NS_05N | 6.5B.4.4.6 | 254 | Table 6.2B.3-3, Table 6.2B.3-5 |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |

Table 6.2B.3-2: A-MPR for "NS_04N" power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.7 dB | 0.2 dB |  | ≤ 0.7 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

Table 6.2B.3-3: A-MPR for "NS_05N" power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 1.5 dB | 0.5 dB |  | ≤ 1.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | ≤ 0.7 dB |  | ≤ 0.7 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

Table 6.2B.3-4: 1-Tone A-MPR for "NS_04N" power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.5 dB | 0 dB | 0 dB |

Table 6.2B.3-5: 1-Tone A-MPR for "NS_05N" power class 3

| Modulation | π/4 QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/9 | 3/4/5/6/7/8 |
| A-MPR | ≤ 1.5 dB | ≤ 1.0 dB | 0 dB | 0 dB |

====================End of the change 1==========================