# 3GPP TSG-RAN WG4 Meeting #114	R4-2501154

# Athens, GR, 17th – 21st February, 2025

Source:	Sony

Title:	Further Views on MPR Reduction

Agenda item:	7.1.1.2.1

Document for:	Approval

# Background

In RAN4#113, the enhancement of the power domain via relaxed ACLR has been further discussed [1], and a WF [2] is agreed which focuses on the approach that converts full RB allocation in UE CBW to “inner RB allocation” with a new emission bandwidth.

In this contribution, we provide our further views on the MPR reduction via the new emission bandwidth.

# The mechanism for MPR reduction via a new emission BW

## The OOBE requirements

In RAN4#112, the approach of defining a “new emission BW” to scale the emission limit was discussed intensively, and the following further agreements have been made in RAN4#113:

- Extended CBW based approach

  - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE

  - Both scenario 1 and 2 should be covered

  - The network should ensure the co-existence be met when signalling the extended CBW.

Moreover, the boundary to apply the ACLR was also discussed:

- ACLR and SEM should be applicable from the edge of extended UE CBW.

    - FFS the following alternatives

      - Alt 1: The OOBE requirements are based on UE CBW

      - Alt 2: The OOBE requirements are based on extended UE CBW

| Alt 1 |  |
| --- | --- |
| Alt 2 |  |

Both alternatives allow the ACLR to be applied from the edge of the extended UE CBW, which relaxes the emission limit on UE and allows the reduced MPR. In Alt 2, ACLR and SEM are scaled according to the extended UE bandwidth. However, a potential issue here is the violation on the ITU recommendation. In ITU rec. 1539, the OOB is applied from 250% of the necessary bandwidths.

Observation 1: Scale the ACLR according to extended UE CBW may violate the ITU OOB recommendation.

Presumably, the extended UE CBW is 200% of the original UE CBW (50% extension on each side). Complying with ITU recommendation requires the 2.5*UE CBW > 1.5*extend UE CBW + 5 MHz, which limits the extended UE CBW < 5/6 UE CBW + 3 MHz. Therefore, for the sake of compliance with ITU rec, it is proposed that the OOBE is NOT scaled with the extended UE CBW, but rather keep it according to the original UE CBW.

Proposal 1: OOBE should be applicable from the edge of extended UE CBW, but the levels are based on UE CBW.

## Extended RB numbers

Then, the next question is how small the new emission limit bandwidth needs to be so that the UE can achieve sufficient MPR reduction, where the following agreements have been made:

- Extended RB numbers on each side of UE CBW could be different

  - Extended RB numbers are up to [ceil(NRB/2)] on each side of UE CBW, depends on the adjacent spectrum by operator in all possible scenarios

    - FFS whether extended RB number should be limited to max 50MHz at each side, which means the extended CBW is relevant to the configured UE CBW

    - Fixed extended RB number to [ceil(NRB/2)] is not precluded.

- Extended RBs are not used for the specific UE UL transmission but can be used by other UEs UL transmissions. It is generally understood that going beyond NRB/2 on each side does not further reduce the MPR.

By looking at the MPR currently specified for PC3 and PC2, it can be seen that about 1 to 1.5 dB gain can be obtained by transfer to the edge/outer RB to inner RB while there is no gain or very limited gain (~ 0.5 dB) by transfer the edge RB to outer RB allocations. Therefore, it is reasonable to focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated. In other words, the “new emission BW” shall be at least 1.5* (legacy UE CBW).

Proposal 2: To obtain sufficient gain in the field, it is proposed to focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated.

From the implementation aspect, it is also critical to simplify the possible configurations on MPR reduction. Suppose 3GPP specification does not limit the possible configuration to a fixed extended RB number set. In that case, the device must comply with all the possible configurations, which will overload the verification and testing effort to support this feature, regardless of how the test setup is defined in 3GPP. It is clear this will negatively impact the adoption of this feature in the field, and thus, it is strongly proposed that the extended RB number be fixed on each side of UE.

Observation 2: without fixing the possible extended RB configuration, device support for this feature needs to guarantee the MPR performance for all the possible configurations regardless of how the 3GPP conformance is defined, which will negatively impact the adaptation of this feature in the real world.

Proposal 3: The extended RB number should be fixed on each side of UE.

## IBE

As previously agreed, IBE will be applied to the edge between the original UE CBW and the extended UE CBW. However, the exact IBE limitation is still to be discussed within this region. We believe the existing IBE requirement can be applied directly by applying the NRB in IBE equations according to the extended UE CBW.

Proposal 4: the IBE requirements shall be applied within the extended UE CBW, with existing IBE requirements, but use the NRB with extended UE CBW

# Limitations on Redcap devices

In addition to the methodology study, the actual performance gain of supporting this reduced MPR also needs to be evaluated. Focus on the scenario that a narrower UE channel BW within wider BS bandwidth, allocating a device with narrow bandwidth, e.g., Redcap and eRedcap, towards the inner location within an operator’s spectrum block, may not be a common scenario in real life to avoid spectrum fragmentation from the network aspect. In addition, since such an operation scenario requires the operator spectrum block to be wide enough, it may not be feasible for most FDD bands, especially those at sub-1GHz.

Observation 3: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator’s spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 4: The frequency bands are usually small in FDD bands in sub 1 GHz, making it not feasible to move the UE away from the edge of the band.

# Signaling aspects

From the UE implementation aspect, all the out-of-band emission limits, including ACLR, SEM, spurious emission, and spurious emission for co-existence, need to be examined to determine the appropriate MPR reduction. Depending on the UE RF front-end implementations, the required MPR to meet each out-of-band emission is different with different RB allocations and frequency bands due to the spurious emission limit for the co-existence.

Observation 5: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 5: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability.

# Conclusion

In this contribution, we provide our views on the MPR reduction via modified out-of-band emission limits, where the following observations and proposals are given:

Observation 1: Scale the ACLR according to extended UE CBW may violate the ITU OOB recommendation.

Observation 2: without fixed the possible extended RB configuration, device support this feature needs to guarantee the MPR performance for all the possible configurations regardless how the 3GPP conformance is defined, which will negatively impact the adaptation of this feature in real world.

Observation 3: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator’s spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 4: The frequency bands are usually small in FDD bands in sub 1 GHz, making it not feasible to move the UE away from the edge of the band.

Observation 5: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 1: OOBE should be applicable from the edge of extended UE CBW, but the level are based on UE CBW.

Proposal 2: To obtain sufficient gain in the field, it is proposed to focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated.

Proposal 3: The extended RB number should be fixed on each side of UE.

Proposal 4: the IBE requirements shall be applied within the extended UE CBW, with existing IBE requirements, but use the NRB with extended UE CBW

Proposal 5: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability.

# References

- R4-2418138	Topic summary for [113][116] NR_ENDC_RF_Ph4_part1	Moderator (Huawei)

- R4-2420527	WF on Power domain enhancement	Huawei	approved

- RECOMMENDATION ITU-R SM.1539-1