# 3GPP TSG-RAN WG4 Meeting #114	R4-2501155

# Athens, GR, 17th – 21st February, 2025

Source:	Sony

Title:	RF requirements of ambient IoT device 1

Agenda item:	7.21.3.2

Document for:	Approval

# Background

In RAN#106, the WI of Solutions for Ambient IoT (Internet of Things) in NR has been approved [1], where the following objects have been defined for RAN4:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we will share our views on how the RF requirement shall be specified for device 1.

# Reception requirements.

## REFSENS

REFSENS of R2D link is the main requirement to be defined for AIoT RX. The REFSENS is calculated as in (1), where the SNR is the minimum SNR level at the input to BB that the receiver can demodulate the signal to meet the target performance, and the BW is the bandwidth of the LPF shown in Fig. 1

REFSENS=-174 dBm+10*logBW+NF+SNR+IM.                              (1)

Therefore, one of the key issues in deriving the REFSENS requirement is to define a proper performance metric and the corresponding target level. As an example, the main receiver REFSENS of legacy UE is defined at the DL power level at the antenna connector, and the throughput is higher than 95% of the maximum throughput of the reference measurement channels. Meanwhile, the REFSENS level is tentatively agreed to be measured at the power level with a 1% miss detection rate for the low-power wake-up receiver currently under discussion. For AIoT devices, considering the waveform of R2D link is similar to the wake-up sequences and the R2D link is also for paging AIoT devices, adopting the miss detection rate similar to the low-power wake-up receiver is more feasible. The detailed percentage numbers can be further studied in RAN4.

Proposal 1: RAN4 can consider adopt X% miss detection rate as the performance metric to define the REFSENS requirements.

Based on the agreed performance metric, RAN4 needs to derive the required SNR at the input to baseband (BB) to meet the target performance metric.

In addition to SNR, the noise figure (NF) is another key parameter that needs to be determined. NF measures the SNR degradation between the input and output of the receiver, and thus, aligning the assumption on the reference architecture of the R2D receiver is critical as well. In this paper, the following assumed architectureis adopted:

Fig. 1. The assumed architecture of R2D receiver to derive the REFSENS requirement.

(2)

The comparator can be seen as a 1-bit ADC which typically has an NF in the range of 10-15 dB. The RF-ED receiver, including the BB-LP, has a lower bound of 10 dB NF assuming an ideal circuit with square law modeling, such a high NF comes form the fact that the noise outside the BW will still be partially mixed into the BB via the self mixing effect of RF-ED receiver.In addition, for a practically implemntation, the NF can be in the range up to 20 dB considering the loss, impedance mismatch, non-ideal diode, etc. Thus, the cascaded NF of the receiver shown in Fig. 1 would be dominated by the RF-ED receiver, which is around 22 dB.

For legacy NR receivers, a 2 dB implementation margin (IM) is usually considered while considering the extremely low cost of the ambient IoT device 1, a higher IM, e.g., 3dB, may be assumed. Assuming that 10 dB SNR as input to BB is required to meet the proposed performance metric and 180 kHz signal BW, this will result in a REFSENS level of about -85 dBm.

Proposal 2: Consider 22 dB NF and 3 dB IM as the starting point to derive the REFSENS.

However, it should be highlighted that the REFSENS level above is only about the successful demodulat the R2D sequence. For AIoT devices, a sufficient high power level from energy source (could be CW node) is also needed that can active the devices, which could be in the range about -30 to -40 dBm based on the outcome of the SI. Therefore, it is critical to distinguish those two parameters.

Observation 1: the REFSENS level of R2D receiver could be in the range of -85 dBm/180 kHz pending on agreed performance metric but the activation threshold from the energy source may be much higher.

As a additional aspect, since OTA test has been agreed as the baseline test approach the referecen plan for the REFSENS needs also to be updated. Similar to FR2, the REFSENS can be defined as the EIS level at the centre of the quiet zone

Proposal 3: the REFSENS of AIoT needs to be defined based on the OTA reference plan which is the EIS level at the center of the quiet zone.

## Maximum input level

Although no decision was made during the SI, the maximum input level may need to be specified. Especially considering the device itself may not have any capability to control the gain of the input power level, it is essential to understand if there is any negative impact on the decoding performance of the RF-ED receiver if the power level is too high, e.g., due saturation or even breakdown of the transistor. Therefore, it is proposed that RAN4 further studies the need and potential requirement of maximum input level.

Proposal 4: RAN4 studies if there is any negative impact on the RF-ED receiver if the input signal level is too high.

## Requirements for interference rejection

Due to the cost and power consumption constraints of an RF-ED receiver in device 1, it is unlikely that any interference rejection mechanism can be supported, especially for any in-band interference. Moreover, as Rel-19 would only focus on the AIoT system indoors while cellular gNB is outdoor, it is also questionable if any relevant requirements would be necessary.

Proposal 5: Do not specify any requirement related to interference rejection, including blocking requirement, ACS/ASCS for device type 1 in Rel-19.

# Transmission part (D2R)

## The transmitted power

For AIoT device 1, power transmitted to the reader is via the reflection of a carrier wave (CW) from external nodes. Therefore, its transmission performance depends on both the design of AIoT devices and the property of the external CW and the emitter node. As no amplification component is assumed for device type 1, the key is to verify how much backscattering loss would be introduced from the RF requirement aspect. The transmitted power of AIoT device 1 shall then be specified as the backscattered power level under a given input/CW power level.

Observation 2: The transmitted power of AIoT device 1 is to verify how much backscattering loss would be introduced.

Proposal 6 The transmitted power of AIoT device 1 shall be specified and verified as the backscattered power level under a given input/CW power level.

## Output RF spectrum emissions

Output RF spectrum emission is another critical parameter for the D2R link that needs to be studied, as a mirror and high-order component could be generated on top of the frequency of the carrier (see figure. 2). The unwanted mirror harmonic contains equivalent power levels while higher order harmonics are −20 × log10(1/n) dB lower when compared with the first harmonic, say 9.5 dB and 14 dB lower for the 3rd and the fifth harmonics [2]. Therefore, the power level of those high-order components needs to be investigated if they would influence the system on the neighboring spectrum and determine the corresponding requirements.

Fig. 2. The illustration of mirror and high-order components of the backscattered signal

Proposal 7 RF spectrum emission performance of device 1 needs to be studied, and the corresponding requirements needs to be specified.

## Transmitted signal quality

For transmitted signal quality, as the small frequency shift method has been considered in the WI [1], it is critical to ensure the frequency accuracy after such an operation. Therefore, at least a frequency error requirement is needed.

For device type 1, considering the extremely low power budget, it is impossible to implement any additional oscillators in the receiver. The only method to support the frequency shift is via switching speed control.

Observation 3: the limited power budget of device 1 may not allow any additional oscillator, and the small frequency shift needs to be realized via controlling the switching speed of the load impedance.

The typical RF switch at 900 MHz used in smartphones has a switching speed of a couple of us, and thus the maximum frequency shift would be around a few kHz to 1 MHz. The stability of the switching speed may be affected by the thermal effect, power supply noise, and the technology of the RF switch. Based on the outcome of the SI, a accuracy of 10^5 PPM can be expected, which is also inlne with our analysis [3]. Therefore, it is essential to understand how accurate the frequency error would be from the system aspect.

Observation 4: The small frequency accuracy would be in the range about 10^5 PPM after clock sync.

Proposal 8: If a small frequency shift is required, the frequency error requirement will need to be specified. The requirement may depend on the system requirements.

In addition to the frequency error, some requirements are also needed to ensure the modulation quality of the backscattered signal. If On-off keying would be specified, the legacy EVM defination may not be able to be applied directly, some new measures may need to be specified here.

Proposal 9: Some requirements to ensure the modulation quality of the backscattered signal need to be studied and specified.

# Conclusion

In this paper, we provide on the RF requirement for AIoT device 1, where the following observations and proposals have been made:

Observation 1: the REFSENS level of R2D receiver could be in the range of -85 dBm/180 kHz pending on agreed performance metric but the activation threshold from the energy source may be much higher.

Observation 2: The transmitted power of AIoT device 1 is to verify how much backscattering loss would be introduced.

Observation 3: the limited power budget of device 1 may not allow any additional oscillator, and the small frequency shift needs to be realized via controlling the switching speed of the load impedance.

Observation 4: The small frequency accuracy would be in the range about 10^5 PPM after clock sync.

Proposal 1: RAN4 can consider adopt X% miss detection rate as the performance metric to define the REFSENS requirements.

Proposal 2: Consider 22 dB NF and 3 dB IM as the starting point to derive the REFSENS.

Proposal 3: the REFSENS of AIoT needs to be defined based on the OTA reference plan which is the EIS level at the center of the quiet zone.

Proposal 4: RAN4 studies if there is any negative impact on the RF-ED receiver if the input signal level is too high.

Proposal 5: Do not specify any requirement related to interference rejection, including blocking requirement, ACS/ASCS for device type 1 in Rel-19.

Proposal 6 The transmitted power of AIoT device 1 shall be specified and verified as the backscattered power level under a given input/CW power level.

Proposal 7 RF spectrum emission performance of device 1 needs to be studied, and the corresponding requirements needs to be specified.

Proposal 8: If a small frequency shift is required, the frequency error requirement will need to be specified. The requirement may depend on the system requirements.

Proposal 9: Some requirements to ensure the modulation quality of the backscattered signal need to be studied and specified.

# References

- RP-243326    New WID on Rel-19 Ambient IoT    RAN1 Vice-chair (Huawei)

- Y. Ding, R. Lihakanga, R. Correia, G. Goussetis and N. B. Carvalho, "Harmonic Suppression in Frequency Shifted Backscatter Communications," in IEEE Open Journal of the Communications Society, vol. 1, pp. 990-999, 2020,

- TR 38.769  Study on solutions for Ambient IoT (Internet of Things) in NR V19.0.0