3GPP TSG-RAN WG4 Meeting #114	R4-2501162

Athens, Greece, 17th – 21st February, 2025

Agenda item:	8.2

Source:            	Google

Title:                	Discussion on collision between SSB and RA occasion for LTM

Document for:	Approval

# Introduction

In RAN1#119, a LS on collision between SSB and RA occasion for LTM was approved to send to RAN4 [1], where the LS description is captured as follows. In this contribution, we would like to share our view for handling collision in the following discussion.

1. Overall Description: On R18 LTM, for unpaired spectrum, RAN1 identified possible collision between valid RA occasion (RO) and SSB associated with different cells in time domain, which is due to that RO validation is performed per cell. The collision could happen when the different cells are in the same frequency band or in different frequency band. It is unclear to RAN1 on whether/how to handle the collision between an SSB and a valid RO associated with different cells. 2. Actions: To RAN4: RAN1 respectfully asks RAN4 on whether/how to handle the collision between an SSB and a valid RO as described above and to provide answer for it.

# Discussion

On Rel-18 LTM, for unpaired spectrum, if a UE capable of rach-EarlyTA-Measurement-r18 is indicated to perform RACH procedure toward a LTM candidate cell before receiving cell switch command MAC-CE, there is a possibility that a serving cell SSB and RA occasion for the LTM candidate cell are very close, which may cause UE unable of receiving SSB and transmitting PRACH. According to the current RO validation rule in TS 38.213, the RO validation between SSB and RO is handled per LTM candidate cell, e.g., only consider LTM candidate cell#1 SSB and LTM candidate cell#1 RO. Hence, for the same LTM candidate cell, SSB and RO overlapping (or being very close to each other) can be avoided, since a RO overlapping with or close to a SSB is determined as invalid RO.

However, it is not very clear about how to handle the collision, when the SSB and RO are from different cells. For example:

- Serving cell SSB and LTM candidate cell #1 RO, or

- LTM candidate cell#1 SSB and serving cell RO, or

- LTM candidate cell#1 SSB and LTM candidate cell#2 RO, or

- LTM candidate cell#2 SSB and LTM candidate cell#1 valid RO.

For example, a valid RO determined for a LTM candidate cell could collide with a SSB from the serving cell. It is because when the UE determines valid RO for a LTM candidate cell, the UE does not consider other SSBs from the serving cell or other LTM candidate cells. The similar situation can be also applied to LTM candidate cell#x SSB and LTM candidate cell#y valid RO. Hence, the issue can be summarized that the collision could happen between SSB associated with cell#a and valid RO associated with cell#b, where cell#a or cell#b can be serving cell or LTM candidate cells, since the current RO validation rule for a cell does not consider SSBs from different cells.

In order to resolve this issue for unpair spectrum, one solution is that the UE is not required to transmit the PRACH and receive SSB associated with different cells simutaneously. Either drop transmitting PRACH in the cell#a or drop receiving SSB in the cell#b is the feasible solution when collision is occurred. Since it is now in the 5G later stage discussion, minimizing specification impact would be beneficial for reducing the UE implementation effort. Considering to drop receiving SSB may needs lots of change for the specification, to drop PRACH transmission would be an easier solution when the collision is happened between validated RO and SSB are associated from different cells. Hence, it is proposed to drop PRACH transmission as the baseline when the collision is happened between validated RO and SSB associated from different cells.

Proposal 1: For Rel-18 LTM, considering minimizing the specification impact, it is proposed to drop PRACH transmission as the baseline solution when the collision is happened between RO and SSB associated from different cells.

For the UE with the advanced capability, the UE may transmit PRACH and receiving SSB simultaneously, when the different cells are from different frequency band. However, considering minimizing the specification impact, it is proposed to apply the principle of dropping RO for the same frequency band case or different frequency band case.

Proposal 2: For Rel-18 LTM, considering minimizing the specification impact, Proposal 1 applies for the different cells in the same or different frequency band.

Then, we should discuss the conditions of dropping ROs. For example, when SSB and RO are partially overlapped or when SSB and RO are close to each other (e.g., below a threshold). For unpair spectrum, on RO validation, there is already a RO validation rule with Ngap symbols defined in TS38.213 for SSB associated with the same cell. If a UE does not receive SSB at least Ngap symbols before RO, the RO in a PRACH slot can be determined as invalid and PRACH transmission can be dropped in this RPACH slot. However, for RO validation rule with different cells (including serving cell and LTM candidate cells), whether the Ngap is also applicable for different cells in the same or different frequency bands may depend on the UE RF/ANT implementation.

Proposal 3: Further study the exact condition of dropping RO from below cases

- when RO of cell#a and SSB of cell#b are partially overlapped;

- when distance RO of cell#a and SSB of cell#b are below an offset threshold

  - whether the offset threshold can reuse the Ngap defined in TS 38.213

Proposal 4: RAN4 send LS in the Annex to RAN1 for Rel-18 LTM issue about collision handling between SSB and RACH occasion associated with the different cells.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: For Rel-18 LTM, considering minimizing the specification impact, it is proposed to drop PRACH transmission as the baseline solution when the collision is happened between RO and SSB associated from different cells.

Proposal 2: For Rel-18 LTM, considering minimizing the specification impact, Proposal 1 applies for the different cells in the same or different frequency band.

Proposal 3: Further study the exact condition of dropping RO from below cases

- when RO of cell#a and SSB of cell#b are partially overlapped;

- when distance RO of cell#a and SSB of cell#b are below an offset threshold

  - whether the offset threshold can reuse the Ngap defined in TS 38.213

Proposal 4: RAN4 send LS in the Annex to RAN1 for Rel-18 LTM issue about collision handling between SSB and RACH occasion associated with the different cells.

# Reference

- R1-2410911/R4-2500008, “LS on collision between SSB and RA occasion for LTM”, Google, RAN1#119

# Annex

3GPP TSG-RAN WG4 Meeting #114	R4-25XXXXX

Athens, GR, 17th – 21st February, 2025

Title:	[Draft] Reply to LS on collision between SSB and RA occasion for LTM

Response to:	R4-2500008 (R1-2410911)

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name: Clement Huang

E-mail Address: clementhuang@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 would like to respectfully thank RAN1 for LS on collision between SSB and RA occasion for LTM.

Regarding Rel-18 LTM collision handling between SSB and RA occasion associated with the different cells for unpaired spectrum, RAN4 concludes that dropping PRACH transmission can be the baseline solution in order for minimizing the specification impact. Whether it is applied for both the same frequency band case or different frequency band case is still under discussion. RAN4 will inform RAN1 once RAN4 make the conclusion in the future.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN1 to take the above decisions into account for their future work.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #114bis             7th April – 11th April 2025              Wuhan, CN

TSG-RAN WG4 Meeting #115.                19th May – 23th May 2025              Malta, MT