# 3GPP TSG-RAN WG4 Meeting #114                                                           R4-2501176

# Athens, Greece, 17th – 21st  February, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for less than 5MHz in NTN enhancement

Agenda item:	7.10.3.4

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN evolution [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. In the last meeting we provided our initial views on potential impacts on RRM requirements and in this meeting we will continue to study the concrete RRM requirements.

Discussion

The objectives of Rel-19 in NR NTN evolution are described in [1] including:

2.1 Less than 5MHz for NTN

- In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective

  - 2.1 Less than 5MHz for NTN

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

2.1.1 Requirements for NTN less than 5MHz

In the last meeting , we still have one issue on NTN less than 5MHz and the wayforward can be seen as below:

Wayforward: - Option 1: - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Option 2: Two sets of requirements - Set 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Set 2: smaller delay with higher SNR side condition. - FFS on the exact number of delay to be extended base on the simulation results under the same simulation assumptions for TN less than 5MHz [R4-2306400] - FFS on the separate UE capability to support set 2 requirements

We also find there is the impact on conditional handover and RAN4 shall consider studying it. Conditional handover contains two parts: Conditional handover with L3 measurement in NTN and Conditional handover without L3 measurement in NTN. In the previous meeting, we have already defined requirements on conditional handover without L3 measurement. In this part we mainly focus on the conditional handover delay with L3 measurement.

Conditional handover delay with L3 measurement

UE shall start RRM measurement before the time or distance condition is met. For the conditional handover delay with L3 measurement, the main handover delay is:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

The definition of Tinterrupt is:

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

And the measurement time delay Tmeasure is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

Each component has the different definition and we shall focus on the parameters which will be impacted by punctured PBCH. From my perspective, only Tmeasure and Tinterrupt will be impacted since they depend on PBCH detection. From the TS38.133, we know that the measurement time delay L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause nine which is related to TSSB time index and TPSS/SSS sync. For Tinterrupt, it depends on SSB index reading.

For TΔ, we shall follow the legacy TN extension as below:

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Few companies has one concern that if the Tmeasure and TΔshall be enhanced both. From my perspective, Tmeasure depends on the another clause in TS38.133 and whether  the Tmeasure shall be enhanced or not shall depend on whether RAN4 define new L3 measurement requirements or not. For  TΔ, we shall only follow the same legacy TN  principle.

Observation 1: Tmeasure and Tinterrupt will be impacted since they depend on PBCH detection.

Proposal 1: Whether the Tmeasure in conditional handover with L3 measurement shall be enhanced or not shall depend on whether RAN4 defines new L3 measurement requirements.

Proposal 2: TΔ shall be extended to 3*Trs.

Conclusion

In this paper we provided our views on RRM requirements on R18 NR NTN enhancement.

Observation 1: Tmeasure and Tinterrupt will be impacted since they depend on PBCH detection.

Proposal 1: Whether the Tmeasure in conditional handover with L3 measurement shall be enhanced or not shall depend on whether RAN4 defines new L3 measurement requirements.

Proposal 2: TΔ shall be extended to 3*Trs.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133

- R4-2420091, “WF on RRM requirements for NR_IoT_NTN_req_test_enh ”, Xiaomi.