# 3GPP TSG-RAN WG4 Meeting #114                                                             R4-2501178

# Athens, Greece, 17th – 21st  February, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on IoT NTN TDD mode

Agenda item:	7.31.5

Document for:	Approval

Introduction

At RAN 105# meeting, the revised WID on IoT NTN TDD mode [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. In this meeting, we will provide the initial analysis on the RAN4 impact on periodic pattern.

Discussion

The work item description can be seen as below:

This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

- In this meeting, we will discuss the scenarios and express our views on potential impact to RAN4 requirements towards the above objective based on RAN1’s discussion.

  - 2.1 RRM impact

- In RAN1, the concrete periodic pattern is still under discussion and the following figure shows us the specific periodic pattern:

- Figure 1 Periodic pattern

- We find that there are three parts which contains:

- Downlink subframe: The number of consecutive DL subframes at the behinning of each DL-UL pattern.

- Uplink subframe: The number of consecutive UL subframesof each DL-UL pattern.

- Guard period: Indicates the downlink to uplink guard period for TDD operation.

- RAN1 is discussing the number of downlink/uplink/guard period without any conclusion. In the current WID, it says that the periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. Thus, the periodicity is 9 and the frame structure will be impacted. First of we can see the FDD mode frame structure as below [2]:

- Figure 2 FDD mode frame structure

- Based on the N=9 periodic pattern, the TDD mode frame structure can be seen as below [3]:

- Figure 3 TDD mode frame structure

- We find that in legacy FDD mode, the NPSS is sent in 6th subframe of every radio frame, while NSSS is sent in the 10th subframe every other radio frames. However, we can see figure 3 that the periodicity of the NPSS and NSS will also increase, at least extend 9 times than legacy FDD mode. Then the UE can only search the NPSS/NSSS with a larger period than legacy FDD, the following table can be seen:

- Table 1 The detection time for NPSS/NSSS/NPBCH (at least N=9)

| - Channel | - Legacy NB-IoT NTN | - TDD NB-IoT NTN |
| --- | --- | --- |
| - NPSS | - Once every 10ms | - Once every 90ms |
| - NSSS | - Once every 20ms | - Once every 180ms |
| - NPBCH | - 64 transmissions in 640ms TTI | - 7 or 8 transmissions in 640ms TTI |

- Actually, other working groups are still discussing the concrete frame structure for IoT NTN TDD mode and based on the latest discussion from RAN1, some RRM measurement requirements will be impacted by extended NPSS and NSSS detection time. Also, RAN1 has the latest agreement in the last meeting as below:

Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

- Based on the above figure in RAN1’s agreement and the latest WID, we know that at least N=9 shall be considered. Thus, we would like to analyze the updated RRM requirements based on N=9 firstly.

- For cell re-selection requirements, there are three parameters shall be considered if N=9, they are Tdetect, Tevaluation and Tmeasure. Based on above analysis, NPSS and NSSS has extended nine times, that is, the SSB transmission interval is extended nine times, UE may need more time to detect/measure/evalute at least one SSB. Thus, we deem that the measurement period shall be also extended in order to guarantee the measurement accuracy. However, if reuse the same sample numbers in legacy NB-IoT, the whole measurement/evaluation period would be very long, the initial measurement results may not be useful any more for cell re-selection and other impacts. Thus, we do not think the measurement period shall be extended nine times like SSB transmission interval. To my understanding, the trade off between measurement accuracy and measurement delay shall be considered like the method in NR positioning. The detailed values shall be discussed after RAN1 has concrete conclusion on N and D.

- Observation 1: If reuse the same sample numbers in legacy NB-IoT, the whole measurement/evaluation period would be very long.

- Proposal 1: The trade off between measurement accuracy and measurement delay shall be considered like the method in NR positioning.

- Proposal 2: The detailed change of measurement/evaluation period shall be discussed after RAN1 has concrete conclusion on N and D.

- For RRC re-establishment, the UE re-establishment delay requirements may be impacted by extended periodic pattern, such as N=9. However, if the target cell is known, we deem that Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown, the Tsearch shall be updated. From my perspective, the TDD periodic pattern is different from FDD pattern, we may think that the changed NPSS, NSSS, NPBCH, and SIBX-NB as punctured NPSS, NSSS, NPBCH, and SIBX-NB from the existing subframe/SFN mappings of these channels. Thus, TSI_NB1-NC shall also be changed.

- Observation 2: The TDD periodic pattern is different from FDD pattern, we may think that the changed NPSS, NSSS, NPBCH, and SIBX-NB as punctured NPSS, NSSS, NPBCH, and SIBX-NB from the existing subframe/SFN mappings of these channels.

- Proposal 3: For RRC re-establishment delay, if the target cell is unknown, the Tsearch shall be updated. TSI_NB1-NC shall also be changed.

- For RRC connection redirection, the time delay includes Tperiod_DL_bitmap which is impacted by periodicity of the downlink subframe configuration for downlink transmission. The periodicity of DL subframe may be extended accordingly due to periodic pattern.

- Proposal 4: For RRC connection redirection, Tperiod_DL_bitmap may be changed due to updated periodic pattern.

- To my understanding, the periodic pattern is scaled by at least N=9 times and the available NPSS/NSSS shall be scaled same times if one radio frame only one available NPSS/NSSS is used.  In cases where Rmax is less than 64 (indicating weaker signal strength), the UE requires more frequent link quality assessments to ensure timely detection of link failures; hence, adopting OOS evaluation periods of 400 milliseconds and INS evaluation periods of 200 milliseconds is reasonable. However, when Rmax exceeds 64 (indicating better signal strength), the link quality becomes more stable. As a result, the evaluation periods can be appropriately extended to 4000 milliseconds for OOS and 2000 milliseconds for INS, which helps to minimize unnecessary resource consumption. Thus, if considering non-DRX configuration for radio link monitoring, the evaluation period shall be scaled by N.

- Observation 3: The periodic pattern is scaled by at least N=9 times and the available NPSS/NSSS shall be scaled same times if one radio frame only one available NPSS/NSSS is used.

- Proposal 5: For radio link monitoring, the evaluation period shall be scaled by N.

Conclusion

In this paper we provided our views on RRM requirements on R19 IoT NTN TDD mode enhancement.

- Observation 1: If reuse the same sample numbers in legacy NB-IoT, the whole measurement/evaluation period would be very long.

- Proposal 1: The trade off between measurement accuracy and measurement delay shall be considered like the method in NR positioning.

- Proposal 2: The detailed change of measurement/evaluation period shall be discussed after RAN1 has concrete conclusion on N and D.

- Observation 2: The TDD periodic pattern is different from FDD pattern, we may think that the changed NPSS, NSSS, NPBCH, and SIBX-NB as punctured NPSS, NSSS, NPBCH, and SIBX-NB from the existing subframe/SFN mappings of these channels.

- Proposal 3: For RRC re-establishment delay, if the target cell is unknown, the Tsearch shall be updated. TSI_NB1-NC shall also be changed.

- Proposal 4: For RRC connection redirection, Tperiod_DL_bitmap may be changed due to updated periodic pattern.

- Observation 3: The periodic pattern is scaled by at least N=9 times and the available NPSS/NSSS shall be scaled same times if one radio frame only one available NPSS/NSSS is used.

- Proposal 5: For radio link monitoring, the evaluation period shall be scaled by N.

- 4 References

- RP-242415, New WID on introduction of IoT-NTN TDD mode, Iridium Satellite LLC

- TS36.133

- R1-2408871, Qualcomm Incorporated

- R1-2407772, Thales

- R1-2407937, ZTE Corporation, Sanechips