3GPP TSG-RAN WG4 Meeting #114						R4-2501190

Athens, Greece, Feb 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | draftCR | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2 with updates to Annex N for Expected EIRP calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is draft CR to TS 38.141-2 with updates to Annex N. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to annex N added with EIRP calculation. Addition of definitions of some variables for EIRPθHLi , θHHi . Addition of Annex N.1.3 with Spatial sampling grid – based on TR 38.908. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will not include details on EEIRP calculations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex N |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of draft CR>

# Annex N (normative):

# Expected EIRP calculation and spatial sampling grid

# N.1	General

The key requirements of ITU Resolution Resolution 220 [25] are:

- compliance with values of Expected EIRP above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to Resolution 220 [25].

The Expected EIRP calculation is based on two averaging processes:

- Averaging of beamforming directions

- Averaging over horizontal and vertical angles

These two averaging processes are inter-related and should not be viewed as the order in which the calculation is done.

    - N.1.1 Averaging over beamforming directions

The first averaging process involves averaging over different beamforming directions. For a given vertical angle θ and horizontal angle φ with a sampling of K beamforming directions, the equation can be written as:

EIRPθ,φ=1Kk=1KwkEIRPk(θ,φ)                              (1)

The power measured is the sum of the EIRP in both polarisations.

The EIRP of each individual AAS beam denoted by EIRPk(θ,φ) is an average EIRP calculated from the second averaging process (i.e. in the next section, N.1.2).

    - N.1.2 Averaging over horizontal and vertical angles

The second averaging process is over a spherical strip. This average EIRP for a spherical strip is therefore:

EIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,                       (2)

where

θHLi , θHHi	is the elevation angular range bounded by θHLi and θHHi, e.g. for bin 1, this will be 0⁰ and 5⁰ respectively.

θn	is the corresponding angle of the nth midpoint between the elevation intervals, referenced from the horizon. Midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°.

φm	is the corresponding angle of the mth midpoint between the azimuth intervals

nL	is the lowest elevation point within the θHLi , θHHi range

nH	is the highest elevation point within the θHLi , θHHi range

M	is the number of azimuth points within the (-180°, 180°) range

N	is the number of elevation points within the θHLi , θHHi range

Note: All angles are in degrees.

    - N.1.3 Spatial sampling grid

Maximum measurement/spatial sampling step size in the elevation angle is 1 degree.

Maximum measurement/spatial sampling step size in the azimuth angle, is as TRP measurement captured in Annex I:

Δϕref=180∘πλDcyl

Dcyl are calculated as:

`	Dcyl=d2+w2

The definition of d, w and h is shown in figure N.1.3-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure N.1.3-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Alternatively, when EUT radiating dimensions are not known,

∆ϕref=λλo BeWϕ

where λo is the wavelength of the wanted signal, and BeWφ are the minimum beamwidth of the wanted signal in the φ-axis,.

BeWφ may be set to minimum beamwidth declared for the OTA BS radiated transmit power requirement.

<End of draft CR>