3GPP TSG-RAN WG4 Meeting #114	R4-2501193

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.1.1.3.4

Source:            	Google

Title:                	Discussion on SRS IL imbalance for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the SRS IL (insertion loss) imbalance for 6Rx handheld and FWA UEs.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#113, the SRS IL imbalance for 6Rx handheld and FWA UEs has been discussed and some open issues are captured in the approved WF [2]. In WF, the main issue is to first discuss the general considerations for SRS IL imbalance issue and then decide whether to introduce SRS IL imbalance issue solution in Rel-19. In this paper, we would like to share our view in the following.

# Topic 3: SRS IL imbalance issue ## Sub-topic 3-1: General considerations for SRS IL imbalance issue Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1. Issue 3-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1. ## Sub-topic 3-2: SRS IL imbalance issue solutions Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: RAN4 to further discuss the candidate solution framework below and present proposals concerning the details. Companies are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed.. Candidate solution framework: - UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on whether condition would introduce any additional RAN4 performance requirement. No RAN1 specification impact. - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others - Details of SRS IL self-compensation and assistance framework are FFS

# Discussion

In Rel-15, SRS Tx switching feature is introduced to obtain more accurate channel state information (CSI) by utilizing channel reciprocity for TDD bands. The motivation to introduce SRS Tx switching feature is that Tx antenna port number is usually less than Rx antenna port number from UE implementation perspective. In other words, in order to obtain more accurate CSI for DL-MIMO transmission at the base station side, the UE can be scheduled to perform antenna switching for SRS transmission in the corresponding Rx antenna port sequentially instead of simultaneous SRS transmission on the corresponding Rx ports.

From UE capability signalling perspective, SRS Tx switching feature can be indicated by the corresponding IE srs-TxSwitch/srs-TxSwitch-v1610 with “xTyR”, e.g., 1T2R/1T4R/ 2T4R/2T2R/4T4R/1T4R-2T4R, where “x” is the capable antenna port for SRS transmission and “y” means the total number of receive antenna where SRS transmission can be performed for the target band or the target band in the band combination. In Rel-17, the xTyR has been extended to support at most “1T8R/2T8R/4T8R” in the corresponding IE srs-AntennaSwitchingBeyond4Rx-r17.

From implementation perspective, if the UE is configured to perform SRS antenna switching, SRS Tx transmission may go through different number of RFFE antenna switch components, e.g., SPXT, DPXT where X=2,3,4,…8, at each associated Rx path on PCB board, which may cause different insertion loss for each SRS Tx path. In addition, the PCB tracing loss may also be different for each SRS Tx associated Rx path. Hence, SRS Tx power among each associated Rx paths would be imbalanced due to RFFE insertion loss and PCB tracing loss, which may cause SRS IL imbalance when the UE performs SRS antenna switching.

In RAN4#113, the SRS IL imbalance issue for 6Rx handheld and FWA UEs has been discussed and some open issues are captured in the WF. In WF, the first open issue whether to solve SRS-IL imbalance issue in Rel-19 is still not determined. In our understanding, SRS-IL imbalance issue for SRS Tx power imbalance can be analyzed from two perspective, where the first is from conducted RF antenna connector perspective and the second is from OTA perspective.

From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry. In addition, the cost for factory RF calibration would be minor that the UE may need to save all calibration data in the non-volatile (NV) memory. In our understanding, for all SRS Tx power level after well RF calibration, SRS-IL imbalance among each SRS Tx associated Rx paths would be only around 1~2 dB. Hence, SRS-IL imbalance would not be a critical issue if the device is well calibrated from conducted RF antenna connector perspective.

Observation 1: From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry. Hence, the SRS-IL imbalance would not be a critical issue if the device is well calibrated from conducted RF antenna connector perspective.

From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed due to more protection on main Tx/PRx. In addition, if the UE is well calibrated, SRS Tx power imbalance may still often be occurred in the real live network. Which means that the received SRS Tx power at the network side may be varying very fast due to the different channel condition. In our view, even if SRS-IL imbalance is known in advance, SRS Tx power imbalance may still become higher and the imbalance may be caused by different antenna implementation and/or different channel condition. Hence, it is straightforward that SRS-IL imbalance would not be a critical factor to impact SRS Tx power imbalance from OTA perspective, and it would be better to handle it by enhancing the network UL channel estimation performance.

Observation 2: From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed because of more protection on main Tx/PRx. Even if SRS-IL imbalance is known in advance, SRS Tx power imbalance may still become higher and the imbalance may be caused by different antenna implementation and/or different channel condition. Hence, it is straightforward that SRS-IL imbalance would not be a critical factor to impact SRS Tx power imbalance from OTA perspective, and it would be better to handle it by enhancing the network UL channel estimation performance.

In RAN4#113, several candidate solutions for SRS-IL imbalance are merged into two candidate solution frameworks for the SRS-IL imbalance issue on 6Rx handheld and FWA UE. The first one is that UE performs self-compensation of SRS IL up to its configured maximum output power and the second one is UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic). In our view, we think we should first identify whether SRS-IL imbalance is a critical issue or not. As above mentioned, from conducted perspective, it would be a minor issue when the device is well factory RF calibrated at each SRS Tx on the associated Rx paths. From OTA perspective, it would still not be the critical factor to impact SRS Tx power imbalance at the network side because different antenna radiation pattern and/or channel condition may also impact a lot even if the device SRS Tx is well calibrated. Therefore, considering that SRS-IL imbalance can be reduced a lot by factory RF calibration process from conductive perspective and that SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance from OTA perspective, it is proposed not to introduce any SRS-IL imbalance reporting mechanism for 6Rx UE in Rel-19.

Proposal 1: Considering that SRS-IL imbalance can be reduced a lot by factory RF calibration process from conductive perspective and that SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance from OTA perspective, it is proposed not to introduce any SRS-IL imbalance reporting mechanism for 6Rx UE in Rel-19.

# Conclusion

The proposals in this contribution are summarized in the following.

Observation 1: From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry. Hence, the SRS-IL imbalance would not be a critical issue if the device is well calibrated from conducted RF antenna connector perspective.

Observation 2: From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed because of more protection on main Tx/PRx. Even if SRS-IL imbalance is known in advance, SRS Tx power imbalance may still become higher and the imbalance may be caused by different antenna implementation and/or different channel condition. Hence, it is straightforward that SRS-IL imbalance would not be a critical factor to impact SRS Tx power imbalance from OTA perspective, and it would be better to handle it by enhancing the network UL channel estimation performance.

Proposal 1: Considering that SRS-IL imbalance can be reduced a lot by factory RF calibration process from conductive perspective and that SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance from OTA perspective, it is proposed not to introduce any SRS-IL imbalance reporting mechanism for 6Rx UE in Rel-19.

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2420376, “WF on 6Rx requirements”, AT&T, RAN4#113

- R4-2418836, “Discussion on SRS IL imbalance for 6Rx UE”, Google, RAN4#113