3GPP TSG-RAN WG4 Meeting #114	R4-2501218

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.17.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The enhancement on FR2-1 L3 measurement by optimizing Rx beam sweeping factor was further discussed in last meeting with agreements and issues captured in [1]. In this paper, we further provide our views on remaining issues.

# Discussion

The agreements achieved in last meeting are summarized as follows:

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions): Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

One issue is whether to define conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”. From under understanding, it is essential to have a quit method from UE perspective. As agreed in last meeting, the activation condition is only based on serving cell quality (e.g. RSRP and/or RSRQ threshold). If UE moves to the cell edge and stops moving for a long time, UE will activate multiple panels for fast L3 measurement for a long time. Another reason is that, the channel condition may vary from UE to UE. For instance, the threshold conditions are met due to some blocking but the UE is still in the cell centre. It does not necessarily require to UE to perform FBS all the time.

Observation 1: Based on agreed triggering condition (serving cell’s signal quality), it is possible that UE will always activating multiple panels for L3 measurement, which is problematic for at least following case:

- Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving.

- Case 2: UE is still in the cell centre and the triggering conditions are met due to block.

As emphasized several times in previous discussion, activating multiple panels simultaneously are very power consuming, and we should not assume that UE will keep activating multiple panels for very long time. Companies propose to rely on UAI introduced in Rel-18 multi-Rx feature. However, it is different situation here. In Rel-18 multi-Rx, activating multiple panel is for data boosting and the control is up to gNB even UE reports UAI. From UE perspective, if there is indeed huge amount of data, it is worthy to sacrifice the power consumption for better throughput. It is reasonable to let gNB to decide the status as gNB is aware of the data quantity and this is only temporary status. But for Rel-19, activating multiple panels is only for L3 measurement, it is long term RRM activity. If UE indicates that there is power consumption issue, gNB has no idea whether UE is in case 1 and 2 (fast measurement is not that important).

Observation 2: Rel-18 UAI indication is not sufficient as the decision is made by gNB, and gNB has no idea whether fast L3 measurement is urgent or not (e.g. case 1 and case 2 mentioned above). And gNB even don’t known whether fast measurement is triggered or not.

Based on the scenarios corresponding gain as mentioned above, no matter for potential HO or SCell change, if UE has been activating FBS L3 measurement, and has reported qualified measurement results to NW, it means NW already has the candidate target cell for HO or SCell modification/addition. Thus, UE shall be allowed to fall back to normal measurement mode (i.e. single panel).

Proposal 1: Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification.

Another issue is about the searcher assumption, and it is concluded that RAN4 will not limit the searcher number and introduce SSB processing time.

Issue 1-1-3: searcher number assumption for RRM measurement with two panels activated on single carrier Agreement: Directly discuss the SSB processing time. Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

For scenario 5, we are support option 1 to also introduce 2 ms processing delay in the as agreed for other scenarios.

Proposal 2: For SSB processing time for scenario 5, RAN4 to also consider an additional processing delay (2ms) as agreed for other scenarios.

As explained during searcher assumption discussion, enough flexibility shall be allowed for UE implementations. Supporting these features requires additional UE capability for both RF and baseband resources. And the capacity of RF/baseband resource varies for different band combinations. For instance, if UE is configured with highest band combination with maximum number of CCs, it is most likely that UE will not be able to support this feature as there is no spare resource for thi. Or if UE is configured with many MOs to be measured, UE maybe also difficult to support this feature.

Observation 3: Whether UE can support this feature may varies depending on

- Different BC

- Number of Carriers/MOs to measured

- Intra-frequency/inter-frequency

Proposal 3: The UE capability design shall allow enough flexibility on supporting this feature considering:

- Different BC

- Number of Carriers/MOs to measured

- Intra-frequency/inter-frequency

# Conclusions

Observation 1: Based on agreed triggering condition (serving cell’s signal quality), it is possible that UE will always activating multiple panels for L3 measurement, which is problematic for at least following case:

- Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving.

- Case 2: UE is still in the cell centre and the triggering conditions are met due to block.

Observation 2: Rel-18 UAI indication is not sufficient as the decision is made by gNB, and gNB has no idea whether fast L3 measurement is urgent or not (e.g. case 1 and case 2 mentioned above). And gNB even don’t known whether fast measurement is triggered or not.

Proposal 1: Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification.

Proposal 2: For SSB processing time for scenario 5, RAN4 to also consider an additional processing delay (2ms) as agreed for other scenarios.

Observation 3: Whether UE can support this feature may varies depending on

- Different BC

- Number of Carriers/MOs to measured

- Intra-frequency/inter-frequency

Proposal 3: The UE capability design shall allow enough flexibility on supporting this feature considering:

- Different BC

- Number of Carriers/MOs to measured

- Intra-frequency/inter-frequency

# References

[1] R4-2420093 WF for [111][205] NR_RRM_enh3_part1

[2] R4-2415656 Topic summary for [112bis][209] NR_RRM_Ph5_Part1