3GPP TSG-RAN WG4 Meeting #114	R4-2501219

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.17.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on maintenance for fast SCell activation based on EMR

Document for:	Discussion

# Introduction

The enhancement about fast SCell activation based on Rel-18 EMR is completed in last meeting with agreements captured in [1] and the CR is endorsed in [2]. In this contribution, we provide our views on maintenance about how to accommodate the new requirements in the spec.

# Discussion

In the endorsed CR [2], the new requirements are inserted in the existing 8.3.2 which is categorized as a branch of unknown case. As commented during the discussion, how to accommodate the new requirements need reconsidered as many requirements about SCell activation are directly referred to 8.3.2. There is offline discussion about having a new dedicated section for fast SCell activation requirements based on measurement in IDLE/INACTIVE mode. We are in favour of the framework will has less impact on existing requirements.

Proposal 1: Capture Rel-19 fast SCell activation requirements based on EMR reporting in a new dedicated section, the update related requirements (e.g. direct SCell activation and PUCCH SCell activation) accordingly.

In particular, for PUCCH SCell activation requirements, Tactivation_time can directly refer to the new section. Apart from that, there are other aspects which needs to be updated accordingly. Taking Rel-18 L3 reporting after SCell activation as reference, the known conditions for PL-RS should be updated. We provide TP as follows:

For the UE capable of l3-MeasUnknownSCellActivation, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 reporting after SCell activation command and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for the L3 reporting after SCell activation command - The target pathloss reference signal determination is based on the L3 reporting after SCell activation command, if UE only sends L3 reporting before receiving TCI activation command, or - The target pathloss reference signal determination is based on either the latest L1-RSRP measurement reporting or the L3 reporting after SCell activation command, if UE reports both before receiving TCI activation command - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB ------------------------------------ Text proposal 1------------------------------------ When UE sends a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3 with following conditions are met: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode, the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for EMR report and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for the EMR reporting - The target pathloss reference signal determination is based on the EMR reporting - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB

Another aspect is the applicability of the unknown PUCCH SCell activation requirements, the TP as proposed as follows:

------------------------------------ Text proposal 2------------------------------------ The requirement for unknown PUCCH SCell applies provided that the PDCCH order (when applicable) and the activation commands for TCI, UL spatial relation and PL-RS (when applicable) are based on the latest valid L1-RSRP reporting or the L3 reporting after SCell activation command via Primary PUCCH group or EMR reporting as defined in 8.3.2A.

Proposal 2: Based on proposal 1, the requirements for unknown PUCCH SCell activation based on EMR shall be updated accordingly as proposed TP in this contribution.

# Conclusions

Proposal 1: Capture Rel-19 fast SCell activation requirements based on EMR reporting in a new dedicated section, the update related requirements (e.g. direct SCell activation and PUCCH SCell activation) accordingly.

Proposal 2: Based on proposal 1, the requirements for unknown PUCCH SCell activation based on EMR shall be updated accordingly as proposed TP in this contribution.

# References

[1] R4-2420094    WF on RRM requirements for NR_RRM_Ph5_Part2

[2] R4-2420132    draftCR on Fast SCell activation for UE supporting Rel-18 eEMR